Code No. 311102

Set. No.

1

III-B.Tech I-Semester Supplementary Examinations, June 2003

DIGITAL SIGNAL PROCESSING

(Common to Bio-Medical Engineering, Electronics and Computer Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Find the impulse and step responses for the given system:

y(n)+y(n-1) = x(n)-2x(n-1)

b)	Test the following systems for linearity, time invariance, causality and stability.

i)	y(n) = a |x(n)|

ii)	y(n) = sin(2п fn/F) x(n)

2. a)	 State and prove time and frequency shifting properties of Fourier transform.

b)	 Find the Fourier transform of the following signals

- x(n) = (αn sin won)u(n) 		|α|<1

ii)	x(n) =(1/4)n u(n+4)

3. a)	Define DFT of a sequence x(n). Obtain the relationship between DFT and DTFS.

b)	Consider a sequence x(n) = { 2, -1, 1, 1 } and T = 0.5 compute its DFT and compare it with its DTDT.

4. a)	Implement the decimation in time FFT algorithm for N=16.

b)	In the above Question how many non – trivial multiplications are required.

5.	Find the transfer function of a system whose input is:

x(n)=(12)nu(n)−14(12)n−1u(n−1) and the out put is y(n)=(13)nu(n) . Is the system stable? Realize the system with minimum possible amount of memory. Determine its impulse response.

6. a)	What is an IIR digital filter?

- How are IIR digital filter realized?

c)	What are the various realizability constraints imposed on transfer function of an IIR digital filter.

7. a)	Design a linear phase low pass filter with a cut-off frequency of π /2 radians/seconds. Take N=7

b)	Write the magnitude and phase functions of Finite Impulse Response filter when 			i)	impulse response is symmetric & N is odd

ii) 	impulse response is symmetric & N is even

Contd…2

Code No. 311102				.2.				Set No.1

8. a)  	What are the basic elements used to construct the block diagram of discrete time system?

b) 	Construct the block diagram and signal flow graph of the discrete time system whose input-output relations are described by following difference equation

(i)     	y(n)=0.5x(n)+0.5x(n-1)

(ii)    	y(n)=0.25y(n-1)+0.5x(n)+0.75x(n-1)

###

Code No. 311102

Set. No.

2

III-B.Tech I-Semester Supplementary Examinations, June 2003

DIGITAL SIGNAL PROCESSING

(Common to Bio-Medical Engineering, Electronics and Computer Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Show that a relaxed linear time invariant system is equal if and only if h(n) = 0 for n<0.

b)	Determine the range of values of parameter ‘a’ for the linear time invariant system with impulse response h(n), to be stable, where

0  otherwise

an n ≥ 0 n even

h(n) ={

2. a)	Prove that the convolution in time domain leads to multiplication in frequency domain for discrete time signals

- The out put y(n) for a linear shit invariant system, with the input x(n) is given by

Y(n) =  x(n)-2x(n-1)+x(n-2)

Compute and sketch the magnitude and phase response of the system |w|≤ п.

3. a)	Distinguish between DFT and DTFT .

b)	Consider a sequence x(n) of length L. Consider its DTFT  Xd (w) is sampled and N is the number of frequency samples. Discuss the relation between L and N for inverse DTFT = inverse DFT comment on the aliasing problem.

c)	Compute the DFT of x(n) = { 1, 0, 0, 0 } and compare with Xd (w).

4. a)	Implement the Decimation in frequency FFT algorithm of N-point DFT where   N-8. Also explain the steps involved in this algorithm.

- Compute the FFT for the sequence x(n) = { 1, 1, 1, 1, 1, 1, 1, 1  }

5. a)	With reference to Z-transform, state the initial and final value theorem.

b)   	Determine the causal signal x(n) having the Z-transform

































4

1

2

1

)

(

2

2

Z

Z

Z

Z

Z

X

6. a)	Discuss impulse invariance method of deriving IIR digital filter from corresponding analog filter.

- Convert the following analog filter with transfer function  HA(S)=S+0.2/(S+0.2)2+16 using impulse invariance method

Contd…2

Code No. 311102				.2.				Set No.2

7. a)	Define Infinite Impulse Response & Finite Impulse Response filters and compare

b)	Design a low pass Finite Impulse Response filter with a rectangular window for

a five stage filter given:

Sampling time 1 msec;  fc = 200 Hz

Draw the filter structure with minimum number of multipliers.

8. a)	What are the advantages in cascade and parallel realization of IIR systems.

- The transfer function of a system is given by

H(Z)=(1+Z−1)31−14Z−11−Z−1+12Z−2

Realize the system in cascade and parallel structures.

###

Code No. 311102

Set. No.

3

III-B.Tech I-Semester Supplementary Examinations, June 2003

DIGITAL SIGNAL PROCESSING

(Common to Bio-Medical Engineering, Electronics and Computer Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Represent the following sequence graphically

x(n) = δ(n+3)- 2δ(n+2)+ 3δ(n+1)+ δ(n-1)-4 δ(n-3)

- If the above x(n) is given as input to a system with h(n) = u(n) – u(n-5), Plot the

Output y(n).

c)	Define stability and causality of a time invariant system and prove the stability condition.

2. a)	Discuss the frequency-domain representation of discrete-time systems and      signals by deriving the necessary relation.

- Draw the frequency response of LSI system with impulse response

h(n) = an u(-n)			(|a| < 1)

3. a)	Define DFT. Give two properties of DFT.

b)	Discuss the effects of truncating a sequence x(n) of infinite duration.

c)	Compute the DFT of x(n) = { -1, 0,-1 } with T = 0.5. Plot the DFT sequence suggest a method for improving frequency resolution.

4. a)	Let x(n) be a real valued sequence with N-points and Let X(K) represent its DFT, with real and imaginary parts denoted by XR(K) and X1(K) respectively. So that X(K) = XR(K) + JX1(K). Now show that if x(n) is real, XR(K) is even and X1(K) is odd.

b)	Compute the FFT of the sequence x(n) = { 1, 0, 0, 0, 0, 0, 0, 0  }

5. a)	An LTI  system is described by the equation

y(n)=x(n)+0.81x(n-1)-0.81x(n-2)-0.45y(n-2).  Determine the transfer function of          the system. Sketch the poles and zeroes on the Z-plane.

- Define stable and unstable system. Test the condition for stability of the first-order     IIR filter governed by the equation y(n)=x(n)+bx(n-1).

6. a)	Derive a relationship between complex variable S used in Laplace Transform (for analog filters) and complex variation Z used in Z-transform (for digital filters)

b)		Discuss the various properties of Bilinear transformation method.

Contd…2

Code No. 311102				.2.				Set No.3

7. a)	Design a low pass filter by the Fourier series method for a seven stage with cut-off frequency at 300 Hz if ts= 1 msec. Use hanning window.

b) 	Explain in detail, the linear phase response and frequency response properties of Finite Impulse Response filters.

8. a)	List the different types of structures for realizing FIR system and determine the direct form-I, direct form II of the following LTI system

y(n)=-0.5y(n-1)+0.25y(n-2)+0.125y(n-3)+x(n)+0.5x(n-1)+0.75x(n-2)

- Write briefly about digital processing of speech.

###

Code No. 311102

Set. No.

4

III-B.Tech I-Semester Supplementary Examinations, June 2003

DIGITAL SIGNAL PROCESSING

(Common to Bio-Medical Engineering, Electronics and Computer Engineering)

Time: 3 hours							Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1. a)	Write four advantages of Digital Signal Processing over Analog Signal Processing.

- A signal y(n) is governed by the recursive equation

y(n) = 2y(n-1)+ δ(n) with y(0) =  4.

Find y(-2),y(3). Is the signal bounded or not?

c)	Convolve the two signals

x(n) = (½)n x(n) and h(n) = u(n)-u(n-10)  where u(n) is unit step function.

2. a)	Show that the frequency response of a discrete system is a periodic function of frequency.

b)	Obtain the frequency response of the first order system with difference equation

y(0) =  x(n)+10y(n-1) with initial condition	y(-1) = 0 and sketch it. Comment about its stability.

c)     State and prove the frequency shifting property of Fourier transform.

3. a)	What is “ padding with Zeros ”. With an example explain the effect of padding a sequence of length N with L Zeros or frequency resolution.

b)	Compute the DFT of the three point sequence x(n) = { 2, 1, 2 }. Using the same sequence, compute the 6 point DFT and compare the two DFTs.

4 	An 8 point sequence is given by x(n) = {2,2,2,2,1,1,1,1}. Compute 8 point DFT of x(n) by

  - radix – 2 D I T  F F T

  - radix – 2 D I F  FF T

Also sketch magnitude and phase spectrum

5. a)	Explain how the analysis of discrete time invariant system can be obtained      using convolution properties of Z transform.

b)	Determine the impulse response of the system described by the difference equation y(n)-3y(n-1)-4y(n-2)=x(n)+2x(n-1)  using Z transform.

6. a)	What is warping effect?  Discuss influence of warping effect on amplitude response and phase response of a derived digital filter from a corresponding analog filter.

b)	Discuss impulse invariance method.

Contd…2

Code No. 311102				.2.				Set No.4

7. a)	Design a low pass filter using Fourier series method using rectangular windows for 5 taps only, if the folding frequency is 5 kHz and the corner frequencies are 1 and 3 kHz.

b)	List the merits and demerits of Finite Impulse Response filters.

8. a) 	Describe how targets can be decided using RADAR

b) 	Give an expression for the following parameters relative to RADAR

i) Beam width       ii) Maximum unambiguous range

c) 	Discuss signal processing in a RADAR system

###