Code No: 311302

Set No.

1

III - B.Tech. I Semester Supplementary Examination, June 2003

COMMUNICATION ENGINEERING

(Electronics and Control Engineering)

Time: 3 hours.				                 		Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	Explain the method of detecting SSB using coherent detector.

2.	Explain Filter method to suppress the sidebands in AM wave. Mention its  drawbacks.

3.a)	Draw the block diagram of Armstrong Transmitter, and explain the function of each block.

- In a commercial FM system,  the frequency deviation is 75kHz and maximum modulating frequency is 15 KHz.   For a unit amplitude of the modulating Signal,  Calculate (i) the number of significant bandwidths .  (ii) the bandwidth of FM Signal.

4.a)	Explain the terms

- Sensitivity	 ii) Selectivity and Fidelity in a typical radio system.

b)	Describe any one method for measurement of any two parameters of a receiver.

5.a)	What is the necessity of tracking in radio receivers? Explain the three-point tracking employed in a radio receiver.

b)	Explain clearly keyed AGC in radio receiver with a neat diagram. How it differs from simple AGC?

6.	What is meant by Nyquist rate?  What is meant by Nyquist interval?

7.	Draw the block diagram of QPSK transmitter and explain the operation.

8.	Explain what is meant by the term data transparency, and how it may be achieved using

a) character stuffing.

b) zero bit insertion.

&&&&&

Code No: 311302

Set No.

2

III - B.Tech. I Semester Supplementary Examination, June 2003

COMMUNICATION ENGINEERING

(Electronics and Control Engineering)

Time: 3 hours.				                 		Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	Explain the method of generation of FM by indirect method.

2.	Explain with the aid of waveforms, how a grid modulated class C amplifier generates AM.

3.a)	Why is frequency stability of a transmitter important?   Explain one method to            	achieve good stability?

b)	A typical transmitter is to operate on a frequency of 146.94MHz and is to have a deviation z.  The Bandwidth of the audio amplifier is 300 – 3000 Hz. The Modulating signal is 1 KHz Sine wave.   Determine the frequency of the 'Crystal Oscillator.

4.a)	Draw practical diode detector and explain its working.

b)	Why AGC is required in radio receivers? Distinguish between Simple AGC and       delayed AGC.

5.a)		Write short note on “Image frequency and its reduction”.

b)	Explain the role of mixer and IF stages in AM receiver.

6.	What is meant by continuous wave modulation? Discuss with example.

7.	Draw the block diagram of FSK transmitter and explain.

8.a)	Explain mechanical, electrical, functional specification of E1A – 232 interface        standard.

b)	 What is the purpose of a null modem?

&&&&&

Code No: 311302

Set No.

3

III - B.Tech. I Semester Supplementary Examination, June 2003

COMMUNICATION ENGINEERING

(Electronics and Control Engineering)

Time: 3 hours.				                 		Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	Explain the amplifier method of generating AM wave.

2.	Compare the three main systems of SSB generation by drawing up a table of the outstanding characteristics of each system.

3.a)	Why is frequency multiplication needed in FM transmitters and how is it 	accomplished? What is the equipment usually used for generating a carrier 	frequency?   Draw the circuit diagram of such a generator and explain its 	operation.

b)	Determine the number of FM station that can be accommodated in a 8 MHz Band being for use with the same standards that apply to the 88 – 108 MHz band.

4.a)	List and discuss the factors influencing the choice of intermediate frequency for a radio receiver.

b) 	Explain the operation of a self-excited transistor mixer by 3-frequency approach with a neat diagram.

5.a)		Draw a block diagram of a typical AM receiver and describe briefly function of each block.

b)	Explain the necessity for

- Tone and volume control

- Alignment and tracking in radio receiver.

6.		What is meant by aperture effect?  Discuss with example.

7.		Draw the block diagram of DPSK receiver and explain.

8.a)	Why FSK is not suitable for high speed modems.

b) 	Why are modems needed for telephone communications?

c)	Explain the asymmetry of 56 k modems.

&&&&&

Code No: 311302

Set No.

4

III - B.Tech. I Semester Supplementary Examination, June 2003

COMMUNICATION ENGINEERING

(Electronics and sControl Engineering)

Time: 3 hours.				                 		Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	Explain the circuit diagram of linear diode detector and explain how it is used for AM detection.

2.	With aid of vector diagrams, explain what happens when a carrier is modulated by a single noise frequency.

3.a)	How is the distortion in the power amplifier taken care of in AM transmitter?             	Discuss with necessary diagram.

b)	What is the frequency range standards for AM Broadcast and FM Broadcast.

c)	Prove that frequency – multiplication of an FM signal increases the deviation             	while frequency conversion does not increase the deviation.

4.a)	Draw the block diagram of a FM Receiver and explain the functions of each block.

b)	Write short notes on “Fading and diversity reception”.

5.a)	Describe the principle of operation of,

i)  Tuning Indicator,

ii)  Mute circuit.

b)	Explain with a diagram, the procedure for measurement of noise figure of a radio receiver.

6.	Discuss the two different forms of pulse time modulation for the case of a sinusoidal modulating wave.

7.		Draw the block diagram of DPSK transmitter and explain.

8.a)	Explain the major goals of packet switching.

b)	Give the characteristics of computer controlled circuit switching.

c)	Give the characteristics of computer controlled message switching.

&&&&&