Code No: 311303

Set No:

1

III B.Tech  I-Semester (Supplementary) Examination June, 2003

OPERATIONS RESEARCH

(Electronics and Control Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks.

- - -

| 1. |  | Use Big M method to maximize Z = 6X1 + 4X2 Subject to the constraints 2X1+ 3X2  30 3X1 + 2X2  24 X1 + X2  3 and X1, X2  0 Is the solution unique? If not give two different solutions. |
| --- | --- | --- |

2.a)	Explain the assignment algorithm

b)	Five workers are available to work with the machines and the respective costs    (in rupees) associated with each worker and machine assignment is given below. A sixth machine is available to replace one of the existing machines and the associated costs are also given below.

Machines

|  |  | M1 | M2 | M3 | M4 | M5 | M6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Workers | W1 W2 W3 W4 W5 | 12 4 8 - 5 | 3 11 2 7 8 | 6 - 10 8 9 | - 5 9 6 4 | 5 - 7 12 6 | 8 3 5 10 - |

- Determine whether the new machine can be accepted?

- Also find the optimal assignment

3.a)	Gives a brief description of the various types of queues.

- In a color TV manufacturing plant, a loading unit takes exactly 10 minutes to load two TV sets at a time into a wagon and again comes back to the position to load another set of TVs. If the arrival of TVs  is a Poisson stream at an average of 2 TVs every 20 minutes, calculate the average waiting time of 2 TV sets in a stationary state.

Contd…2

# Code No: 311303 				-2-				Set.No.1

4.a)	 Distinguish between a Travelling salesman problem  and a Assignment problem

b)	 Solve the following travelling salesman problem

To

|  |  | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- | --- |
| From | A B C D E | - 3 6 2 3 | 3 - 5 2 3 | 6 5 - 6 4 | 2 2 6 - 6 | 3 3 4 6 - |

5.a)	What is safety stock? Why are safety stocks necessary?

b)	A company uses annually 48,000 units of raw material costing Rs. 1.25 per unit.

Placing order costs Rs. 45 and carrying cost is 15% of average inventory. Find

the EOQ . Suppose the company follows EOQ policy and it operates for 300

days in a year. The procurement time is 12 days  with a safety stock of 500 units.

Find the  re-order point, the maximum, minimum and the average inventory.

6. 	There are two offers of coal handling equipment in a thermal power station

Offer A: cost: Rs.20,00,000	capacity:200 tons/hr

Block		 I	II	III	IV	V	VI

Operating-cum-

maintenance cost

per year

(in Rs thousand)	120	130	140	160	190	220

Resale (salvage value)

(in Rs thousand)	1600	1550	1450	1300	1100	800

Offer B: cost: Rs.40, 00,000	capacities: 300 tons/hr

Block		I	II	III	IV	V	VI

Operating-cum-

Maintenance cost

per year

(in Rs thousand)	140	145	151	160	172	190

Resale (salvage value)

(in Rs thousand)	3500	3400	3250	3050	2800	2500

Each block is of 5 years duration. Which offer should be accepted consistent with optimum replacement policy for minimum average annual cost?

Contd…3

# Code No: 311303 				-3-				Set.No.1

7.a)	Briefly explain “dominance property”.

b)	A and B play game in which each has three coins 5p, 10p and a 20p. Each selects

a coin without the knowledge of the others choice. If the sum of the coins is an

odd amount, A wins B’s coin. If the sum is even B wins A’s coin. Find the best

strategy for each player and the value of the game.

8.a)	What are the reasons for stocking items in inventory?

b)	A manufacturing company purchases 10000 parts of a machine for its annual requirements ordering one month usage at a time. Each part costs Rs.30/-. The ordering cost per order is Rs.25/- and the carrying charges are 25% of the average inventory per year. You have been asked to suggest a more economical purchasing policy for the company. What advice would you offer, and how much would it save the company per year?

$$$

Code No: 311303

Set No:

2

III B.Tech  I-Semester (Supplementary) Examination June, 2003

OPERATIONS RESEARCH

(Electronics and Control Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks.

- - -

| 1.a) |  | Explain in brief primal and dual problems. |
| --- | --- | --- |
| b) |  | Solve the following LPP Minimize Z=X1 + X2 Subject to 2X1 + X2  4 X1 + 7X2  7 And X1, X2  0 |

2.a)	Distinguish between assignment and transportation problems.

b)	The owner of a small machine shop has  four machinists available to assign to jobs for the day. Five jobs are offered with expected profit for each machinist on each job as follows.   			Jobs

|  |  | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- | --- |
| Machinist | M1 M2 M3 M4 | 12 12 37 0 | 28 34 42 14 | 0 11 61 37 | 51 23 21 27 | 32 9 31 30 |

Assign machinist to jobs which results in overall maximum profit

3.a)	Name two applications of traveling sales man problem

b)	A machine operator process five types of item on his machine each week, and must choose a sequence for them. The set up cost per change depends on the items presently on the machine and item to be made according to the following table.

To

|  |  | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- | --- |
| From | A B C D E | - 4 7 3 4 | 4 - 6 3 4 | 7 6 - 7 5 | 3 3 7 - 7 | 4 4 5 7 - |

If he produces only one piece of  item in a week, how should he sequence the 	items on his machine in order to minimise the total set up cost.

Contd….2

# Code No: 311303 				-2-				Set.No.2

- Two repairpersons are attending five machines in a workshop. Each machine breaks down according to a Poisson distribution with mean three per hour. The repair time per machine is exponential with mean 15 minutes.

(a) Find the probability (i)	That the two repairpersons are idle

(ii)	That one repairperson is idle

(b) What is the expected number of idle machines not being serviced?

5.a)	A manufacturing concern has a demand of  10,000 units of an item. Each item costs Rs.2, ordering cost is Rs.80 per order and inventory carrying cost is 25%, past lead times are 15,13,25,12 and 10 days.

Calculate:	 i) EOQ

ii ) Safety stock

iii) Normal lead time

iv) Average inventory

v) Reorder level.

b)	Explain briefly VED analysis.

6.	A large computer has 2000 components of identical in nature which are subjected to failure as per the probability distribution given below:

Week end			1	2	3	4	5

Probability of failure	0.10	0.25	0.50	0.80	1.00

If the cost of individual replacement per unit is Rs.3 and for group replacement per unit is Re.1, assess which of the replacement would be economical and when?

7.a)	Define the terms

i)	Inventory.	ii)	Economic Order Quantity.	iii)	Lead time.

b)	A Manufacturing company has determined from an analysis of its accounting and production data for a certain part that.

i)	Its demand is 9000 units per annum and is uniformly distributed over the year.

ii)	Its cost price is Rs. 2 per unit.

- Its ordering cost is Rs 40 per order

- The inventory carrying charge is 9 percent of the inventory value. Further it is known that the lead time is uniform and equals 8 working days, and that the total working days in a year are 300. Determine

  - Economic order quantity

  - Optimum number of orders per annum

  - Total variable cost

  - Reorder level

  - The length of the inventory cycle.

# Contd…3

# Code No: 311303 				-3-				Set.No.2

8.a)	For the following pay-off matrix, find the optimal strategies and the value of

the game. Use algebraic method.

B

1	2	3

1	40	50	-70

A	2	10	25	-10

3	100	30	60

b)	Explain clearly optimal strategy and pure strategy

$$$

Code No: 311303

Set No:

3

IV B.Tech  I-Semester (Supplementary) Examination May, 2003

OPERATIONS RESEARCH

(Electronics and Control Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks.

- - -

| 1. |  | Write the dual of the following LPP: Minimize Z = 3X1 + 5X2 + 6X3 Subject to X1 + 4X2 + 6X3  5 2X1 + 3X2 + 5X3  4 3X1 + X2 + 7X3 = 3 and X1, X2, X3  0 |
| --- | --- | --- |

2.a)	What are the conditions to be satisfied in order to convert a 3 machine sequencing problem into a 2 machine sequencing problem?

b)	State Johnon’s algorithm for sequencing of jobs in machines.

c)	Find the optimal sequence, the minimum total elapsed time and the idle

time. The processing is in the order G F H.

| Job Machine | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- |
| G H F | 12 7 3 | 8 10 4 | 7 9 2 | 11 6 5 | 10 10 5 | 5 4 4 |

3.a)	What are the route conditions for travelling salesman problem?

b)	A salesman wants to visit 4 cities B, C, D and E and come back to his head quarters A. the distance matrix for these cities is given below. Find the optimal route for the salesman and the corresponding total distance he has to cover in one such cycle.

To

|  |  | A | B | C | D | E |
| --- | --- | --- | --- | --- | --- | --- |
| From | A B C D E | 0 50 90 20 140 | 50 0 40 80 70 | 90 40 0 120 110 | 20 80 120 0 65 | 140 70 110 65 0 |

Contd…2

# Code No: 311303 				-2-				Set.No.3

4.	Jobs arrives at a workstation in a manufacturing plant is in a Poisson fashion at an average rate of five per hour. The time to machine one job is an exponential distribution with a mean time of 20 minutes. What is the expected time a job has to wait at the workstation? What will be the average number of jobs waiting at the workstation at any time? What is the probability that there will be more than four jobs?

5.a)	Write a note on periodic review inventory system and Fixed order quantity

system.

b)	For a periodic review system find out the various parameters for an item with the

following data;

Annual consumption 14,000 units

Cost of one unit Rs.10.

Suppliers minimum quantity 1000 units.

Normal lead time 10 days

Maximum lead time 15 days.

Maximum consumption 1.20 times average consumption.

6.a)	Briefly explain what is meant by “individual and group replacement policy” in Replacement Analysis.

b)	There is a special light bulb that never lasts longer than 2 weeks. There is a chance of 0.3 that a bulb will fail at the end of first week. There are 100 new bulbs initially. The cost for individual replacements is Rs.1.25 and the cost per bulb for group replacement is Re.0.50. Is it cheaper to replace all the bulbs,

i)	individually

ii)	every week

- every second week

7.a)	Discuss the impact on Economic Order Quantity if shortages are allowed.

b)	The demand for an item is deterministic and instantaneous time and it is equal to 600 units per year. The per unit cost of the item is Rs.40/- while the cost of placing an order is Rs.10/- The inventory carrying cost 20% of the cost of inventory per annum and the cost of shortage is Rs.2/- per unit per month.Find the optimal ordering quantity when stock outs are permitted. If the stock outs are not permitted, what would be the loss to the company?

Contd…3

# Code No: 311303 				-3-				Set.No.3

8.a)	For what value of ‘a’, the game with the following pay-off matrix is strictly determinable?

Player B

B1	B2	B3

A1	  a	6	  2

Player A	A2	-1	a	-7

A3	-2	4	  a

b)	Differentiate between strictly determinable games and non strictly determinable games.

$$$

Code No: 311303

Set No:

4

IV B.Tech  I-Semester (Supplementary) Examination May, 2003

OPERATIONS RESEARCH

(Electronics and Control Engineering)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks.

- - -

| 1.a) |  | Discuss Scientific method in OR. |
| --- | --- | --- |
| b) |  | Write down the dual of the following LP Problem and solve it. Maximize Z = 4X1 + 2X2 Subject to the constraints -X1 - X2  -3 -X1 + X2  -2 and X1, X2  0 |

2.a)	Write the condition for Johnson’s algorithm to be applicable in finding the       	optimal sequencing order of ‘n’ jobs through 3 machines.

b)	Find the sequence that minimises the total elapsed time required to complete the 	following jobs. Also find the minimum total elapsed time.

| Jobs Machine | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| A B C | 5 2 3 | 7 1 7 | 6 4 5 | 9 5 6 | 5 3 7 |

3.a)	Name three methods of finding an initial basic feasible solution to a transportation problem.

b)	Four gasoline dealers A,B,C and D require 50,000, 40,000, 60,000 and 40,000 gal of gasoline respectively.  It is possible to supply these demands from locations 1,2 and 3, which have 80,000, 1,00,000 and 50,000 gal respectively.  The costs for shipping 1,000 gal of gasoline  are given in the table such that costs Rs.70 per 1000 gal to ship gasoline from location 1 to dealer A, Rs 80 per 1000 gal from location 2 to dealer B, etc.

Cost of shipping 1000 gas of gasoline

|  | A | B | C | D |
| --- | --- | --- | --- | --- |
| 1 | 70 | 60 | 60 | 60 |
| 2 | 50 | 80 | 60 | 70 |
| 3 | 80 | 50 | 80 | 60 |

Determine the amounts of gasoline to be shipped from each location to each 	dealer so that all the dealers’ requirements are satisfied and the total shipping 	costs are a minimum.

Contd…2

# Code No: 311303 				-2-				Set.No.4

4. 	A box-office ticket window is manned by a single server. Customers arrive to purchase tickets according to a Poisson input process with a mean rate of 30 per hour. The time required to serve a customer has an exponential distribution with mean of 80 seconds. Determine the following:

  - Fraction of time the server is busy.

  - The probability of having more than 12 customers in the system.

  - The average number of customers queuing for more than 3 minutes.

5.a)	What is inventory build up? Explain the reason for buildup.

b)	A shopkeeper sells one of the items at a profit of Re.1 per kg. He disposes balance items at a loss of Rs.0.25 per kg. If demand is rectangular between 3000 and 4000kgs, determine optimum quantity he should purchase each day. If the demand is known to be rectangular with probability distribution function   f(R) = 0.02- 0.0002R, find out the optimum quantity he should purchase daily.

6.a)	Explain group replacement concept and it’s applications.

b)	Find the cost per period of individual replacement policy of an installation

of 300 bulbs given in the following:

i) Cost of replacing individual bulb is Rs 3/-

ii) Conditional probability of failure is given below

Week no.                       0	   1	    2	    3	    4

Conditional prob.	  0	  1/10    1/3	   2/3        1

of failure

7.a)	Briefly explain the properties found in competitive games

b)	Reduce the following game by dominance and find the game value:

Player B

I	II	III	IV

I	3	2	4	0

Player A	II	3	4	2	4

III	4	2	4	0

IV	0	4	0	8

8.a)	Write a short notes on Inventory models with price breaks.

b)	A company uses annually 24,000 units of raw material which costs Rs. 1.25 per unit. Placing each order costs Rs.22.5 and the carrying cost is 5.4% per year of the average inventory. Find the Economic Order Quantity and the total inventory cost. Should the company accept the offer made by the supplies of a discount of 5% on the cost price on a single order of 24000 units?

$$$