Code. No.: 311304

Set No.

1

III-B.Tech. I-Semester Supplementary Examinations June 2003

PROCESS CONTROL

(Electronics and Control Engineering)

Time: 3 hours							     Max. Marks:80

Answer any Five questions

All questions carry equal marks

- - -

1.a)	What are the basic elements/ blocks necessary in process control loop? Explain, the importance of each element with examples.

b)	Explain the following terms as applicable to system evolution with necessary graphs.

i)	Stability.	ii)	Measure of quality.

2.a)	Explain the following terms as applicable to process control

i)	Control lag.		iii)	Cycling.

ii)	Dead time.		iv)	Error.

b)	A Controller outputs a 4-20m. A signal to control motor speed from 140 – 600rpm with a linear dependence. Calculate.

i)	Current corresponding to 310 rpm and

ii)	The value of expressed as the percent of control output.

3.a)	Explain the detailed procedure for Ziegler – Nichols method of PID settings.

b)	In the Ziegler – Nichols method, the critical gain was found to be 4.2 and the critical period was 2.21 minutes. Find the standard setting for

i)	Proportional mode control	ii)	PI control       iii)	PID control

4.a)	Explain, pneumatic proportional controller with neat sketch and its applications.

b)	A proportional pneumatic controller has A1 = A2 = 5cm2, X1 = 8 cm, X2 = 5 cm.

The input and output pressure ranges are 3-15 psi. Find the input Pressures that will drive the output from 3 – 15 psi. The set point pressure is 8 PSI and P0 = 10 PSI.  Find the proportional band.

5.a)	What is a ratio control system? Discuss such a control system with a specific process.

b)	Where would you use a split – range control? Mention some fields of application of such a control.

6.	Write a short notes on: (a) Reaction rate.	(b) Exothermic reactors.

7.a)	Discuss the temperature control of endothermic reactor with relevant diagrams.

b)	Explain the control of preheat temperature for regenerative reactor.

8.	Write short notes on the following:

(a)	Configuring the controls.	(b)	Composition feed back.

(c)	Evaporation.

$$$

Code. No.: 311304

Set No.

2

III-B.Tech. I-Semester Supplementary Examinations June 2003

PROCESS CONTROL

(Electronics and Control Engineering)

Time: 3 hours							     Max. Marks:80

Answer any Five questions

All questions carry equal marks

- - -

1.a)	Explain how the basic strategy of control is employed in a room air – conditioning system. What is the controlled variable? What is the manipulated variable? Is the system self – regulating?

b)	Derive the dynamic equations for batch process and continuous process.

2.a)	Discuss Two position controller mode with relevant sketches and its applications.

b)	A liquid level control system linearly converts a displacement of 2-3 meters in to a 4- 20m.A control signal. A relay serves as the two – position controller to open or close an inlet value. The relay closes at 12mA and opens at 10mA. Find

i)	The relation between displacement level and current and

ii)	The neutral zone or displacement gap in meters.

3.a)	Explain  pressure sensing elements used in various measurements.

b)	Derive an expression for second order response of a measuring element.

4.a)	Describe with a neat diagram the operation of hydraulic actuators.

b)	Explain the principle of operation of self operated controllers.

5.a)	Explain the following terms.

i)	Quick opening valve.

ii)	Control valve sizing

- Cavitation.

b)	An equal percentage valve has a maximum flow of 50 m3/s and a minimum of 2m3/s. If the full travel is 3cm, find the flow at a 1 cm opening.

6.a)	What is heat transfer? Explain the principle of heat exchange with neat sketch.

b)	Discuss the combution control with relevant examples.

7.	Write short notes on

i)	Drum – Level Control.

ii)	Drum – Pressure Control.

8.	Explain the typical distillation column with fine primary controlled variables, two compositions, two level and pressure with the help of neat process diagram.

$$$

Code. No.: 311304

Set No.

3

III-B.Tech. I-Semester Supplementary Examinations June 2003

PROCESS CONTROL

(Electronics and Control Engineering)

Time: 3 hours							     Max. Marks:80

Answer any Five questions

All questions carry equal marks

- - -

1.a)	Derive the dynamic equations for resistance and capacitance of liquid process and thermal process.

b)	Explain the following terms.

i)	Process Load.		ii)	Process lag.	iii)	Process Equation.

iv)	Self – regulation.

2.a)	Discuss process reaction method for control loop tuning.

b)	What are the stability methods adopted to PID controller for optimum settings.

3.a)	Derive an expression for electronic proportional controller with an example.

b)	A proportional controller with scaling so that 0 – 10 Volts corresponds to a

0 – 100% output. If RZ = 10kΩ  and full – scale error range is 10 volts. Find the values of V0 and R1 to support a 20% proportional band about a 50% zero error controller output.

4.a)	What is feed forward control? Explain, Feed forward control scheme with the help of block diagram.

b)	Discuss the properties of inner loop of a cascade control.

5.a)	When are single – seated and double – seated valves used? List and compare their advantages and disadvantages.

b)	What is the Cυ factor of a control valve? How is it useful in valve selection and sizing.

6.a)	Discuss the steam – temperature control.

b)	A reboiler is heated with hot oil of varying temperature. How can heat transfer rate be controlled?

7.a)	Discuss various pumps and compressors as applicable to process control.

b)	Explain the fixed heaters used in refineries.

8.a)	With a neat sketch, explain the operation of barometric condensers

- A triple effect evaporator concentrates fructose from 35 to 70 weight percent solids. The density of the feed is 1.54 g/ml. Estimate the steam flow in pounds per hour required to concentrate each gallon per minute of feed.

$$$

Code. No.: 311304

Set No.

4

III-B.Tech. I-Semester Supplementary Examinations June 2003

PROCESS CONTROL

(Electronics and Control Engineering)

Time: 3 hours							     Max. Marks:80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Discuss the following terms.

i)	Capacity.		ii)	Process Variables.

iii)	Self – regulation	iv)	Single time - Constant liquid process.

b)	The deviation e of a PI controller is sinusoidal. Calculate the phase lag of the

output m, and prove that the phase lag depends on integral time.

2.a)	Explain the following terms.

- Proportional band.	iii)	Integral time.

- Off set.			Iv)	Phase shift

.

b)	A Controlling variable is a motor speed that various from 800 to 1759 rpm. If the speed is controlled by 25 to 50 volts dc signal, calculate.

i)	The speed produced by an input of 38 volts

ii)	The speed calculated as a percent of spam.

3.a)	List various temperature measuring elements and their applications.

b)	A type thermo couple measures 22.5mV with a 00C reference. What is the junction temperature?

4.a)	Illustrate the procedure for turning of cascade controllers.

b)	Discuss the ratio control system with relevant sketch.

5.a)	Explain the principle of operation of current – to – pressure converter.

b)	Discuss various types of actuators.

6.a)	Discuss the heat exchangers without phase change with neat diagram.

b)	How combustion efficiency is depending on control of fuel and air? Explain.

7.	Write short notes on

a)	Reaction rate.		b)	pH Control.

8.a)	With the help of an example, explain feed forward control of pH.

b)	Calculate the pH at neutrability for 1 N solution of acetic acid neutralized by caustic. Estimate the process gain dpH/dxB at that point. Assuming missing data.

$$$