Code No: 311402

Set No.

1

III-B.Tech I-Semester Supplementary Examinations, June 2003

DIGITAL ELECTRONICS

(Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	The input to a high pass RC circuit is periodic and trapezoidal as shown below.  Assume that the time constant RC is large compared to either T1 or T2.  Find and sketch the steady state output if RC=10T1 = 10 T2.

b)	Draw the Schmitt trigger circuit and explain its function.

2. a)	A periodic ramp voltage shown below is applied to a low pass RC circuit.  Find the equations from which to determine the steady state output waveform.

- What are the transistor clippers?  Explain their functions.

3. a)	In the circuit shown below if Rs = 50, Rf = 100, Rr= and R = 10k,                C = 0.1µF  and the diode break occurs at Vr = 0.2V.  The input signal Vs is as shown below.  Draw the output waveform and label all voltages.  Assume initially capacitor is uncharged.

Contd…2

Code No. 311402					.2.			Set No. 1

b)	Construct a circuit which exhibits the following transfer characteristics.

4. a)	Draw the logic diagram corresponding to the following Boolean expressions with and without simplication.

(i)	(A+B) (C+D) (A+ B +D)

(ii)	(AB + AB)  (C D+CD)

b)	Convert the following expressions into sum of products and product of sums.

(i)	(AB + C) (B + CD)

(ii)	x+x(x+y)(y+z)

5. a)	Simplify the following functions.

(i) F = ABCE+A B C D + B D E + B C D + CD E + BD E

(ii) F(w, x, y) =  (0, 2, 4, 5, 6, 7, 8, 10, 13, 15)

- Simplify the following function and implement with 2 level NOR gate circuits

F = wx + y z + w y z

- Draw the circuit diagram of Full subtractor and explain its working.

6. a)	Design a  combination circuit that detects an error in the representation of a decimal digit in BCD output of the circuit must be 1 when the inputs contain any one of the six unused bit combinations in the BCD code.

b)	Construct a 16 X 1 MUX with 8 X 1 MUXs and 2 X 1 MUXs.

7. a)	Design a Right shift 4 bit register.

b)	Design a BCD ripple counter and explain its output.

8.	Write short notes on

(a)	Linear current sweep circuits  	(b)	Universal gates.

###

Code No: 311402

Set No.

2

III-B.Tech I-Semester Supplementary Examinations, June 2003

DIGITAL ELECTRONICS

(Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	A current pulse of amplitude I is applied to a parallel RC combination. Plot to scale the waveform of the current ic for the cases (a) tp < RC, (b) tp = Rc and (c) tp > Rc and explain.

b)	Construct the circuits which exhibit the following terminal characteristics as shown below.

2. a)	A 10Hz symmetrical square wave whose peak to peak amplitude is 2V is impressed upon a high pass circuit whose lower 3dB frequency is 5Hz.  Calculate and sketch the output waveform.  What is the peak to peak output amplitudes.

b)	For the circuit shown, Rf = 100, Vi = 20Sinwt, Rr =  and Vr =0.  Draw the Input and output waveforms and label the maximum and minimum values if R = 10k.

3. a)	Draw an astable circuit and explain its working.

b)	What are the transistor clippers?  Explain their functions.

4. a)	Convert (0.513)10 to octal.

b)	Using 10’s complement, subtract 72532-3250.

c)	Write a short note on different codes.

Contd…2

Code No. 311402				.2.				Set No. 2

5. a)	Simplify the following Boolean expression to a minimum number of laterals.

(i)	 xy + x(wz + wz)

(ii)	(BC + AD)(AB + CD ).

- Convert the following function to sum of product form.

(i) f(A, B, C, D)	= 	(0, 2, 6, 11, 13, 14)

(ii) f(x, y, z)		=	(0, 3, 6, 7)

6. a)	Design a combinational circuit that adds one to a 4 bit binary number A3 A2 A1 Ao.  Design using half adders.

b)	Determine the Boolean functions for F and G as a function of four inputs, A, B, C and D in figure.

7. a)	Construct a 4 X 16 decoder using two 3 x 8 decoding.

b)	Implement the following function with a MUX F(A, B, C, D) =  (0, 1, 3, 4, 8, 9, 15)

8	Write short notes on:

(a) Transistor as a switch

(b) Crystal controlled clock generator.

###

Code No: 311402

Set No.

3

III-B.Tech I-Semester Supplementary Examinations, June 2003

DIGITAL ELECTRONICS

(Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	A symmetrical square wave of peak to peak amplitude V and frequency f is applied to a high pass RC circuit.  Show that the percentage tilt is given byP = 1−e−12fRCe−12fRC X 200%.

b)	The limited ramp is applied to an RC differentiator.  Draw to scale the output waveform for (a) T=RC 	(b) T=0.2RC	and 	(c) T = 5RC.

2. a)	The i/p voltage Vi to the clipper shown below is 1 µsec pulse whose voltage varies between 0 and 10V.  The forward diode resistance is Rf = 100, Vr = 0.5V and    Rr = .  Sketch the output waveform Vo and indicate the time constant of the exponential portions.

- What are the different clamper circuits. Explain them.

3. a)	A symmetric 10kHz square wave whose peak excursions are 10V with respect to ground is impressed upon the diode clamping circuit shown below.  If R=10K, C = 1µf and the diode has R1 = , Rf = 0, Vr = 0 and the source impedance is zero.  Sketch the output waveform.

Contd…2

Code No. 311402				.2.				Set No. 3

- Draw the monostable circuit and explain its working.

4 a)	Convert the following numbers to decimal (i) (12121)3	(ii) (4310)5

(iii) (198)12.

- Perform subtraction with the given unsigned numbers by taking 2’s complement.

11010 – 10000.

5. a)	Obtain the truth table of the following function. f = (xy +z) (y + xz).

b)	Simplify the Boolean function using k-map.f(w, x, y, z) = (0, 1, 2, 4, 5, 6, 8, 9, 12, 13, 14).

6. a)	A logic circuit implements the following Boolean functions F= AC + A C D.  It is found that the circuit input combination A = C = 1 can never occur.  Find a simpler expression for F using don’t care conditions.

b)	Design a combinational circuit with three inputs and six outputs.  The output binary number should be the square of the input binary number.

7. a)	Design a BCD to decimal decoder using the unused combinations of the BCD code as don’t care conditions.

b)	What is “race around condition”.  How is this overcome in JK Master slave flipflop.  Explain.

8.	Write a short notes on:

(a) Triggering Techniques.

(b) LCD display system.

###

Code No: 311402

Set No.

4

III-B.Tech I-Semester Supplementary Examinations, June 2003

DIGITAL ELECTRONICS

(Mechatronics)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	The pulse from a high-voltage generator rises linearly for 0.05µsec and then remains constant for 1 sec.  The rate of rise of the pulse is measured with an RC differentiating circuit whose time constant is 250p sec.  If the +ve output voltage from the differentiator has a maximum value of 50V, what is the peak voltage of the generator.

b)	The periodic wave form shown below is applied to an RC integrator whose time constant is 10µsec.  Sketch the output. Calculate the maximum and minimum value of output voltage with respect to ground.

2. a)	In the circuit shown below if Rs= Rf=50, Rr = , R = 10K, C = 1.0µF and the diode break occurs at Vr =0.  The input signal Vs is as shown.  Draw the output waveform  and label all voltages.  Assume initially capacitor is uncharged.

- Draw the bistable circuit and explain its working.

Contd…2

Code No. 311402				.2.				Set No. 4

3. a)	Draw the complete loop gain characteristic of Schmitt trigger circuit and explain its operation.

b)	The diode has resistance Rf = 1K, Rr = 100k.  Assume Vr = 0.  Draw the output waveform Vo and label all voltage levels and time constants.

4. a)	Represent the decimal number 8620 in (a) BCD, (b) Excess-3code (c) 2421 code and (d) as a binary code.

b)	Reduce the following Boolean expression to the indicated number of literals.

(i)	(A + C) ( A + C ) (A + B CD) to four literals

(ii)	A B D + C  D) + B(A + A CD)to one literals.

5. a)	Simplify the following Boolean function.

F(A, B, C, D) =  (0, 6, 8, 13, 14) +   (2, 4, 10)

- Simplify the following

F(A, B, C, D)	=  (0, 1, 2, 3, 4, 10, 11)

6. a)	Implement the following Boolean function F together with the don’t care conditions using no more than two NOR gates.  Assume that both the normal and complement inputs are available.

F(A, B, C, D)	=  (0, 1, 2, 9 11) +   (8, 10, 14, 15).

b)	Design a combinational circuit with 4 inputs that represents a decimal digit in BCD and 4 outputs that produces the 9’s complement of the input digit.

7. a)	Implement the given Boolean expression using Ex-OR and AND gates only.

F = AB C D + A B C D + A B C D + A BC D.

b)	Implement a Full adder with two 4 x 1 MUXs.

8.	Write short note on:

(a) 	Boot strap and Milla sweep circuits.

(b)	Dot Matrix display.

###