Code No: 311901

Set No.

1

III-B.Tech. I-Semester (Supplementary) Examination, June 2003

DIGITAL SYSTEMS DESIGN

(Electronics and Computer Engineering)

Time: 3 hours							        Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	The cubical form of a Boolean function is as follows f=0112+1002+1221+2112. Find all the intersecting pairs of cubes without the help of map.

b)	Find out the CAMP-II print out for the following function

f = (0, 2, 4, 5, 6, 7, 11, 13, 15) + (1,12)

2.a)	Draw the ASM charts for the following state transitions.

i)   If x = 0, control goes from state T1 to state T2; if x = 1, generate a conditional    operation and go from T1 to T2.

ii)  If x = 1, control goes from T1 to T2 and then to T3, if x = 0, control goes from T1 to T3.

b)	Implement the following Boolean functions with a PLA.

f1 (A, B, C) =  (0, 1, 2, 4)

f2 (A, B, C) =  (0, 5, 6, 7)

3.a)	Draw an ASM chart for a digital system which can detect the sequence 010 and 011 by displaying HZERO and HONE outputs respectively.

b)	Realize the above system using only NAND gates.

4.a)	Derive by the path sensitization method the test vectors for SAO and SAI faults at l and m in the network.

b)	Find out the test vectors for the above circuit where both l and m has SAO and SAI faults simultaneously.

5.a)	Apply the IISC algorithm and find the EPC and SPC for the following function

f = 1220+2201+0212

b)	What are the characteristics of an folded compatibility matrix of a given PLA.

(Contd…2)

Code No: 311901                                      	-2-                                     	Set No.1

6.a)	What is a successor tree, what are its properties.

b)	Find out the distinguishing sequence for the given machine

| q\x | 0 | 1 |
| --- | --- | --- |
| A | D, 0 | C, 0 |
| B | C, 1 | D, 1 |
| C | B, 1 | A, 0 |
| D | A, 0 | A, 0 |

7.a)	Obtain the minimal synchronizing sequence for the given machine

| q\x | 0 | 1 |
| --- | --- | --- |
| A | A | B |
| B | C | C |
| C | C | D |
| D | D | A |

b)	When can you say a machine can be diagnonizable? Explain.

8.	Write short notes on:

a)	DFT goals and schemes.

b)	BIST design.

***     ***     ***

Code No: 311901

Set No.

2

III-B.Tech. I-Semester (Supplementary) Examination, June 2003

DIGITAL SYSTEMS DESIGN

(Electronics and Computer Engineering)

Time: 3 hours							        Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Find out the CAMP printout for the given function

f =  (1, 2, 3, 6, 8, 13, 14, 15).

b)	Define the terms degree of adjacency, set of summing minterm, candidate solution cubes.

2.	Design a digital circuit with three 16 bit registers AR, BR and CR to perform the following operations.

a)	Transfer two 16bit signed numbers (in 2’s complement representation) to AR and BR.

b)	If the number in AR is negative, divide the number in AR by two and transfer the result to register CR.

c)	If the number in AR is positive but nonzero, multiply the number in BR by two and transfer the result to register CR.

d)	If the number of AR is zero, clear register CR to C.

3.a)	List the PLA programming table for the BCD to excess-3 code converter.

b)	What is CPLD? Explain the structure of any one PCD.

4.a)	Two level OR-AND network realize the following function. Find a minimum set of a and b tests using Kohavi’s algorithm.

f=x1+x2x2+x4x1+x2+x3

b)	Using the Boolear difference method, find the test vectors for SAO fault on input line1 and SAI fault on the internal line 2 of the circuit.

(Contd…2)

Code No: 311901                                   	-2-                                     	Set No.2

5.	Find a simple column folding of the PLA whose set of subsumming rows is as follows:

| Column | SSR |
| --- | --- |
| X1 | 1, 2, 6 |
| X2 | 1, 5 |
| X3 | 2, 3, 5, 6 |
| X4 | 3, 4, 7, 9 |
| X5 | 2, 3, 4, 8, 9 |
| X6 | 7, 8, 9 |

6.	Obtain a complete test sequence to test a given sequential circuit. The state table is as follows

| x q | 0 | 1 |
| --- | --- | --- |
| A | D, 1 | C, 0 |
| B | B, 0 | D, 1 |
| C | A, 1 | B, 1 |
| D | A, 0 | C,1 |

7.a)	What are the major goals of designs for testability?

b)	How does fault tolerant VLSI processor arrays function? Explain in detail.

8.	Write short notes on:

a)	Fault masking in PCAs and test generation.

b)	Cube based operations.

***     ***     ***

Code No: 311901

III-B.Tech. I-Semester (Supplementary) Examination, June 2003

Set No.

3

DIGITAL SYSTEMS DESIGN

(Electronics and Computer Engineering)

Time: 3 hours							        Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	What are the different operations in cube based algorithms? Explain them with an example each.

b)	Find the candidate solution cubes and solution cubes of the following function.

f =  (2, 8, 10, 11, 13, 15)

2.a)	Design a combinational circuit using ROM where the circuit accepts a 3 bit number and generates an output number equal to the square of the input number.

b)	Explain how the ASM chart differes from a conventional flow chart.

3.a)	Draw an ASM chart of a JK flip flop and realize the JK flip flop using SR flip flops and other basic gates.

b)	What is the difference between ASICs and PLDs.

4.a)	Consider the circuit given below and find three undetectable faults in the circuit.

b)	Using the path sensitization method. Find the test vectors for SAO and SAI faults on each line of the circuit.

(Contd…2)

Code No: 311901                                   	-2-                                     	Set No.3

5.	Find out the optimum test sequences for detecting the faults  and  of a machine whose state transition tables of fault-free and faults are as follows:

T fault free

| x1x2 q | NS, Z 00 01 10 11 |  |  |  |
| --- | --- | --- | --- | --- |
| Qo | Qo, 0 | Qo, 1 | Qo, 1 | Q1, 0 |
| Q1 | Qo, 1 | Q1, 0 | Q1, 0 | Q1, 1 |

T faulty ()

| x1x2 q | NS, Z 00 01 10 11 |  |  |  |
| --- | --- | --- | --- | --- |
| Qo | Qo, 0 | Qo, 0 | Qo, 0 | Q1, 0 |
| Q1 | Qo, 1 | Q1, 0 | Q1, 0 | Q1, 1 |

T faulty ()

| x1x2 q | 00 01 10 11 |  |  |  |
| --- | --- | --- | --- | --- |
| Qo | Qo, 0 | Qo, 1 | Qo, 1 | Q1, 1 |
| Q1 | Qo, 1 | Q1, 0 | Q1, 0 | Q1, 1 |

6.	Consider the minimal and strongly connected machine as shown below. Determine the complete test experiment.

| x q | NS, Z 0 1 |  |
| --- | --- | --- |
| A | D,1 | E, 0 |
| B | C, 0 | E, 1 |
| C | C, 0 | B, 1 |
| D | C, 0 | C, 1 |
| E | C, 0 | A, 0 |

7.a)	Find a minimized PLA for the following Boolean function

f =  ( 2, 3, 5, 7, 8, 9, 10, 11, 13, 15)

b)	Define the terms  (i) FCM	(ii) Compavion pain in PLA folding algorithm.

8.	Write short notes on:

a)	Faults in PLA.

b)	DFT schemes.

***     ***     ***

Code No: 311901

III-B.Tech. I-Semester (Supplementary) Examination, June 2003

Set No.

4

DIGITAL SYSTEMS DESIGN

(Electronics and Computer Engineering)

Time: 3 hours						Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Determine the CAMP printout for the given function

f =  (2, 3, 4, 6, 7, 8, 12, 15) +  (5, 11, 13)

b)	What are the improvements in CAMP-II algorithm over the CAMP-I.

2.a)	Construct an ASM chart for a digital system that counts the number of ones in a register of length N bit.

b)	Realize the above system using D flip flops.

3.a)	Tabulate the PLA programming table for the given form Boolean functions given below. Minimize the no of product terms

A(x, y, z)	=	(1, 2, 4, 6)

B(x, y, z)	=	(0, 1, 6, 7)

C(x, y, z)	=	(2, 6)

D(x, y, z)	=	 (1, 2, 3, 5, 7)

b)	What are the basic building blocks of an ASM chart?

4.a)	Find a minimum test set for the function realized using AND-OR circuit apply Kohavi’s algorithm.

f=x1x2+x1x3x4+x2x4

b)	A circuit realizes the function Z=x2x3+x3x4+x1x4.  Using the Boolean difference method, find the test vectors for SAO faults on all inputs of the circuit.

5.a)	Define the tunes successor tree, distinguishing tree and homing tree.

b)	Design a complete test sequence for the given diagnosable machine

| x q | 0 1 |
| --- | --- |
| A | B, 0 A, 1 |
| B | C, 0 A, 1 |
| C | A, 1 B, 0 |

(Contd…2)

Code No: 311901                                      	-2-                                     	Set No.4

6.	Obtain the minimum output function for a PLA

f = 0200 + 1102 + 2201 + 0011 + 0010

7.	Obtain the maximum folded column PLA for the given SSR table

| Column | SSR |
| --- | --- |
| A | 14 |
| B | 1, 3 |
| C | 3, 4, 5, 9, 12 |
| D | 1, 2, 5, 10, 11, 15 |
| E | 2, 4, 7, 8, 10, 15, 16 |
| F | 1, 2, 15 |
| G | 6, 8, 11, 13 |
| H | 3, 5, 6, 14, 15 |
| I | 7, 10, 12, 13, 16 |
| J | 4, 9, 16 |

8.	Write short notes on:

a)	Kohave’s algorithm.

b)	DFT schemes.

***     ***     ***