Code No: 312002

Set No

1

III-B.Tech. I-Semester Supplementary Examinations, June 2003.

METROLOGY AND QUALITY CONTROL

(Common to Production Engineering and Mechanical Manufacturing Engineering)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	What are slip gauges? How are they manufactured?

b)	What is wringing and what is its advantage?

2.a)	Describe briefly the principle of operation of an Autocollimator and enumerate its uses in metrology.

b)	Write short notes on:

i)	Materials used for gauges.

ii)	Tolerance on limit gauges.

3.a)	Distinguish between a flat and smooth surface?

b)	Describe the construction and working of MECRIN.

4.a)	Explain the working of an electronic comparator. State its applications.

b)	What is a coarse thread? How will you determine the effective diameter of a taper thread?

5.a)	Compare a three-wire and a two-wire method of measuring the effective diameter of a screw.

b)	Describe briefly the gear tooth vernier caliper.

6.a)	Explain how control limits are fixed for Xand R charts.

b)	For the data given below, construct an appropriate chart for control purposes. The data given are from the inspection records of a foundry.

| Day | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Castings inspected | 42 | 55 | 60 | 71 | 53 | 49 | 61 | 93 | 50 | 65 |
| Number of Defective Castings | 1 | 3 | 1 | 2 | 2 | 9 | 0 | 2 | 5 | 9 |

Give your interpretation of the chart.

7.a)	What does an operating characteristic curve show, and what determines the shape of the curve?

b)	Explain the working of a typical sequential sampling plan and its advantages over other sampling plans.

8.a)	State the main objectives of a quality control programme.

b)	Discuss the concept of total quality management.

^^^

Code No:312002

Set No

2

III-B.Tech. I-Semester Supplementary Examinations, June 2003.

METROLOGY AND QUALITY CONTROL

(Common to Production Engineering and Mechanical Manufacturing Engineering)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Distinguish between line standards  and end standards with suitable examples.

b)	Explain with neat sketches how you measure the angle of taper plug gauge.

2.a)	Compare the CI surface plate with granite surface plates.

b)	What is meant by roughness and waviness of machined surfaces?

3.a)	Explain briefly about any one type of instrument used for obtaining a graphical record of primary surface texture.

b)	State the advantages and disadvantages of a gauging machined shaft by means of limit gauges.

4.a)	Describe with neat sketches the working principles and applications of profile projector.

b)	Describe a back pressure type pneumatic comparator. Explain its use in measurement of precision machined holes.

5.a)	Explain  how the various elements of internal threads are measured.

b)	State and explain Taylor’s Principle of gauging of threads.

6.a)	Explain the importance of normal curve in control charts for variables.

b)	The following tabulation lists the results of inspection of 10 samples lot by 40 units each.

| Sample Number | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Defects | 11 | 2 | 10 | 18 | 5 | 12 | 6 | 3 | 11 | 8 |

Determine the upper and lower control limits and plot the attribute control chart.

7.a)	Write short notes on process capability and process capability index.

b)	Explain the need for acceptance sampling.

8.a)	Draw a neat sketch of OC curve showing its different zones. Also discuss the characteristics of OC curves.

b)	Explain briefly about ‘Quality motivation’.

^^^

Code No:312002

III-B.Tech. I-Semester Supplementary Examinations, June 2003.

Set No

3

METROLOGY AND QUALITY CONTROL

(Common to Production Engineering and Mechanical Manufacturing Engineering)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	How does a dial indicator operate? With a suitable example explain its application in metrology.

b)	How are the slip gauges assembled? Why is it considered to be highly accurate method in linear measurement?

2.a)	Discuss the relative features of the following for angle measurement.

i)	Sine bar	ii)	angle bar	iii)	slip angle gauges.

b)	A straight edge was tested by a spirit level having the vial radius (R) of 60 m and standing on feet 100mm apart the readings are 0.75, 0.5, (-)2.5, 2.0, (-)0.75 divisions which are 2.5mm apart. Find the maximum deviation from linearity from the reference line joining the end points.

3.a)	Explain the terms “waviness” and “roughness”. How is the waviness removed in the measurement of roughness?

b)	Define the term “Limit gauges”. What is the difference between ring gauge, snap gauge and gap gauge?

4.a)	Define CLA, Rt and RMS indices of surface roughness and their relative importance in the evaluation of roughness.

b)	Briefly describe the working principle operation and two typical applications of a profile projector.

5.a)	Discuss the relative merits of mechanical and electrical comparators along with the working principle of any one of them.

b)	Explain the method of measuring the pitch and pitch circle diameter of a gear.

6.a)	Distinguish the following terms with suitable example of each

i)	Measurement		ii)	gauging 	iii)	Comparator.

b)	Explain the two wire method of measuring screw thread effective diameter and the term “best size wire”.

7.a)	What type of statistical distribution are associated with Normal, Binomial and Poisson’s distribution? How are the control limits estimated in each case? Name the associated control charts.

b)	Explain the concept of statistical quality control and process capability.

Contd…2

Code No. 312002				-2-				       Set No.3

8.	Briefly explain the following:

a)	Sequential sampling

b)	Quality assurance

c)	Total quality management.

^^^

Code No:312002

III-B.Tech. I-Semester Supplementary Examinations, June 2003.

Set No

4

METROLOGY AND QUALITY CONTROL

(Common to Production Engineering and Mechanical Manufacturing Engineering)

Time: 3 hours							         Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	List any two Linear measuring instruments with Line and End standards and compare their features.

b)	Explain one method of calibrating slip gauges.

2.a)	Explain the method of angle measurement using precision level. In which situations is this method suitable and unsuitable?

b)	Briefly describe the flatness measurement of any component familiar to you using auto collimator.

3.a)	What is the difference in the terms “profilograph” and “roughness profile” from Taly surf? Explain the function of skid in the measurement of roughness.

b)	What is Taylor’s principle of gauging? Do plug and ring gauges satisfy it? If not, how can they be modified to satisfy Taylor’s principle?

4.a)	Describe the construction and operation of a Talysurf.

b)	Distinguish the following from each other

i)	Roughness and Waviness

ii)	Profile and Position gauges

iii)	CLA and RMS roughness.

iv)	Workshop and Inspection gauges.

5.a)	Discuss the relative merits of mechanical and pneumatic comparators along with the working principle of any one.

b)	Explain the method of measuring in screw threads

i)	Pitch	ii)	effective diameter	iii)	thread angle.

6.a)	What are the possible errors in the geometry of screw threads from their ideal profile? Briefly explain their significance.

b)	Explain the method of checking the gear teeth profile, its addendum and dedendum.

7.a)	What is meant by the term “process capability”? How can it be evaluated?

b)	Distinguish the following from each other.

i)	P chart and C chart

ii)	Sampling inspection and 100 percent inspection

- Variables and attributes

Contd…2

Code No. 312002				-2-				       Set No.4

8.a)	Explain the concept of

i)	Acceptance sampling

ii) 	Zero defect programs

- Rectification plans.

- Briefly describe single and double sampling plans.

^^^