Code No: 312003

Set No.

1

III-B.Tech. I-Semester Supplementary Examination, June 2003

TURBO MACHINERY

(common to Production Engineering and Mechanical Manufacturing)

Time: 3 hours.							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Write a three dimensional Navier-stokes equation of motion and explain the various terminology as applied to compressible fluid and derive an Euler’s equation using above equation.

b)	Bring out the differences between impulse and reaction principles.

2.a)	Discuss the effect of friction during expansion of steam through a C-D nozzle when i) steam at entry to the nozzle is saturated, and

ii) the steam at entry is super heated.

Assume the pressure of steam to be initially same in both the cases.  Mark the process on a h-s diagram.

b)	Differentiate between choked flow and super saturated flow as applied to nozzles.

3.a)	Explain the methods of improvement of performance of open cycle gas turbine plant.

b)	Describe the flame stabilization techniques.

4.	Steam expands in a nozzle under the following conditions:  Inlet pressure=15bar; inlet temperature=2500C, final pressure = 4bar; mass flow= 1kg/s. 	Calculate the required throat and exit areas, using Mollier diagram, when (i) the expansion is frictionless and (ii) the friction loss at any pressure amounts to 10% of the total heat drop down to that pressure.

5.	In a reaction turbine, the blade tips are inclined at 350 and 200  in direction of motion.  The guide blades are of the same shape as moving blades, but reversed in direction.  At a certain place in a turbine, the drum diameter is 1m and the blades are 100mm high.  At this place the steam has a pressure 1.7bar and dryness 0.935.  If the speed of the turbine is 250 rpm and the steam passes through the blades without shock, find the mass of steam flow and the power developed in the ring of the moving blades.

6.a)	Explain the working of an axial flow compressor with a neat sketch.

b)	Differentiate between centrifugal compressor and axial flow compressor.

7.a)	Why is it governing necessary in turbines? Explain any two methods in detail.

b)	What is meant by cogeneration?  Explain any one method of cogeneration.

8.a)	Classify jet and rocket propulsion  systems.

b)      What are athodyds? Explain the working of Ramjet with a neat sketch.												&&&&&

Set No.

2

Code No: 312003

III-B.Tech. I Semester Supplementary Examination, June 2003

TURBO MACHINERY

(common to Production Engineering and Mechanical Manufacturing)

Time: 3 hours.							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	What are the assumption made in one-dimensional isoentropic flow for a compressible fluid.

b)	Derive an expression for change in area, change in velocity and Machumber for a compressible fluid.

2.a)	Write a short notes on Cascade analysis

b)	What are characteristic curves – explain.

3.a)	Explain the effect of intercooling reheat and regeneration  on the performance improvement of gas turbine.

b)	Write on design requirements of a centrifugal compressors.

4.	In a four stage impulse turbine, the steam is supplied at 3500C and at a pressure of 20 bar, the exhaust pressure is 0.05 bar and the overall turbine efficiency is 80%.  Assuming that work is shared equally between the stages and condition line to be straight, determine i) stage steam pressures, efficiency of each stage and reheat factor.

5.a)	Discuss the method of finding the width of impeller blades in a rotary air     compressor.

b)	An axial flow compressor, with compression ratio as 4, draws air at 200C and delivers it at 1970C.  The mean blade speed and the flow velocity are constant through the compressor.  Assuming 50% reaction blading and taking blade velocity as 180 m/s, find the flow velocity and number of stages.  Take work factor=0.82, =120, =420 and    =1.005 kJ/kgk.

6.a)	Write a note on design calculation of a gas turbine plant.

b)	What is Wilson’s line?  Derive an expression for critical pressure of flow through a nozzle.

7.a)	Write basic equations for thrust, useful power and propulsion efficiency, from first principles.

b)	Write a short note on the working of a solid propellant rocket motor, Name any two solid propellants.

8.	Dry and saturated steam at a pressure of 8bar enters a C-D nozzle and leaves it at a pressure of 1.5 bar.  If the flow is isentropic, and the corresponding expansion index is 1.135.  Find the ratio of isoentropic cross-sectional area at exit and throat for maximum discharge.

&&&&&

Code No: 312003

Set No.

3

III-B.Tech. I Semester Supplementary Examination, June 2003

TURBO MACHINERY

(common to Production Engineering and Mechanical Manufacturing)

Time: 3 hours.							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Explain the principle of reaction turbines and how they differ from their counter parts?.

b)	How boundary layer is formed?  Explain how boundary layers going to affect the flow phenomenon.

2.a)	Write a note on Howell’s method of blade design.

b)	Explain the various losses encountered in the working of centrifugal compressors.

3.a)	What is meant by flame stabilization?  Explain the methods of flame stabilization in gas turbine.

b)	Why is it necessary to maintain air to fuel ratios around 100:1 in gas turbine?  Comment on your answer?

4.	Steam is supplied to a simple impulse turbine at 10bar, 2500C.  The pressure in the wheel casing is 1.2 bar and the nozzle efficiency is 91%.  Determine the exit area required for a steam flow rate of 5000kg/h.  The nozzles are inclined at 20o to the plane rotation. The blades are inclined at 200 to the plane rotation. The blades are equiangular and have a velocity coefficient of 0.8.  Assuming that ratio of mean blade speed to steam speed at nozzle exit to be 0.45, calculate the blade angles, power developed and stage efficiency.

5.a)	What is meant by choked flow as applied to centrifugal compressors?  Explain.

b)	Write a note on impeller design of a compressor.

6.	A gas turbine unit receives air at 100kpa and 300k and compresses it adiabatically to  620 kpa with efficiency of the compressor  88%.  The fuel has a heating value of 44.2 MJ/kg and the fuel/air ratio is 0.017 kg fuel / kg air.  The turbine internal efficiency is 90%.  Calculate i) Compressor work ii) turbine work and iii) thermal efficiency.

Contd..2

Code No:312003				-2-			Set No.3

7.a)	What is governing of  steam turbine?  Describe any two methods of governing.

b)	Derive an expression for maximum discharge through a nozzle.

8.a)	Write a note on the requirements of good combustion chamber of a gas turbine?

b)	How propulsion systems are classified?  Write a note on various types of solid propellers materials.

&&&&&

Code No: 312003

Set No.

4

III-B.Tech. I Semester Supplementary Examination, June 2003

TURBO MACHINERY

(common to Production Engineering and Mechanical Manufacturing)

Time: 3 hours.							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	What is meant by compressible fluid?

b)	What is compressibility factor?  What is the effect of Mach number on compressibility factor?

c)	Write a Navier-stoke’s equation of motion for an compressible fluid?  Explain the significance various terms in the equation.

2.a)	Define stage efficiency.  Determine the condition for maximum efficiency of a 50% reaction turbine.

b)	Explain the term “compounding of steam turbine”.  What are the different methods of reducing rotor speed?

3.a)	Describe with the help of a typical aerofoil section, the various terminology including lift and drag forces.

b)	Write a note on cascade testing of blades.

4.a)	Differentiate between centrifugal compressor and axial flow compressor.

b)	With regards to centrifugal compressors, explain the phenomenon of surging and  stalling.

5.	A centrifugal compressor with 70% isentropic efficiency delivers 20kg of air per minute at a pressure of 3bar.  If the compressor receives air at 200C arl at a pressure of a 1 bar, find the actual temperature of air at exit.  Also find the power required to run the compressor, if its mechanical efficiency is 95%.  Take  and Cp of air as 1.4 and 1.005kJ/kg K respectively.

6.a)	Define and explain the following terms as applied to gas turbine

i)  Component efficiencies  ii) specific output  iii)  Regenerator efficiency.

b)	In a gas turbine plant, air is compressed from 1bar and 150C through a pressure ratio of 4:1.  It is then heated to 6500C in a combustion chamber and expanded back to a pressure of 1bar in a turbine.  Calculate the cycle efficiency and work ratio, if a perfect heat exchange is new.  Assume isoentropic efficiency of the turbine and the compressor one as 85% and 80% respectively.  Take CV of fuel as 44MJ/kg.

Contd..2

Code No:312003				-2-			Set No.4

7.a)	What is the effect of friction on the flow through a nozzle?  Describe super saturated flow through a nozzle.

b)	Write a note on cogeneration power plant.

8.a)  	Explain the working of liquid propellant rocket motor with a neat sketch.

b)	What are Athodyds?  Explain the working of “scram” jet.

&&&&&