Code No: 312103

Set No.

1

III-B.Tech I Semester Supplementary Examinations, June 2003

AERODYNAMICS-I

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	Write Euler’s equation for inviscid incompressible flow field.  Hence obtain Bernoulli equation.

b)	State and prove Kelvin’s theorem of constancy of circulation.

2. a)	A velocity field is given by

Vx	=	3+6y-5z

Vy   	=	2-6x + 7z

Vz	=	8 + 5x – 7y

What kind of flow field is represented by above.

b)	Show that stream lines due to a point source are the same as the equipotential lines due to a point vortex.

3.	Obtain distribution of velocity over a circular cylinder µ=µ(), when it is kept in a uniform velocity U. Assume the flow to be irrotational.  Does it generate lift force?

4.	How can you relate the terms starting vortex and kutta condition with reference to flow over an airfoil.

5.	What is Joukowaski transformation? Apply this technique to obtain a symmetrical airfoil from a circle.  Is this airfoil a practical airfoil?

6.	Making use of fundamental equation of thin airfoil theory of Glauert, obtain       Cℓ + Cm over a flat plate of Chord C.

7.	Camber line of an airfoil is given below

yδ= 8.28 (x3 − 158x2 + 78x)

Determine CL and CM1/4.  Hence obtain zero lift angle.

8. a)	Explain the generation of lift over an airfoil and a wing, with sketches.

b)	What is Biot-savart’s law?  How is it used to obtain velocity field from a finite vortex.

###

Code No: 312103

Set No.

2

III-B.Tech I Semester Supplementary Examinations, June 2003

AERODYNAMICS-I

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	Define the term CIRCULATION. Hence prove that integral of the normal component of the curl of velocity vector over any surface A is equal to the line integral of the tangential components of the velocity around the closed curve C which bounds A.

b)	Define an irrotational flow.  Hence prove that curl of grad  = 0 ( x  = 0) where  is the velocity potential.

2. a)	Velocity vector in a flow field is given by qf = i (Az – By) + j (Bx – Cz) +            k (Cy – Ax) where A, B, C are nonzero constants.  Determine the equation of vortex lines.

b)	An incompressible flow field is given by  = ax2 – ay2 where a is a constant. Show that flow is irrotational and hence determine velocity potential for this flow.

3. a)	A flow field is described by a complex potential function w = Azn, where A is a constant and n is real.  Could you obtain flow patterns and sketch the same.

b)	Sketch flow field around an airfoil in an irrotational flow, at an arb. angle of attack.  How does it differ from what it would be in a real or rotational flow?  Illustrate.  Hence define the term starting voltage.

4.	Obtain pressure distribution over a circular cylinder in an ideal incompressible flow.  Hence plot stream lines over it.

Compare the pressure distribution and stream line with that in obtained in wind tunnel.  Comment on the differences.

5. 	Define conformal transformation.  Hence transform a circle with Jonkowaski transformation into

(i)	an Ellipse

(ii)	a flat plate, and

- Circular arc.

6.	Making use of Joukowaski transformation, transform a circle in to a symmetrical airfoil.

7.	Obtain from fundamental considerations and assumptions, Glauert’s equation for a thin airfoil.

Contd…2

Code No. 312103				.2.				Set No. 2

8.	A House shoe vortex of strength  is placed in a flow field.  Considering the figure below obtain net down ward velocity at point p.

y

a

	  	x=3a

y=0.5a

P(x,y)

x

####

Code No: 312103

Set No.

3

III-B.Tech I Semester Supplementary Examinations, June 2003

AERODYNAMICS-I

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	Consider a velocity field given as V→ = 2xyi + (x2 + z2)t2i + y2t2k x, y, z in meters, t is seconds, V→ in m/sec.  Find the acceleration of particle located at       (0, -1, -1) at t=1.

b)	Define stream function and velocity potential function of an inviscid, incompressible flow.  Hence explain how components of a velocity field may be obtained if a stream function or velocity potential function is given.  Are they related?

2. a)	Show that in a given velocity field curl of the velocity is equal to vorticity.

b)	Write Euler’s equation of motion in vectorial form.  Explain each term and its significance.

3. 	How can you model Non-lifting flow over a circular cylinder making use of elementary flows.  Hence show that for inviscid, incompressible flow over a stationary cylinder.

Cp = 1-4Sin2.

4. a)	Show that for steady flow over a lifting airfoil (T.E) = 0.

Make use of sketches to explain your answer.

b)	Prove that for a lifting flow, circulation T, and free steam velocity V are related to Lift force as

L = P V T where P is the density of the fluid.

5.	A long elliptic cylinder of t/c ratio = 1/7 is set in with its major axis parallel to a uniform irrotational stream of 35.4m/s.  Calculate the pressure difference between pressure holes located in the nose and at the point of maximum thickness.

6.	A Cubic camber line for an airfoil is given by Zc = 2.6595 x3c −0.6705 xc2 + 0.1147 xx for 0  xc 0.2025 = 0.02208 1−xcfor 0.2025 xc 1.0 Calculate αCL= 0,  CLα= 4o.

Contd…2

Code No. 312103				.2.				Set No. 3

7. a)	Prove that the strength of vortex remains constant along its length.

b)	State Bio-savarts law.  Hence calculate velocity at a point p, due to a linear vortex of finite length.

8. a)	Consider the fundamental equation of Prandtl’s lifting line theory.  Hence explain three main aerodynamic characteristics of a finite wing.

b)	State Kelvin’s theorem and show that it holds for a lifting airfoil.

###

Code No: 312103

Set No.

4

III-B.Tech I Semester Supplementary Examinations, June 2003

AERODYNAMICS-I

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	A velocity field is given by V→ = 2yi + xj +0k.  Obtain the acceleration vector at x=3, y=1.  At this position obtain components of acceleration parallel and normal toV→.

b)	Euler’s acceleration formula is given by DVDt = ∂v∂t + (V.∇) V.  Explain the above clearly and completely.

2. a)	Define circulation in a flow field.  Hence provecudxudy=A(∂u∂x− ∂u∂y) dxdy.

b)	Show that stream lines and equipotential lines due to a point source are perpendicular to each other.  Is it time for other cases too?

3.	Consider a source in a 2-D uniform stream.  Plot stream lines for this combination, if the source strength is 20m2/sec and is placed at the origin.  Obtain local velocity and position of the stagnation points. (Make use of graph sheet)

4.	Obtain distribution of velocity over a circular cylinder µ = µ (), when it is rotating about its own axis, and kept in a uniform flow of U.  Assume flow to be irrotational.

Also determine lift force, in case it is generated in this case.

5.	Make use of Jonkowaski transformation to obtain a Cambered airfoil from a circle. What kind of trailing edge is obtained?  Is this a practical airfoil?

6.	Making use of fundamental equation of thin airfoil theory of Glauert, obtain        Cℓ and Cm over a symmetrical airfoil.

7.	The camber line of a circular airfoil is given by yc =4h xc (1−xc ).

Obtain CL and CL = 0.

Find the load distribution at an angle , showing separate effects of camber and incidence.  Compare the lift curve of this airfoil with that of a flat plate.

8. a)	Explain the vortex theory of lift over an arbitrary airfoil.

b)	Consider a finite wing of span 2b.  Show the complete vortex system over this wing when it is producing lift.  Make use of sketches in explaining your answer.

###