Code No: 312104

Set No.

1

III-B.Tech I Semester Supplementary Examinations, June 2003

GAS DYNAMICS

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	Define Mach number and compressibility.  Show that the condition of incompressibility for gases is M22≪1.

b)	Establish from the propagation of plane pressure pulse that Sinμ=1M, where µ is mach angle and M is Mach number.

2. a)	Derive the equation of adiabatic steady flow ellipse.  Explain realms of compressible flow through its plot.

b)	A storage chamber maintains air at 2.0 atm g and 18oC.  Considering air as a perfect gas, calculate

(i)	discharge velocity into atmospheric pressure through a unit area hole.

(ii)	Mass flow rate per unit area.

3. a)	Derive area-velocity-relationship for compressible fluid flow.  Explain in details.

b)	Derive area-mach number relationship for compressible fluid flow.  Explain in detail.

4. a)	Describe flow through a convergent-divergent diffuser, making use of oblique shock.

b)	A convergent-divergent diffuser is used at M=3.0.  The diffuser uses variable throat area so as to swallow the starting shock.  What percentage of increase in throat area will be necessary?

(take po1po2=3.046atM=3)

5.	Consider a normal shock in a supersonic flow of M1.  Derive expressions for flow properties across the shock.  If M1 = 3.0, P1 = 19400 Pa, T1 = 220k, work out temperature, pressure and density ratios.  Also work out change in entropy.

What is the shock strength?

6.	What are similarities and differences between normal and oblique shock waves.  Explain in details.

Hence establish that downstream mach number is always subsonic for a normal shock, but not for an oblique shock which is always more than M=1.

Contd…2

Code No. 312104				.2.				Set No.1

7.	Establish the effect of frictional duct upon a supersonic flow with given reservoir and exhaust region pressures.  Make use of T-S diagram for the purpose.

8. a)	Illustrate the phenomenon of shock wave-Boundary layer interaction in a supersonic flow.

b)	Consider a conventional airfoil placed in a supersonic stream.  Explain if the resulting shock wave is

(i) Normal

(ii) Oblique

(iii) Bow shock.

Support your answer with principles of gas dynamics.

###

Code No: 312104

Set No.

2

III-B.Tech I Semester Supplementary Examinations, June 2003

GAS DYNAMICS

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	Derive the basic energy equation for gas dynamic adiabatic flow process.  Hence define stagnation enthalpy and specific enthalpy.

b)	Consider a moving point source generating pressure pulse,  with V=0, to V > speed of sound.  Making use of neat sketches, show and define mach angle and mach line.

2. a)	Air under high pressure in a reservoir is discharged through an opening.  Show that the maximum velocity of discharge is independent of reservoir pressure.  Hence find an expression for mass flow per unit area.

b)	A pressure reservoir maintains air at 1.8atm and 20oC.  Considering air as a perfect  gas, calculate

(i) Discharge velocity into atmospheric pressure, through a unit area hole

(ii) Mass flow rate per unit area.

3. a)	Derive an expression for variation of area with velocity.  Hence show that a supersonic nozzle behaves as a subsonic diffuser.  What is the significance of this expression at M=1.

b)	Consider Bernoulli’s equation for isentropic flow.  Connect actual mach number M with characteristic mach number M*, Hence determine the value of M*, as m→.

4.	Describe flow in a convergent-Divergent nozzle, as a function of pexit.  Explain the terms under expanded and over expanded nozzle.  What is choking?

5. a)	Explain the term SHOCK WAVE in a supersonic flow.  How does it affect the thermodynamics and aerodynamics of the gas flow?  Hence explain

(i)	Normal shock

(ii)	Oblique Shock

- Bow Shock

with neat sketches and descriptions:

b)	With reference to Normal Shock, derive M1M2=1.

6. a)	Derive Normal shock relations from first principles.

b)	A supersonic flow at M=2.0, p1 = 0.5 atm and  t1 = 300k experiences a normal shock.  Work out the down stream properties.

Contd…2

Code No. 312104				.2.				Set No.2

7. a)	Show that in a Rayleigh flow, effect of addition of heat is to drive Mach number towards 1.

b)	Explain the effect of friction on subsonic flow in a pipe flow using h-s diagram of Fanno line.

8. a)	A Mach 3 flow passes over a wedge of angle 1, which further changes into 2 with 2 > 1.  Sketch flow pattern and resulting oblique shocks.  What do you observe?

b)	A supersonic flow at M=3.0 at p1 = 18500Pa, T1 = 220k makes a wave angle          = 32o with free stream. Find the stream properties downstream of the weak shock wave attached to the semi wedge angle .  Determine this angle as well.

###

Code No: 312104

Set No.

3

III-B.Tech I Semester Supplementary Examinations, June 2003

GAS DYNAMICS

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	Define the basic energy equation for gas dynamic adiabatic flow process.  Hence define stagnation enthalpy and specific enthalpy.

b)	Establish from the propagation of plane pressure pulse that Sin µ=1m, where µ is the mach angle and M= Mach number.

2. a)	Air under high pressure in a reservoir is discharged through an opening.  Show that maximum velocity of discharge is independent of the pressure.  Hence find expression for mass flow per unit area.

b)	A storage chamber maintains air at 2.2 atm of 20oc.  Assuming air as a perfect gas, calculate

(i)	Discharge velocity into atmospheric pressure through a unit area hole.

(ii)	Mass flow rate per unit area.

3. a)	A De-Laval nozzle is a convergent-Divergent nozzle.  Derive area-Mach number relationship for it and explain it through a plot.

b)	A De-Laval nozzle was designed for exit Mach number of 2.2 and exit diameter of 230mm.  Find the ratio of throat area/exit area necessary.  The reservoir conditions are po = 2 atm g, To = 20oC.  Find maximum mass flow rate through the nozzle.  What will be exit pressure and temperature.

4.	Consider a convergent-Divergent supersonic nozzle.  Show that the nozzle flow characteristics are function of the local area ratio A/A*.  How does flow get affected with changes/variations in pexit/po.

5.	Derive normal shock relations for a perfect gas.  Give below are free stream conditions:

M1 = 2.5, P1 = 19000 Pa, T1 = 220k

Find properties in the flow field downstream of the normal shock, increase in entropy and shock strength.

6. a)	Explain conditions under which a normal shock is formed in a supersonic flow.  Illustrate one such case with diagrams and plots. Is there any alternate .

b)	How can you employ a Pitot-probe in a supersonic flow to measure total pressure.

Contd…2

Code No. 312104				.2.				Set No.3

7. a)	Explain the effect of frictional duct upon a flow with given reservoir and exhaust region pressures in case of subsonic flows.

b)	Sketch flow around a diamond shaped airfoil belt at α = 0 in a supersonic flow of M=3.0.  Explain the flow field and related characteristics.

8. 	A supersonic stream of air at M=3.0, P1 = 19500Pa and T1 = 220k makes a wave angle of 38o with free stream.  Find stream properties downstream of the weak shock wave attached to the semi wedge of angle .  Determine this angle as well.

###

Code No: 312104

Set No.

4

III-B.Tech I Semester Supplementary Examinations, June 2003

GAS DYNAMICS

(Aeronautical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1. a)	Define Mach number and compressibility.  Show that the condition of incompressibility for gases is M22≪1.

b)	Establish from the propagation of plane pressure pulse that Sin µ = 1M, where µ is the Mach angle and M is Mach number.

2. a)	Classify various regimes of flow of compressible gas by plotting adiabatic steady flow ellipse. Derive the equation used.

b)	Explain the terms:  Enthalpy, stagnation Enthalpy, specific enthalpy, M* , a*, V*, T* (with usual notations).

3. a)	Derive area-Mach number relation with reference to a De-Laval nozzle and plot it.  What do you observe?

b)	A de laval nozzle was designed for exit Mach no of 1.5 and exit diameter of 200mm.  Find the ratio of throat/exit area necessary.  The reservoir conditions are po = 1 atm g, To = 20oC.  Find maximum mass flow rate through the nozzle.  What will be exit pressure and temperature?

4. a)	Describe flow through a supersonic diffuser employing Normal Shock.

b)	A convergent- divergent diffuser is used at M=3.0.  The diffuser uses variable throat area so as to swallow the starting shock.  What percentage of increase in throat area will be necessary.

(At M=3, take po1po2=3.046)

5.	Derive normal shock relations in full details.  Hence establish that flow after a normal shock is always subsonic.

6.	Derive the Hugonoit equation in supersonic flow elaborate all the advantages resulting from this equation.

7. a)	Write a note on ‘Choking’.

b)	Explain the effects of frictional duct upon a flow with given reservoir and exhaust region pressures for subsonic flow.

Contd…2

Code No. 312104				.2.				Set No.4

8.	A supersonic stream of air at M=2, at P1 = 19400Pa T1 = 217k, passes through an oblique shock wave making angle  = 53o with free stream.  Find the stream properties downstream of the weak shock wave, attached to the semi wedge angle .  Determine the semi wedge angle  as well.

Is it that the downstream Mach number M2 > 1, always?  If so, why?

###