Code No.312106

Set No.

1

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

AEROSPACE PROPULSION-I

(Aeronautical Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Classify aircraft Piston Engines in different categories.  Describe merits of each.

b)	Explain why airplanes utilize piston-engines at low speeds and low altitudes.

2.	Show with a sketch/line diagram the action of super charging in a prop-piston engine combination.

3.a)	Explain the terms: IHP, BHP, SHP, THP.  How are these inter-related for a Prop-Piston Engine combination.

b)	Show with sketches/Blocks, a typical lubrication system of a piston-engine used in airplanes.

4.a)	Explain the role of a compressor and a turbine in a Gas turbine engine.  Make use of T-S diagrams. How is a turbine different for a Jet-engine and a Turbo-prop engine?

b)	Compare a centrifugal compressor with an axial flow compressor.

5.	A 10 stage axial flow compressor provides an overall pressure ratio of 5:1, with overall isentropic efficiency of 87%, with temperature at inlet being 15oC.  The work is supposed to be divided equally between the stages.  A 5% reaction is used with blade speed of 210 m/sec with constant axial velocity of 170 m/sec.  Estimate the blade angles, assuming a work done factor of 1.0.

6.a)	Explain and show with neat sketches, inlet and exit velocity triangles for various types of blades of a centrifugal compressor.

b)	How do you define degree of reaction for an actual compressor stage for axial flow compressors?

7.a)	Explain how a flame-tube keeps on getting cooled during the process of combustion.

b)	Describe one method of stabilizing the flame of a combustion chamber in a Jet-Engine.

8.a)	Explain difference in the profile of an axial flow compressor and an axial flow turbine.

b)	Considering an axial flow turbine, define: i) Stage efficiency   ii) Blade efficiency

How are they different and related?

@@@@@

Code No.312106

Set No.

2

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

AEROSPACE PROPULSION-I

(Aeronautical Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	What are the considerations and requirements of an aircraft piston engine. Are these same or different from that for a road vehicle?

b)	What are the over-riding factors dictating the arrangement of multi cylinders in an aircraft engine?  Explain one such combination of cylinders with sketches.

2.a)	The performance of aircraft piston engine deteriorates at altitudes.  What are the causes and remedies if any.

b)	Write a note on the combustion process and power stocks of a typical four-cylinder, four stroke piston engine.

3.a)	What are the advantages of direct fuel injection in piston engines.

b)	Explain the phenomenon of back-firing in piston engines.

4.	Draw an arrangement of compressor, combustion chambers and turbine as in a typical Gas Turbine Engine.   How do pressure, temperature and velocity vary across the engine?  Explain with illustrations.

5.	Show with a sketch the arrangement for an axial flow compressor stage, consisting of inlet guide vanes, Rotor blades and stator blades.  Hence draw velocity triangles and explain the variation occurring in the flow properties resulting in pressure rise per stage.

6.	A centrifugal compressor under test gave following data.

Rpm				=	13,150

Inlet total head temp.		=	22oC

Out let total head pressure	=	4 bar

Air Flow			=	141500 kg/h.

Impellor dia			=	75 cms.

If the slip factor is 0.92, work out the compressor efficiency.

7.a)	What are typical arrangements of flame tubes in an airplane Jet engine. Explain with sketches.

b)	Explain various techniques employed for blade cooling in turbines.

8.a)	Explain a single Impulse stage and a single reaction stage of a gas turbine, with sketches.

b)	What are the factors considered for matching a compressor and turbine.  Explain.

@@@@@

Code No.312106

Set No.

3

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

AEROSPACE PROPULSION-I

(Aeronautical Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	What are differences or similarities between an automobile engine and an aircraft engine.  Discuss in detail and with examples.

b)	In how many ways and combinations, multi cylinders of an aircraft piston engine are arranged, and under what constraints.  Is it the same for road vehicles also?

2.a)	What are the factors due to which piston-propeller engines are made use of in airplanes.  Illustrate with examples.

b)	Explain the terms combustion knock and knock rating.

3.a)	Sketch a typical carburettor used in an aircraft piston engine and explain its functioning.

b)	Explain the ignition system of an aircraft piston engine.  What are the techniques used to control firing order in multi cylinder engines.

4.	Show with a neat sketch the arrangement of a typical Gas Turbine Engine. 	 Hence show and explain pressure, temperature and velocity variations from compressor outlet to turbine outlet.

5.a)	Explain the compressive action through a centrifugal compressor used for jet engines, with sketches and thermodynamic principles.

b)	Discuss how the Blade shapes, namely forward, radial and backward affect the performance of the centrifugal compressor.

6.	A centrifugal compressor under test gave the following data.

RPM 				=	11,500

Inlet total head temp.		=	21oC

Outlet total head pressure	=	4 bar

Air flow			=	131600 kg/h

Impellor dia			=	75 cm

If the slip factor is 0.92, what is the compressor efficiency?

7.a)	Sketch a typical combustion chamber (flame tube) and explain the burning of fuel for raising temperature.

b)	What are the factors to be satisfied for matching a Turbine and compressor in a Gas Turbine Engine.

8.	Explain the flow geometry at the entry and exit of a turbo machine stage.  Hence develop the useful relation in this case.

@@@@@

Code No.312106

Set No.

4

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

AEROSPACE PROPULSION-I

(Aeronautical Engineering)

Time: 3 Hours							   Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks.

---

1.	Show with neat sketches:

a) Horizontally opposed cylinder – engines

b) In-line arranged cylinder – engines

c) Radially arranged engines.

Explain the preference of one over the other, with all reasons, aerodynamic and thermodynamic.

2.a)	Explain why super charging is necessitated in airplane piston engines. Show the action with sketches.

b)	Explain the action of a propeller in producing thrust in Piston-Prop engine.

3.	Explain the terms IHP, BHP, SHP, THP, Propellor efficiency.  How are these inter-related for a Piston-Prop engine combination.

4.	Explain the principle of operation of a centrifugal compressor.  Show with sketches, different parts and their role in producing pressure rise at the outlet of the impellor.

5.a)	Outline various steps for the complete design process of an axial flow compressor.

b)	Describe losses and their cause in case of an axial flow compressor.

6.	A centrifugal compressor has to deliver 35 kg of air per second.  The impellor diameter is 76cm, and RPM of 11500, with adiabatic efficiency of 80%.  If the pressure ratio is 4.2:1, estimate the probable axial width of the impellor, at top, if the radial velocity is 120 m/sec.  Inlet conditions are 1 bar and 47oC.

7.a)	Describe the details and functioning of a can-type arrangement of combustion chambers.

b)	Describe with sketch the details of one conventional flame tube, with fuel injection system.

8.a)	Considering the out let from combustion chamber, describe the expansion process through a single stage axial flow turbine.

b)	Describe one technique of Turbine Block cooling.

@@@@@