Code No: 310303

Set No.

1

III-B.Tech. I Semester Supplementary Examinations June, 2003

THERMAL ENGINEERING-II

(Mechanical Engineering)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Explain the operation of any one type of modern high pressure boiler with neat 	sketch.

b) 	Define equivalent evaporation for a boiler.

2.a)	Explain with the help of h-s diagram the phenomenon of supersaturated flow 	through steam nozzle? What are the conditions which produce supersaturation ?

b)	Discuss the importance of divergent portion of convergent divergent nozzle ?

3.a)	With a neat diagram, explain the working principles of central flow steam 	condenser?

b)	Why the vacuum is created in the steam condenser? What is its importance	?

4.	The outlet area of a nozzle in a simple impulse turbine is 15 cm3 and the steam 	leaves the nozzles at 14 N/cm2 , 0.9 dry with a velocity 650 m/s. The nozzle is 	inclined at 200 to the  plane of the wheel, the blade speed is 250 m/sec, the blade 	exit angle is 300 and the blade velocity coefficient is 0.8. Estimate the power 	developed and specific steam consumption.

5.a)	Define and derive an expression for stage efficiency in case of a steam turbine.

b) 	In an impulse turbine the nozzles are inclined at 240 to the plane of rotation of the 	blades. Steam speed is 1000 m/sec and blade speed is 400 m/sec. Assuming 	equiangular blades, determine (i) blade angles,(ii) axial thrust,(iii) force  on the 	blades in the direction of motion, (iv) power developed for  a flow rate of         	1000 kg/hr.

6.a)	Describe with a neat sketch a typical gas turbine combustion chamber. How the 	performance of the gas turbine combustion chamber is given.

b) 	Discuss the relative merits and demerits of closed cycle and open cycle gas 	turbine plants.

Contd…..2

Code No: 310303				-2-				Set No:1

7.a)	Define the following terms:

(i) Thrust (ii) Thrust power (iii) Propulsive power (iv) Propulsive efficiency

b)	What is the basic Thermodynamic cycle used for jet propulsion and draw the T-S 	diagram for turbojet engine.

8.a)	Explain with a neat sketch the working of a screw propeller engine.

b)	Differentiate between a screw propeller and ramjet engines.

- - -

Code No: 310303

Set No.

2

III-B.Tech. I Semester Supplementary Examinations June, 2003

THERMAL ENGINEERING-II

(Mechanical Engineering)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Give the comparision between Fire tube and Water tube boilers.

b)	State the differences between the following boilers :

i) Externally fired and internally fired.

ii) Forced circulation and Natural circulation

iii) High Pressure and Low pressure

iv) Stationery and portable.

2.a)	What is meant by degree of under cooling and degree of saturation? Explain the 	physical significance of choked flow through steam nozzle?

b)	Discuss the criterion for acceleration and deceleration of flow based on cross 	sectional area, velocity and Mach number ?

3.a)	Draw the neat sketch of a parallel flow jet condenser and discuss its working 	procedure along with merits and demerits?

b)	For the flow of steam through a nozzle, explain:

(i)the effect of friction during the passage.  (ii) supersaturated flow and its effect.

4.	In a single row impulse turbine the nozzle angle is 300 and the blade speed is    	215 m/sec.  The steam speed is 550 m/sec. The blade friction coefficient is 0.85. 	assuming axial exit and  a flow rate of 700 kg/hr, determine (i) the blade angles , 	(ii) the absolute velocity of 	steam at  exit and  (iii) the power output of the 	turbine.

5.	In a steam turbine, steam expands from an inlet condition of 7 bar and 3000C with 	an isentropic efficiency of 90%. The nozzle angle is 200. The stage operates at 	optimum blade speed ratio. The blade inlet angle is equal to the outlet angle. 	Determine the blade 	angle. Determine the power output for a flow rate of    	0.472 kg/sec.

6.a)	Show that the specific work out put of a simple gas turbine plant is maximum 	when the pressure ratio is such that the compressor and turbine outlet 	temperatures are equal .

b) 	Why the performance of a real gas turbine cycle differ from that of an Ideal cycle.

c) 	Name the performance improvement methods of a gas turbine plant.

Contd…..2

Code No: 310303				-2-				Set No:2

7.a)	Define the following terms:

(i)Thermal efficiency (ii) Overall efficiency

(iii) Rocket propulsive efficiency

- Explain the significant of nozzle design criteria in jet propulsion.

8.a)	Define and give an expression for the specific thrust of turbojet engine. Explain 	its significance.

b) 	Discus the various methods of providing thrust augmentation.

- - -

Code No: 310303

Set No.

3

III-B.Tech. I Semester Supplementary Examinations June, 2003

THERMAL ENGINEERING-II

(Mechanical Engineering)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.a)	Explain the following boiler terms :

(i) Shell and setting. 	 (ii) Grate and furnace. (iii) Water space and steam space.

(iv) Mountings and accessories.

b)	Enumerate the factors which should be considered selecting a Boiler.

2.a)	Discuss the effect of pressure ratio on the operation of convergent divergent 	nozzle? Give suitable diagrams of pressure distribution, velocity distribution and 	flow rate?

b)	Define the following terms :

(i) Velocity coefficient (ii)discharge coefficient (iii) degree of undercooling and 	(iv) degree of supersaturation.

3.a)	What are different types of surface condensers ? and explain the working 	principles?

b)	Derive the equation to calculate the quantity of cooling water required in a 	condensing unit.

4.	In a single stage impulse turbine the nozzle angle is 250. The absolute velocity of 	steam at exit  is 300 m/sec  in a direction 1200 to the direction of motion of the 	blades. Assuming equiangular blades and no axial thrust, determine (i) the blade 	angles,  (ii) the power developed per kg  and  (iii) the diagram efficiency.

5.a)	Define and derive an expression for diagram efficiency incase of a steam turbine.

b) 	In a simple impulse turbine, the nozzles are inclined at 200  to the direction of motion  of moving blades. The steam leaves the nozzles at 375 m/sec.  The blade speed is 165 m/sec. Find suitable inlet and outlet angles for the blades in order that the axial thrust is zero. The relative velocity of steam as it flows over the blades is reduced by 15% by friction. Determine the power developed if the flow rate is 10 kg/sec.

6.a)	Derive the expressions for efficiency and specific work output of a simple gas 	turbine cycle in terms of pressure ratio.

b) 	What is the difference between tubular and annular type combustion chambers 	for gas turbines?

c) 	What is overall efficiency of a gas turbine plant?

Contd…..2

Code No: 310303				-2-				Set No:3

7.a)	Explain briefly with a sketch the working principle of a Rocket.

b)   	List out the applications of rockets.

8.a)	Differentiate between a jet propulsion unit and a gas turbine unit.

b)    	Define and derive an expression for the thermal efficiency of turbojet engine.

- - -

Code No: 310303

Set No.

4

III-B.Tech. I Semester Supplementary Examinations June, 2003

THERMAL ENGINEERING-II

(Mechanical Engineering)

Time: 3 hours						                  Max. Marks: 80

Answer any five questions

All questions carry equal marks

- - -

1.	With the help of a neat sketch explain a locomotive boiler 	and write down its 	merits and demerits.

2.a)	Estimate the values of critical pressure ratios and equations for the maximum 	discharge if the steam is superheated, dry saturated and wet initially?

b)	What is the condition of velocity of steam in the divergent portion of the 	convergent divergent nozzle?  Explain.

3.a)	Draw the neat diagram of evaporative condenser and discuss its working 	principles ?

b)	Define and derive the equations for the terms:

(i) vacuum efficiency and (ii) condenser efficiency.

4.a)	Define the following as related to steam turbines:

(i) Speed ratio			(ii) Blade velocity coefficient

(iii) Diagram efficiency	(iv) Stage efficiency.

- Derive   the  expression  for  maximum  blade  efficiency  in a single stage impulse

turbine.

5.	In an impulse turbine the steam issues from the nozzle with speed of 600 m/s and 	blade speed is 120 m/s. the velocity is compounded by passing the steam through 	a ring of  moving blades; through a ring of fixed blades and finally through a ring 	of moving blades. The nozzle angle is 180 and the blade exit angles and relative 	velocity coefficients 	are:

1st row moving:		 200     &    0.8

Fixed row        :                	250      &    0.85

2nd row moving:               	300     &    0.9

Find the diagram efficiency under these conditions and the power output for 	steam 	flow rate of 5 kg/sec.

6.a)	What are the different types of combustion chambers for gas turbine ? Discuss 	their relative merits and demerits.

b) 	How can you economize the fuel in a gas turbine plant?

c) 	What are the advantages and disadvantages of closed cycle gas turbines?

Contd…..2

Code No:310303				-2-				Set No:4

7.a)	Explain with a neat sketch a Turbo-jet Unit.

b)   	What are the fundamental differences between jet propulsion and rocket 	propulsion?

8.a)	Explain the significance of effective speed ratio on specific thrust and overall 	thermal efficiency in a turbojet engine.

- Briefly explain the needs and demands to be met by a modern turbo-jet engine.

- - -