Code No: 310252

OR

III-B.Tech. I Semester Supplementary Examination, June  2003

POWER SYSTEMS-I

(Electrical and Electronics Engineering)

Time : 3 hours						                 Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1.a)	Explain with sketches wherever necessary the classification of hydroelectric power plants.

b)	Calculate the available continuous power of a hydro-electric station supplied from a catchment area 4x104 hectares with an annual rain fall of 120 cm and an effective head of 300m. Assume yield factor 50% and efficiency of plant to be 80%.

2.a)	Sketch the layout of a thermal power plant and explain the function of each part there in.

b)	The overall efficiency of a thermal station is 24%. The calorific value of the coal used is  6500 K.cal/kg. Find the consumption of coal per unit..

3.a)	Draw the schematic diagram of a Pressurized Water Reactor plant (P.W.R) and explain its working principle.

b)	ON  what factors does the choice of a site depends for a nuclear power plant?

4.a)	Compare a 3-phase 4-wire system of distribution with the single phase  system.

b)	A two wire distributor 1200m long is loaded as shown in fig.1 B is the mid point.

A                B                 C

60 A	        100A, 0.8 p.f.

0.9p.f lag	         Lag

The power factors at the two load points refer to the voltage at C. The impedance of each line is (0.15 + j0.2). Calculate the sending end voltage current and power factor. The voltage at point C is 220V.

5.a)	What are the different tariff systems in use? Explain their relative merits and demerits?

b)	In a 60 MW Thermal Power Station working at 40%  load factor the energy cost is found to be 1.5 paise/unit. Calculate the cost of energy/kwh. If the power station load factor is improved to 50% . Due to increase in energy generation the fuel cost increases the annual generation cost by 6%.

Contd…2.

Code No. 310252					-2-				OR

6.a)	Define the following

(i)  Load factor  (ii)  Demand factor   (iii)  Diversity factor (iv) Maximum demand (v) connected load.

b)	Explain about the pumped storage plant.

7.a)	Describe the ash handling and fuel handling methods used in modern thermal power plant.

b)	State and prove Kelvin’s Law.

8.	Write short notes on the following

a)	`Mass curve     b)      Breeder reactor   (c) Sub station Equipment .

-x-