Code No: 310255

OR

III-B.Tech. I-Semester Supplementary Examination, June 2003

ELECTRICAL MEASUREMENTS

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1.a) 	Explain different types of deflecting systems which are operating in an indicating

instrument.

b)	Discuss the role of controlling system in an indicating instrument.

c)	How are controlling torques produced. Explain with neat sketches the different methods of producing controlling torque.

2.a) 	Compare between attraction type and repulsion type of moving iron instruments.

b) 	What are the errors found in moving iron instruments? Explain also the method of

compensation.

3.a)	With a neat sketch, explain the working of Single Phase Energy motor.

b)	What are the various adjustments that are required to be made in a single phase energy meter, so that it reads correctly? Explain each one of them.

4.a)	With a neat sketch explain the measurement of resistance using a potentiometer.

b) 	Draw circuit diagram and explain the measurement of power using potentiometer.

5.a)	What are the various limitations of wheatstone bridge for measurement of high and low resistances.

b) 	The four arms of a whetstone bridge are as follows. AB = 100, BC=1000,             CD = 4000 and DA = 400. The galvanometer has a resistance of 100, a sensitivity of 100 mm / Micro Amp. and is connected across AC. A source of 4V dc is connected across BD. Calculate the current through the galvanometer.

6. 	Describe the ‘Variable Mutual Inductance method for testing of a  current transformer.

7. 	Describe the method of measurement of  reactive power in three phase circuits using single dynamometer type wattmeter.

8.	Write short notes on the following:

a)	Loss of energy due to hysteresis.

b)	Synchroscope.

***     ***     ***