Code No.: 310256

OR

III B.Tech I Semester (Supplementary) Examinations June 2003

ELECTROMECHANICS-III

(Electrical and Electronics Engineering)

Time: 3 hours							     Max. Marks: 70

Answer any Five questions

All questions carry equal marks

- - -

1.	Distinguish between integral slot and fractional slot winding and their merits and demerits.

2.	The field form of a star connected 3-phase machine is given by 100 sinq + 30sin 3q + 10 sin5q, where q is measured from the interpolar axis. If the stator has 9 slots per pole and coils are full pitched, determine the wave form of phase voltage in terms of Vmax, the amplitude of the fundamental phase voltage.  Determine also the r.m.s values of phase and line voltages. What are the phase and line voltages if the flux is sine distributed with the same total flux per pole?

3.a)	What happens to the value of synchronous reactance if air gap is increased

b)	What is the excitation voltage in case of synchronous machines?

c)	A 30KVA, 440V, 50Hz, 3-phase, Star-connected alternator gave the following 	test data:

4.	Explain the Phasor diagram of salient-pole synchronous machine

- At Lagging P.f

- At Leading P.f

5.a)	How do you calculate the time constant in case of an alternator

b)	Show that an alternator running in parallel a constant voltage and frequency bus

bars has a natural time period of oscillation.Deduce a formulae for the time of one

complete oscillation and calculate the value for a 500KVA, 3-phase, 10000V

machine running at 1500 rpm and constant 50Hz bus bars.

The moment of Inertia of the whole moving system is 14112 Kg-m2 and the

steady short-circuit current is five times the normal full load value

6.a)	Define and explain different torques of a synchronous motor?

b)	A 400V, 50Hz, 6-pole, 3-f, Y connected synchronous motor has a synchronous reactance of 4W / phase and a resistance of 0.5W / phase. On full-load, the excitation is adjusted so that machine takes an armature current of 60 Amperes at 0.866 p.f. leading. Keeping the excitation unchanged, find the maximum power output. Excitation, friction, windage and iron losses total 2KW.

Contd….2

Code. No.: 310205				-2				Set.No:4

7.a)	Why are synchronous condensers also called as synchronous phase modifiers? Where are these used?

b)	A substation operating at its full load of 1,000KVA supplies a load of power factor 0.71 lagging. Calculate the permissible additional load at this power factor and the rating of synchronous condenser to raise the substation power factor to 0.87 lagging.

8.a)	Show that in a shaded pole motor, the flux in the shaded part of pole segment 	lags behind the flux of the unshaded pole segment both in space and time, 	resulting a rotational torque in the motor.

b)	A capacitor connectected in series with the starting winding of a resistance start split phase induction motor. Explain the changes in the performance characteristics

$$$