Code No: 310351

OR

III - B.Tech. I-Semester Supplementary Examination, June 2003

THERMAL ENGINEERING – II

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering)

Time: 3 hours.							Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1.a)	Explain the actual working of Diesel engine using Diesel cycle.  Explain the reasons for difference between actual and ideal cycle.

b)	The pressure, temperature, and volume of the air at the beginning of diesel cycle are 0.98bar, 300C and 0.1m3.  The volume of air at the end of heat supply is 0.0125 m3.  The compression ration of the cycle is 15.  Find the following:

i)   Heat supplied and rejected per kg of air.

ii)  Thermal efficiency of the cycle.

iii)  Mean effective pressure of the cycle and

iv)  The clearance and cut-off as the percentage of stroke.

2.a)	What are the A:F ration requirements of a spark ignition engine under varying operating conditions?

b)	Explain why choke is used in carburetor and write a short note on automatic choking?

3.a)	What are the functions fuel injector has to perform?  Explain.

b)	Distinguish clearly between common rail fuel injection system, Distributor injection system and individual pump and injection nozzle system for C.I engines.

4.a)	Draw the circuit diagram and describe the working of battery ignition system used for 4 cylinder Petrol engine.

b)	Explain cleanly the function of condenser in the ignition system.

5.a)	What is the purpose of cooking of I.C., engine.  Explain in detail.

b)	Explain the following

i) Radiator	ii) Thermo stat control and 	iii) Antifreeze solutions.

c)	Why scavenging is very important in two-stroke engine?  Explain how it is being done two-stroke S.I engines.

6.a)	Explain combustion stages in C.I engines.

b)	What are the requirements of C.I.  combustion chamber for Diesel engines?  And Explain, in detail MAN combustion chamber for C.I engines.

Cont..2

Code No.310351				-2-					OR

7.a)	Define volumetric efficiency as applied to petrol engine.  How does volumetric efficiency of engine limits the power out put.

b)	The following data were obtained from a test on a  single-cylinder, 4-stroke,oil engine:

Cylinder bore=15cm;  stroke = 25cm;

Area of indicator diagram     = 450 sq.mm;

Length of indicator diagram  =  50mm;

Indicator spring rating	  =  1.2mm for a pressure of 9.80 N/cm2

N = 400 rpm ;		Brake Torque = 225 N-m;

F.C = 3kg /hr.		CV = 42,000 KJ/kg

Cooling water flow rate = 4 kg /minute;

Cooling water temperature  = 420 C.

Specific heat of cooling water = 4.168 kJ/kg 0C

Determine (i) Mechanical efficiency (ii) Break power efficiency

(iii)Specific Fuel Consumption and (iv) Draw heat balance in kW.

8.a)	Describe with neat sketches the working of a simple constant pressure open cycle gas turbine  how does actual cycle differ from theoretical cycle.

b)	Explain working difference between propeller-jet turbojet, turbo prop engines

&&&&&