Code No.310352

OR

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

METALLURGY

(Common to Mechanical Engineering, Production Engineering and Mechanical Manufacturing)

Time: 3 Hours							   Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks.

---

1.	What is solid solution?  Distinguish inter metallic, interstitial and electron compounds highlighting their specific properties and applications with appropriate examples?

2.	Distinguish eutectic, peritectic, eutectoid and peritectiod reactions with appropriate examples?

3.	Explain the necessity of alloying discuss the effect of alloying of various elements in steels with appropriate examples?

4.	What are the advantages of specifying steels based on hardenability.  Sketch the diagram of 1080 steel and a) show cooling curve that will result in a structure of 50% martensite and 50% pearlite b) that will result in a uniform pearlitic structure of rockwell C40.

5.	Distinguish malleable, wrought grey, white and nodular type of cast irons mentioning their specific properties and applications?

6.	Compare and contrast electric arc furnace and induction furnace methods of making steels.  What are the specific advantages of steels produced by electric melting?

7.	What is the specific necessity of case hardening?  Distinguish cyaniding, carburizing and flame hardening.  Mention the method of case hardening crankshafts.

8.	Write short notes on any three:

a) Austempering

b) Low alloy steels

c) Isothermal transformation

d) Tool and die steels.

@@@@@