Code No: 310354

OR

III-B.Tech. I-Semester Supplementary Examinations June, 2003

DYNAMICS OF MACHINERY

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering and Mechatronics.)

Time: 3 hours						                  Max. Marks: 70

Answer any five questions

All questions carry equal marks

- - -

1.a)	What is a friction circle?  Show how many friction axes are there for a link having 	pin joint at its end.

b)	A force of 4,000 N is applied on a piston along the line of stroke of a horizontal 	steam engine.  When the crank is at 60o to I.D.C. calculate the torque on the crank 	shaft; when the length of connecting rod is 0.8 m and length of stroke is 0.4m.

2.	A vertical petrol engine 150mm diameter and 200mm stroke has a connecting rod 	350mm long.  The mass of the piston is 1.6 kg and the engine speed is 1800 rpm.  	On the expansion stroke with crank angle 30o from top dead centre, the gas 	pressure is 750 kN/m2.  Determine the net thrust on the piston.

3.a)	State and explain the following terms:

(i)   Coefficient of friction  (ii)  Angle of repose (iii) Angle of friction.

b)	A lorry is moving on a level road at a sped of 36 kmph. Its centre of gravity lies at 	a distance of 0.6m from ground level.  The wheel base is 2.4m and the distance of 	centre of gravity from the rear wheels is 0.9m.  Find the distance travelled by the	car before coming to rest when brakes are applied, (i) to the rear wheels (ii) to the	front wheels & (iii) to all the four wheels.  The coefficient of friction between the 	tyres and the road surface is 0.45.

4.a)	Explain the principle of working of a screw jack and derive an expression for its mechanical advantage.

b)	In a band and block  Brake, the band is lined with 14 blocks, each of which subtends an angle of 200 at the drums  centre.  One end of the band is attached to the fulcrum of the brake lever and the other to a pin 150 mm from the fulcrum. Find the force required at the end of the lever 1m long from the fulcrum to give a torque of 4k N-m.  The diameter of the brake drum is 1m and the coefficient of friction between the blocks and the drum is 0.25.

5.	A proell governor has arms of 300 mm length.  The upper arms are hinged on the axis of rotation, whereas the lower arms are pivoted at a distance  of 35 mm from the axis of rotation.  The extension of lower arms to which the balls are attached are 100 mm long.  The  mass of each ball is 8 kg and the mass on the sleeve is 60 kg.  At the minimum radius of rotation of 200 mm, the extensions are parallel to the governor axis.  Determine the equilibrium speed of the governor for the given configuration.  What will be the equilibrium speed for the maximum radius of  250 mm.

Contd…..2

Code No:310354					-2-				OR

6.	Explain terms:

(a) Variations in tractive effort  (b) Swaying couple

(c) Hammer blow as applied to locomotive balancing. Derive expressions for 	             these for two 	cylinder uncoupled locomotive balancing.

7.a)	Define free vibrations with neat sketch.

- A Shaft 1.5m  long, supported in flexible bearings at the ends, carries two wheels each of 60 kg mass.  One wheel is situated at the centre of the shaft and the other at a distance 375mm from the centre. The shaft is hollow, of external diameter 75mm, internal diameter 40mm. The density of the shaft material is 7700 kg/m3.

8.	A marine engine, shaft and propeller are approximately equivalent to the 	following three rotor system. The combined moment of inertia of the engine 	masses is 800 kg.m2, that of 	the flywheel is 320 kg.m2  and that of the propeller 	is 20 kg.m2. The equivalent shaft between the engine masses and the wheel is 	5 cm diameter and 200 cm long and that between flywheel and the propeller 	is 2.5 cm diameter and 200 cm long. Find the frequencies of torsional 	vibration of the system and the positions of the nodes.

- - -