Code No.:310355

OR

III-B.Tech. I Semester Supplementary Examinations June, 2003

DESIGN OF MACHINE MEMBERS - I

(Common to Mechanical Engineering, Production Engineering, Mechanical Manufacturing Engineering.)

Time: 3 hours						                  Max. Marks: 70

Answer any five questions

All questions carry equal marks

- - -

1.a)	How will you designate the following varieties of materials according BIS 	system: Grey cast iron with an ultimate tensile strength of 300 N/mm2. Plain 	carbon steel with a minimum tensile strength of 320 N/mm2

b)	Briefly explain different ferrous metals and their properties.

2.a)	State the mechanical properties of metals and explain briefly.

b)	Design a hollow shaft required to transmit 12 MW at a speed of 300 rpm. The 	maximum shear stress allowed in the shaft is 80 MPa and the ratio of inner 	diameter to outer diameter 0.75.

3.a)	Explain the following methods of reducing stress concentration 	 	           	(i) Drilled holes    	(ii)  Using large fillet radius 	(iii)  Added grooves.

b) 	A shaft is made of steel [ultimate tensile strength 700 MPa and yield point 420 	MPa] is subjected to a torque varying from 200N m anti-clockwise to 600 N m 	clockwise. Calculate the diameter of the shaft if the factor of safety is 2 and it is 	based on the yield point and the endurance strength in shear.

4.a)	Explain the following terms in connection with riveted joints

(i)  Pitch   (ii) Back pitch  (iii) Diagonal pitch   (iv) Margin

b) 	A double riveted butt joint, in which the pitch of the rivets in the outer rows is 	twice that in the inner rows, connects two 16 mm thick plates with two cover 	plates each 12 mm thick. The diameter of the rivets is 22 mm. Determine the 	pitches of the rivets in the two rows if the working stresses are not to exceed the 	following limits:

Tensile stress in plates = 100 MPa, Shear stress in rivets = 75 MPa and bearing 	stresses in rivets and plates = 150 MPa.

Make a fully dimensioned sketch of the joint showing atleast two views.

5.a)	Prove that a square key is equally strong in shear and compression.

b)	Design a cotter joint to withstand an axial load varying from 60 kN in tension to 	60kN in compression.  The allowable for the steel used in the joint are 60 Mpa in 	tension; 75 Mpa in crushing; 48 Mpa in shear.

6.a)	Describe the design procedure of Gib and cotter joint with one gib.

b)	Design a knuckle joint to transmit 150 kN, with permissible stresses in tension; 	shear and compression are 75 Mpa; 60 Mpa and 150 Mpa respectively.

Contd…..2

Code No.:310355				-2-				OR

7.a)	How is the shaft  designed when it is subjected to twisting moment only?

b)	Design a shaft to transmit power from an electric motor to a lathe headstock 	through a pulley by means of a belt drive.  The pulley weighs 200N and is located 	at 100mm from the centre of the bearing.  Diameter of the pulley 200mm.  	Maximum power transmitted is 1.5HP at 120 rpm.  Angle of lap of belt 1800.  	Coefficient of friction between belt and pulley 0.3.  Shock factor in bending 1.5 	shock factor in twisting 2.0.  Allowable shear stress in the shaft 35N/mm2.

8.	Design a marine type flange coupling to transmit 500HP at 150 rpm.  Allowable 	stress for the steel used are 70N/mm2 in tension, 140N/mm2 in compression and 	50N/mm2 in shear.

- - -