Code No. 310451

OR

III-B.Tech. I-Semester Supplementary Examinations, June 2003

PULSE AND DIGITAL CIRCUITS

(Common to Electronics and Instrumentation Engineering, Electronics and Communication Engineering, and Electronics and Control Engineering)

Time: 3 hours							Max. Marks: 70

Note:  1) Answer any five questions

2) All questions carry equal marks

3) Answers to different parts of a Question must   be written in one place.

4) Assume reasonable values of data wherever necessary.

---

1. a)	Obtain the expression for transmission error voltage et in the case of High pass R.C. circuit with Ramp input.

b)	A 10Hz symmetrical square wave whose peak-to-peak amplitude is 2v, is impressed upon a High Pass RC circuit whose lower 3db frequency is 5Hz.  Calculate the voltage values and sketch the waveform.  What is the peak-to-peak amplitude?

2. a)	Draw the shunt the clipper and explain its response for a symmetrical square wave input.

b)	Determine the specifications of the diode to be used, and the value of resistance in the circuit, for a negative serves unbiased clipping circuit, when an input square wave of  10V is applied.  The output current from the circuit must be 1mA. Negative output voltage Vo should not exceed 0.5V.  The reverse saturation current of the diode Io = 5µA.

3. a)	Draw the circuit of a biased +ve clamping type and explain its output response for a symmetrical square wave input.

b)	Design a -ve voltage clamping circuit with an input square wave of 1KHz frequency and an amplitude of 10V.  Signal source resistance is 600.  Tilt in the output wave form should not exceed 1%.

4. a)	Explain how a junction diode can be used as a switch.  What are its advantages and disadvantages?  Compare its performance as a switch with that of BJT.

b)	Explain the terms (i) Storage Time (ii) Reverse recovery Time (ii) fall Time and    (iv) Rise time pertaining to Diode switching characteristics.

Contd…2

Code No. 310451				.2.					OR

5. a)	Draw the circuit and with the help of neat waveforms, explain the working of transistor and Mono stable Mulluvibrator circuit using npn transistors.  What are its applications?

b)	Design a transistor Astable Multi Vibrator circuit, to generate a 2KHzs square wave output.  Vcc is +6V.  IL = 20 µA.  hFE(Mini) = 70.  Employ silicon transistors.

6. a)	Explain the terms (i) Sweep speed Error  (ii) Displacement Error (iii) Differential Linearity (iv) Transmission Error pertaining to Time Base waveforms.

b)	Draw the BJT Miller Integrator circuit and explain its working with the help of waveforms.

7.	Draw the circuit for C.E.  Triggered Blocking Oscillator and explain its operation with the help of waveforms.

8.	Write Notes on any TWO.

a) 	Unidirectional sampling gates.

b) 	Synchronization of sweep circuits.

c)	Schmitt Trigger circuits.

###