Code No. 310452

OR

III-B.Tech. I-Semester Supplementary Examinations, June 2003

CONTROL SYSTEMS

(Common to Electronics and Communication Engineering, Electronics and Instrumentation Engineering and Electronics and Control Engineering)

Time: 3 hours							Max. Marks: 70

Answer any five questions

All questions carry equal marks

---

1. a)	For the Mechanical system shown in Fig.1 (i) Write the differential equations governing its dynamics behaviour (ii) Write an analogous electric circuit based on Force-voltage analogy.

Fig.1

b)	Write a block diagram representation of an armature controlled D-C servomotor using appropriate cause-effect equations and hence evaluate its transfer function.

2. a)	Find the overall transfer function C(s)/R(s) for the system shown in Fig.2 using block reduction technique.

Fig.2

Contd…2

Code No. 310452				.2.					 OR

b)	Write clearly all the steps needed and evaluate the transfer function C(s)R(s) for the signal flow graph shown in Fig.3.

3. a)	The open loop transfer function of a unity feed back control system is KG(s).  When k=1 the corresponding unit step response is c(t) = (1-1.25e-2t + 0.25e-10t) u(t). (i) Evaluate G(s)	  (ii) Evaluate the value of k so that the system now has a damping ratio= 0.6.  What is the corresponding undamped natural frequency?

b)

For the system shown in Fig.4, find the values of k1 and k2 so that the step response has a peak-overshoot of 25% and first undershoot occurs at t=2sec.  Find the corresponding rise time and settling time.

4. a)	The forward path transfer function of a unity-feed back control system is

G(s) = k(s+2)s2 (s+4).

(i) Determine the static error constants and hence evaluate the steady state error    when r(t)= (4+t+2t2) u(t)

(ii) With k=1 and r(t) = (t22 + t36) u(t), determine the error as a function of time

b)	Apply R-H criterion and determine the number of roots on the Right side of        s-plane for the equation F(s) = s8 + 2s7 + 6s6 + 10s5 + 11s4 + 16s3 + 9s2 + 10s +1 =0.

Contd…3

Code No. 310452				.3.					OR

5. 	For the system shown in Fig.5, draw the root locus diagram for 0 < <  clearly writing all the Ten steps.  Mark all salient points on the graph.  Determine from your graph, the value of   for which the damping ratio of the dominant closed loop poles is 0.866. For this value of  find all the closed loop poles.

Fig. 5

6. a)	Explain the terms “Gain Margin” and “Phase Margin” of a control system.  A unity feedback system has the forward path transfer function G(s)=10/(s2 +2s +2) (s+2).  Draw the polar plot and hence determine its Gain and phase margins.

b)	Write explanatory notes on the following:

(i) Constant M-circles.	(ii) Constant N-circles		(iii) Nichol’s chart.

7.	The open loop transfer function of a unity feedback system is

G(s) = ks(1+0.1s) (1+0.001s).

Find the value of k so that for a unit velocity input the steady state error in the output position must be less than or equal to 0.1% of the final output velocity.  Draw the Bode plots for this value of k and determine the Gain margin and Phase margin.  Is the system stable?  An amplifier-Network combination having a transfer function.

Gc(s) = 1+0.0167s1+0.00222s is connected in cascade with the plant to make the overall forward path transfer function Go(s) = Gc(s). G(s).  Draw the Bode plots for the modified system and re-compute the Gain and phase margins.  Comment on the results obtained.

8. a)	List the properties of state transition matrix.  Evaluate the STM given                         A= −2 1.

b)	Solve the state equation x = Ax + Bu give A=0 1  B = 0

x(0) = 1 and u = unit step input.

###