Code No.310454

OR

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

POWER AND INDUSTRIAL ELECTRONICS

(Common to Electronics and Communication Engineering and Electronics and Instrumentation Engineering)

Time: 3 Hours							   Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Define conversion efficiency of a power stage.  Draw the diagram of a transformer coupled single transistor output stage and explain the need for impedance matching.

b)	Draw the circuit of a class AB push-pull amplifies using transistors having complementary symmetry and explain how push-pull operation is obtained.

2.a)	What is a cross over distortion?  How is it minimized? What will be the resulting operation of the device in class B push-pull power amplifier?

b)	Explain the reason for potential instability in tuned amplifiers.  Suggest a method for its stabilization.

c)	What are the different types of tuned amplifiers?  How are they classified?

3.a)	SCR can be used as a switch in a power circuit.  Justify the statement using a transistor model of SCR.

b)	What is a snubber circuit required for an SCR?  Explain how snubber circuit parameters can be chosen for a given application?

4.a)	What are the advantages of a free wheeling diode in a rectifier circuit feeding an RL load?

b)	A 1 supply of 230V, 50Hz is supplying power to a 1, fully controlled converter.  Find the average output if the firing angle is 50oC.  What will be the new output voltage if one of the SC RS is open?

5.a)	Draw the circuit diagram of a 1 parallel inverter with reactive load.  Explain the operation of the above circuit with the help of necessary wave forms.

b)	What is dc chopper?  Describe the various types of chopper configurations with appropriate diagrams, wherever necessary.

6.	Explain with a neat sketch how the speed of a dc motor can be regulated automatically by (a) fully controlled single-phase bridge (b) choppers.

7.a)	Explain the principle of resistance welding and discuss the types of resistance welding.

b)	With a circuit schematic explain any one electronic welding control system.

8.	Write short notes on the following:

a) Generation of ultrasonic waves b) Dielectric heating  c) Triac and Diac.

@@@@@