Code No. 310455

OR

III-B.Tech. I-Semester Supplementary Examinations, June 2003

COMPUTER ORGANISATION

(Common to Electronics and Communication Engineering and Electronics and Control Engineering)

Time: 3 hours							Max. Marks:70

Answer any five questions

All questions carry equal marks

---

1.	Describe in detail the functional design of an elementary computer using an appropriate Register Transfer Language.

2.	Trace the various developments in the area of systems software that improve the productivity of the machine as well the programmer.  Highlight the utility of each of the software developed.

3. a)	What are the common addressing modes in processing units?  It has been proposed that the PC relative addressing modes be eliminated in favour of other modes such as the use of stack.  What is the disadvantage of such a proposal?

b)	What are the various instruction formats that are commonly available in processors?  Mention the advantages and disadvantages in having variable length instructions.

4. a)	Explain the concept of microinstructions.  Distinguish between the horizontal and vertical microinstructions formats.  How does the microprogramming based design simplify the design of the control unit?

b)	Explain in detail at least two applications of microprogramming.

5. a)	Explain the various steps in DMA.  Why is DMA given higher priority than the CPU for bus access?

b)	Assume that the number of clock cycles for a polling operation-including transferring to the polling routine, accessing the device, and restarting the user program is 400 and that the processor executes with a 500MHz clock.  Determine the fraction of CPU time consumed for the following three cases, assuming that you poll often enough so that no data is ever lost and assuming that the devices and potentially always busy:

(i) The mouse must be polled 30 times per second to ensure that we do not miss any movement made by user.

(ii) The floppy disk transfer data to the processor in 16 bit-units and has a data rate of 50KB/sec.  No data transfer can be missed.

(iii) The hard disk transfers data in four-work chunks and can transfer at 4 MB/sec.  Again no transfer can be missed.

Contd…2

Code No. 310455				.2.					 OR

6. a)	What is meant by hierarchy of memory?  Why is it necessary?

b)	Explain the working of a disk with the help of a neat diagram.  Identify the important parameters in its specification.

c)	What are the different types of cache memories?  Explain the working of an associative cache.

7. a)	Give Flynn’s classification of the various computing machines.

b)	Describe briefly the various types of multiprocessor architectures that are popular.  How are the issues in multiprocessor architecture design different from the conventional single processor computers?

8. 	Write short notes on any four of the following:

a) Representation of real numbers

b) Instruction formats

c) Memory devices

d) Pipe line processors

e) Multiprocessor architecture.

###