Code No: NR-310456

OR

III-B.Tech. I-Semester Supplementary Examination, June 2003

ANTENNA AND WAVE PROPAGATION

(Electronics and Communication Engineering)

Time : 3 hours							        Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1.a)	What is retarded potential? Give the expressions for electric and magnetic potentials and explain their significance.

b)	Obtain the relation for magnetic and electric fields due to a small, alternating current element and hence define near field.

2.a)	Distinguish clearly between Near field and Far field of an antenna and derive the expressions necessary from the fundamentals.

b)	An antenna has effective height of 100 m, with a current at base being 400 amps(rms) at 40Hz. Find the antenna efficiency.

3.a)	An array consists of three collinear /4 dipoles with  spacing. Determine gain and beam width of the major lobe between 3dB points in the plane containing the array.

b)	What is a parasitic element? Where are they used?

4.a)	Explain how a rhombic antenna is formed by a V antenna and inverted V antenna.

b)	An endfire array composed of /2 radiatiors with axes at right angle to the line of the array required to have a power gain of 20. Determine the array length and width of major lobe between the nulls. Derive the formula used.

5.a)	Derive an expression for the radiation resistance of a square loop antenna .

b)	Find the diameter of a dish antenna that will form a beam having   0.50 half power beam width at 8.2 GHz. Assuming 60% efficiency, calculate antenna gain and effective aperture.

6.a)	Explain the gain measurement of antenna by direct comparison method.

b)	Explain the absolute method for measuring the gain of an antenna.

7.	Discuss the advantages and disadvantages of communication at ultra high frequencies. How does the field strength of UHF signals depend on the heights of the transmitting and receiving antennas.

8.	Write short notes on:

a)	Whistlers		b)	Magneto ionic equation

c)	Fading of radio wave	d)	Super refraction

e)	Line of sight propagation.

$$$