Code No: 310551

OR

III-B.Tech. I Semester Supplementary Examination, May 2003

SIGNALS AND MODULATION THEORY

(Computer Science and Engineering)

Time : 3 hours						                 Max. Marks: 70

Answer any five questions

All questions carry equal marks

---

1.a)	If w(t) has the Fourier transform

w(t)  = [ j 2πt(1+j2πt)]

find x(t) for x(t) = e- jt w(t-1)

b)	Determine the energy spectral density of a square pulse g(t) defined as

g(t) = A      for |t| < /2

= O           |t| > /2

Hence calculate the total energy.

2.a)	Define (i)  Cumulative density function

(ii) sample space

(iii) random variable

b)	A random process is given by v(t) = cos (wot + ) where  is a random variable with probability density as

p() = 1/2        - <  < 

=  0               else where

(i)   Show whether v(t) is stationary or otherwise

(ii)  If the random variable is replaced by a fixed angle o, will v(t) be time   independent?

3.a)	An SSB transmission contains 800 watts.  This transmission is to be replaced by a standard AM signal with the same power content. Determine the power content of the carrier and each of the side bands when the percentage modulation is 85%.

b)	Compare NBFM and WBFM.

c)	Explain generation of phase modulation from frequency modulation and vice-versa.

Contd….2.

Code No: 310551					-2-				OR

4.a)	A TDM system is to be designed to multiplex four signals

em1(t) = cos wot ;  em2 (t) = 0.5 cos wot ; em3 (t) = 2 cos (2 wot);

em4(t) = 2cos 4 wot.

(i)  If each signal is sampled at the same rate, calculate the minimum sampling rate.

(ii) What is the commulator speed in revolution per second?

(iii) Design a commulator  which will allow each of the four signals to be sampled at a rate faster than is required to satisfy the Nyquist Criterion for the individual signal.

b)	What is pulse time modulation ? Explain in detail the two types of PTMs.

5.a)	For a signal, the bandwidth is 3kHz and S/N ratio is 15. (i) Calculate the channel capacity (ii) If the bandwidth is increased to 4kHz and signal is transmitted over the same channel, calculate the required S/N rated and percentage change in the signal power.

b)	Draw a schematic diagram of an equivalent noiseless discrete channel.

6.a)	State and prove convolution theorem.

b)	The probability density function can be written as

P (X < x1) =    k=−∞jP(xk)

What is the interpretation of this expression?

7.a)	A PDM signal has  < Ts and fixed leading edges at t=mTs. By sketching typical waveforms show that, the circuit below will reconstruct x(t)  if RC >> Ts and switch is closed momentarily every t= mTs

b)	Discuss (i)  Algebric codes   (ii) convolution codes

8.	Write notes on

(a)	Synchronous and asynchronous TDM.

(b)	Cross-talk

(c)	Central limit theorem

(d)	Parity check

-x-