Code. No.: 310851

OR

III-B.Tech I-Semester Supplementary Examinations June 2003

MECHANICAL UNIT OPERATIONS

(Chemical Engineering)

Time: 3 hours							     Max. Marks:70

Answer any Five questions

All questions carry equal marks

- - -

| 1. | Write the distinguishing features of the following: a) angle of internal friction and angle of repose. - differential and cumulative analysis of particle sizes. - mixers for dry powders and mixers for pastes in general. |
| --- | --- |
| 2.a) | How does DN differ qualitatively from the volume mean diameter Dv? |
| b) | The screen analysis shown below applies to a sample of crushed quartz. The density of particles is 2650 kg/m3 and the shape factors are: a = 2 and Φs=0.571. Calculate the arithmetic mean diameter DN for the –4 mesh to +200 mesh fraction of the material analyzed in the table given below. |
|  | Mesh no.; Dpi, mm; Mass fraction, Xi; Mesh no.; Dpi, mm; Mass fraction, Xi. 4; 4.699; 0.0000; 35; 0.417; 0.0210. 6; 3.327; 0.0251; 48; 0.295; 0.0102. 8; 2.362; 0.1250; 65; 0.208; 0.0077. 10; 1.651; 0.3207; 100; 0.147; 0.0058. 14; 1.168; 0.2570; 150; 0.104; 0.0041. 20; 0.833; 0.1590; 200; 0.074; 0.0031. 28; 0.589; 0.0538; Pan; ---; 0.0075. |

3.a)	What are various objectives of size reduction in chemical industry?

b)	Name four ways, which are commonly used, for communication of solids.   Discuss each one of them with reference to product size and equipment?

c)	What do you understand by stages of reduction? Explain them with example.

4.a)	Discuss in detail about the principles of cake filtration.

b)	What are different types of filtration and discuss?

5.a)	Give a neat sketch of a kneader mixer and explain its working and uses.

b)	Discuss the importance of Froude number in determining the power consumption of an agitator

c)	Describe a suitable equipment for mixing miscible liquids.

6.a)	Explain in detail the different mechanisms involved in blending of solids.

b)	Draw the Rushton plot for mixing and describe its application in the selection of mixing devices.

Contd….2

Code. No.: 310851				        -2-				OR

7.a)	What is secondary nucleation? Explain fluid shear nucleation.

b)	Explain the phenomenon of contact nucleation.

8.a)	Discuss the principle of centrifugal filtration.

b)	A rotary drum filter with 25% submergence is to be used to filter a concentrate aqueous slurry of CaCO3 containing 250 kg of solids per cubic metre of water.  The pressure drop is 500 mm Hg. If the filter cake contains 50% moisture (wet basis), calculate the filter area required to filter 10 kg / min of slurry when the filter cycle time is 5 min. Assume that filter medium resistance is negligible.  The temperature during test cycle is 20oC.

Given		:	0	 =	1.2 x 1011 m/kg ; s = 0.25

 	 = 	0 (p)s

For water		:	 = 1 CP ;  = 980 kg/m3

Density of calcium carbonate = 250 kg/m3

$$$