Code No: 310852

OR

III-B.Tech. I-Semester Supplementary Examinations June, 2003

CHEMICAL ENGINEERING THERMODYNAMICS-II

(Chemical Engineering)

Time: 3 hours							  Max. Marks: 70

Answer any five questions

All questions carry equal marks

- - -

1.	In a vapor compression refrigeration cycle employing Freon-12 as the refrigerant, the evaporator and the condenser are maintained at 0.1237 MPa and 0.9 MPa, respectively. Freon-12 leaves the evaporator as saturated vapor and enters the compressor. The Freon leaving the condenser is in the saturated liquid state. Calculate the COP of the refrigerator.

2.a)	Define approach temperature. Show its effect on COP.

b)	Enlist characteristics of good refrigerant.

3.a)	Define the terms : An ideal Liquid solution, Raoult’s Law, Fugacity, Chemical Potential.

b)	Derive Duhem-Margules equation for a system consisting of liquid solution of two components in equilibrium.

4.a)	Show that for the non volatile solute, boiling point of the solution is greater than that of the solvent.

b)	Define molal boiling point elevation constant and discuss its utility.

5.a)	Explain Lewis-Randall rule and Henry’s law. For a binary system, show that if        	Henry’s law is applicable for one component, Lewis-Randall rule is applicable for        	the other component.

b)	Explain Henry’s law as a model for ideal behavior of a solute.

6.	Vapor-liquid equilibrium data for the system 1,2 dichloro methane (1)/methanol (2) at 50 oC are as follows :

P/kPa		  x1		   y1

55.55		0.000		0.000

61.76		0.097		0.174

65.66		0.292		0.324

65.76		0.349		0.349

65.15		0.493		0.386

62.36		0.720		0.438

54.92		0.893		0.537

31.10		1.000		1.000

Determine the values of ln1 and ln2 using Van Laar equations. Also plot ln1 and ln 2  vs x1.

Contd…2

Code No: 310852					-2-				OR

7.	Using condition U=U (T) and stability condition can an equation of state for a gas can be obtained? Is it unique?

8.	Ethyl acetate can be prepared by vapour phase catalytic dehydrogenation as shown in the following equation:

2C2H5OH(g)  						CH3COOC2 H5 (g) + 2H2 (g)

the equilibrium constant is given by Log 10 Kp = -2100/T + 4.66 where T is in K. Calculate the composition of the equilibrium mixture at 1970C and 1 atm pressure, if the feed consists of pure ethanol vapor.

^^^