Code No: 310853.

OR

III-B.Tech  I Semester Supplementary Examination - June 2003

MASS TRANSFER OPERATIONS - I

(Chemical Engineering)

Time: 3 hours							Max.Marks:70

Answer any five questions

All questions carry equal marks

---

1.a)	For a binary mixture of A and B show that DAB = DBA for equimolar counter 	diffusion.

b)	A mixture of He and N2 gas is contained in a pipe at 298K and

1.01325 x 105 N/m2 total pressure, which is constant throughout. At one end of 	the pipe at point 1 the partial pressure pA1 of He is 0.60795 x 105 N/m2  and at the 	other end 0.2m, pA2 = 0.20265 x 105 N/m2.  Calculate the flux of He at steady 	state if DAB of He-N2 mixture is 0.687 x 10-4 m2/s.

2.a)	What is the effect of temperature and pressure on the diffusivity of liquids?

b)	Estimate the liquid diffusion coefficient of ethanol, C2H5OH, in a dilute solution 	of water at 100C. The atomic volume of C, H and O are 14.8 x 10-3, 3.7 x 10-3 m3 	and 7.4 x 10-3 m3/ kg mole respectively. At 100C, the viscosity of a solution 	containing 0.05 mole of alcohol per 10-3 m3 of water is 1.45 x 10-3 kg/m.s, FB = 	2.6 for water.

3.	Discuss the following Mass Transfer Theories:

i)	Film Theory.

ii)	Boundary Layer Theory.

iii)	Penetration Theory.

iv)	Two Film theory.

4.	Explain in detail:

i)	Liquid Hold up in Packed Towers.

ii)	Mass Transfer in Packed Towers.

iii)	End effects and Axial mixing.

( Contd…2 )

Code No: 310853.			.. 2..				OR

5.	Ammonia is to be recovered from its mixture with air by scrubbing with pure 	water in a multistage continuous counter current apparatus. 99% of ammonia 	(mole basis) present in the inlet mixture, of initial composition 10% by weight, 	is to be recovered. Determine the minimum rate of water flow required and the 	number of practical stages if Murphree stage efficiency based on gas composition 	is 80% for 1.6 times the minimum water flow rate.

Equilibrium data:

Partial pressure 	12	18	32	50	70	166

of  NH3, mmHg

gms NH3 per 		0.2	0.3	0.5	0.75	1	2

10 gms H2O

Temperature and pressure of operation are 200C and 1 bar respectively.

6.	What are the most important methods for determining humidity? Explain any 	three methods in detail.

7.	Air at a temperature of 300C and a pressure of 750mmHg has a percentage 	humidity of 60. Calculate the pressure to which the air must be compressed at 	constant temperature in order to remove 90%of the water present.

8.a)	With the help of a typical drying rate curve for constant drying conditions explain 	the terms: Constant rate period, falling rate period, equilibrium moisture content 	and critical moisture content.

b)	Write a short note on spray drier with a neat diagram.

~ ~ ~ ~