Code No: 311051

OR

III-B.Tech  I-Semester Supplementary Examination - June 2003

PROCESS CONTROL INSTRUMENTATION

(Electronics and Instrumentation Engineering)

Time: 3 hours							Max.Marks:70

Answer any five questions

All questions carry equal marks

---

1.a)	Write the differential equations of the following liquid level process and hence 	derive the transfer function of the system.

m                                    u2

h2

C2		R2

u1

h1	  R1

C1

b) 	Say whether the above system is interacting or Non-interacting. Justify your

answer.

c) 	What are the effective time constants of this liquid level process?

2.a) 	Differentiate between a “Servo operation” and “Regulator operation”.

b)	Give a typical example for servo operation and explain.

c)	Explain the regulator operation with an example.

3.a) 	Discuss relative advantages and disadvantages of the proportional, integral and derivative control actions.

b) 	Discuss  the  characteristic  effects  of  a  proportional  controller  on  the  closed  loop  response  of   a  process.

4.a) 	Explain  with  analytical  expression,  the  concept  of   a  pure  controller  mode  which is  anticipatory  in  nature.  List  its  characteristics.

b)	Why  a  pure  derivative  is  not  preferred?  Draw  the  derivative  controller  output  for  the error  given  below.

e

+

_

t

(Contd…2)

Code No: 311051.				..2..				OR

5.a)	Explain the following terms as applicable to system evaluation with 	necessary graphs.

(i) Stability		(ii) measure of quality

- Discuss the quarter – amplitude criterion to evaluate the response.

6.	Explain the reflections problems in

a)  TTL integrated circuits.							B) C-MOS Integrated circuits.

7.	Explain the following Etching techniques with sketches.

a)	Simple batch – production etching.

b)	Continuous – feed etching.

c)	Etching with open – loop regeneration.

8.	a) 	Explain the principles of cascade controller tuning.

b) 	State the fundamental difference feed forward and conventional feed back 			control.

c) 	Why is a process model required for feed forward controller?

~ ~ ~ ~