Code No: 311052

OR

III-B.Tech  I-Semester Supplementary Examination - June 2003

ELECTRO MAGNETIC THEORY

(Common to Electronics and Instrumentation Engineering, Electronics and Control Engineering )

Time: 3 hours							Max.Marks:70

Answer any five questions

All questions carry equal marks

---

1.a) 	Define potential. Derive the equation for the potential at a point inside a solid 	sphere having a uniform volume charge density.

b) 	A point charge Q=30 nC is located at the origin in Cartesian coordinates. Find the 	electric flux density D ,E and potential at (1,3,-4)m.

2.a)	State and prove divergence theorem applicable to electric fields.

b) 	Find the voltage across each dielectric in a capacitor with an area of 1 square 	meter for the plates. The thickness of one dielectric is 1mm with relative 	permittivity of 4.The thickness of the second dielectric is 2mm with relative 	permittivity of 2 and is attached to the first dielectric. Applied voltage is 200V 	and the distance between the plates is 3mm.

3.a) 	Define curl of a vector.

b) 	State and prove Stoke’s theorem.

4.a)	Write Maxwell’s  equations  in point form. Using these equations derive

continuity equation  in point form.

- In a certain region with   = 0,  = 0,  and   = 6.250 , the magnetic  field  of  an

EM  wave  is   H =  0.6 cos(x) cos(108 t)  az (A/m). Find    and  the 	corresponding E field  using  Maxwell’s equations.

c)      A conductor with cross-sectional area of 0.001m2  carries a conduction current 0.2 	sin(109t) (mA). Given that  = 2.5 x 106 S/m and  = 6o, calculate the magnitude 	of the displacement current density.

5.a) 	Starting from the Maxwell's curl equations , derive the wave equation in Electric

field for free space.

b) 	A 9 GHz plane wave is propagating in a medium with r = 2.5.  If  E = 20 V/m 	and the material is assumed to be loss less, find the phase constant, wave length, 	phase velocity, propagation constant, intrinsic impedance and the magnitude of 	the H field.

6.a) 	Starting from the Wave equation in H field for free space, obtain the solution for

the same.

b) 	For sea water calculate the maximum frequency which may be used in submarine

communication, if the signal is to attenuate by at least 20 dB over a 30m depth

( Contd…2)

Code No: 311052.			. . 2 . .				OR

7.a)	Obtain the equations for a phase velocity and index of refraction.

b)	Determine the propagation constant  for a material having r = 1, r = 8 and

 =0.25ps/m if the wave frequency is 1.6Mz.

8.a) 	Explain total internal reflection.

b) 	Explain about the reflection of uniform plane wave at the surface of a conductive

medium

~ ~ ~ ~