Code No: 311152

OR

III - B.Tech. I Semester Supplementary Examinations, June 2003

DIGITAL CIRCUIT DESIGN AND APPLICATIONS

(Bio Medical Engineering)

Time: 3 hours.							Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1.a)	List relative advantages and disadvantages of linear wave shaping circuits using RC and RL components.

b)	Calculate the stable state currents and voltages for the binary circuit shown, which used p-n-p Ge transistors.  Find the minimum value of  hFE which will keep the ON transistor in saturation.

2.a)	Draw a Schmitt trigger using 555 timer and explain its operation.

b)	A sweep linear within 0.1% is required for a sweep amplitude of 20v.  Is it possible to use RC network?  Assume the sweep period 100 µ second. Discuss the other methods (Bootstrap and miller) to improve linearity taking some typical numerical values.

3.a)	Draw a  logic diagram of 2-line to 4-line decoder / demultiplexer using NOR gates only.

b)	Show that a positive logic AND gate is a negative logic OR gate, and vice-versa.

c)	Draw the configuration of a unidirectional diode gate and explain its working giving suitable waveforms.

Contd..2

Code No:311152				-2-				OR

4.a)	What are the advantages and disadvantages of TTL ICs over other digital ICs?

b)	Implement a full adder circuit with multiplexer.

5.a)	Draw the functional block schematic of PLLIC and explain its functioning.

b)	What is the need of display system?  What are the display system design requirements?  Discuss.

6.a)	Show how to convert JK flip-flop into delay (D-type) unit..  Give the truth table and verify.

b)	Show how to implement an AND with OR and NAND gates.

c)	Define a NAND gate and give its truth table.

7.a)	In an SCR sweep generator circuit shown below the circuit parameters and VBB=500v.  Find the amplitude and frequency of oscillation.  Make necessary assumptions, if any.

R = 1MΩ

c = 0.02 µf

VF = 25V

VB0= 25

b)	With a neat diagram and wave form explain the operation of a RS flip-flop.

8.	Write notes on:

a)  LED	b) Encoder	c) Decoder

&&&&&