Code No:311451

OR

III-B.Tech. I-Semester Supplementary Examinations June 2003.

THERMAL ENGINEERING AND HEAT TRANSFER

(Mechatronics)

Time: 3 hours							Max. Marks:70

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Differentiate between two stroke engine with four stroke engine.

b)	Explain the strichiometric air fuel ratio and its importance.

2.a)	Draw a neat diagram of a simple carburettor and explain salient features?

b)	What are different types of fuel injection systems?  Explain the working of fuel 	pump.

3.	The following observations were recorded for the testing of a single cylinder four 	stroke diesel engine:

bore = 350 mm;  stroke = 450 mm;  speed = 400 rpm;                                         	Area of indicator diagram = 472 mm2; length of indicator diagram = 62 mm; 	spring constant = 0.59 bar/mm; 	Load on dynamometer = 970N; 	dynamometer constant = 7500; fuel used = 10.78 	kg/h;                           	Calorific value of fuel = 50000 kJ/kg.

Calculate:  (a) IP;   (b) BP;  (c) Mechanical efficiency;  (d) Brake thermal 	efficiency and  (e) Indicated thermal efficiency.

4.a)	What are different methods to improve the performance of a gas turbine?  	Explain.

b)	Draw p-V and T-s diagram of closed gas turbine cycle and derive the equations 	for efficiency and work ratio.

5.a)	Derive 3-D general heat conduction equation in cartesean coordinates for a 	homogeneous material?

b)	Differentiate among three modes of heat transfer i.e. conduction, convection and 	radiation?

6.	An electric semiconductor device has a rating of 60MW, and for its proper 	operation the inside temperature should not be more than 80oC.  It is found that 	the device can dissipate about 15MW of heat on its own when placed in an 	environment at 40oC.  To avoid overheating of the device it is proposed to install 	aluminium (190W/mk) square fins of size 0.5 mm x 0.5 mm and 10 mm long to 	provide additional cooling.  Find number of fins required if h=12.5 W/m2K.

Contd…..2

Code No:311451				-2-					OR

7.a)	Derive the relation between heat transfer coefficient and other parameters for 	forced convection using Buckingham’s   theorem.

b)	A horizontal pipe 0.3048m in diameter is maintained at a temperature of 250oC 	in a room where the ambient temperature is at 15oC.  Calculate the free 	convection heat loss per meter of length.

8.a)	Distinguish between black body and grey body.

b)	Prove that intensity of radiation is given by Ib = Ebπ.

- - -