Code No.311452

OR

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

Principles of Machine DesigN

(Mechatronics)

Time: 3 Hours							   Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Name a few non-ferrous metals/alloys and their uses.

b)	What are the primary factors influencing the selection of materials in mechanical design?

2.a)	Find the diameter of a shaft to transmit twisting moments varying from 500 N-m to 2000 N-m.  The ultimate tensile strength is 600N/mm2.  Yield stress is 450 N/mm2.  Assume stress concentration factor as 1.2, surface finish factor as 0.8, size factor as 0.85 and factor of safety as 2.

b)	What is endurance limit?  Explain.

3.a)	In what way splines are superior to keys?

b)	Assuming suitable design stresses select and check a flexible flange coupling for transmitting 20 KW at 1440 rpm from a motor to a centrifugal pump.  Sketch the coupling.

4.	A railway wagon is supported on 16 No. of helical springs.  The weight of the wagon under load is 180 kN.  The shear stress in the springs under the load is 150N/cm2.  During running the load acting on the spring varies 140 to 220 kN in a cyclic manner.  Using soderberg line, determine the factor of safety used in the design of springs.  Yield point in shear for the springs = 400 N/cm2.  Endurance limit in shear for springs = 180 N/cm2.

5.	Design a connecting rod of I-section for an engine for the following:

Crank radius = 300 mm

Connecting rod length = 1800 mm

Speed of the crank = 400 rpm

Specific weight of material = 7.2 gm/cc

Neglect weight of the reciprocating parts.

Assume any necessary data reasonably.

Contd…2.

Code No.311452				-2-					OR

6.	A rope drive is required to transmit 740 kW from a pulley of 1 m diameter running at 450 rpm.  The safe pull in each rope is 2250 N and the rope weighs 10N/m length.  The angle of lap and the groove angle is 150o and 45o respectively.  Find the number of ropes required for the drive if the coefficient of friction between the rope and pulley is 0.3.

7.a)	Derive the expression for the length of a crossed belt drive.

b)	Design a spur gear drive required to transmit 45kW at a pinion speed of 800 rpm.  The velocity ratio is 3.5:1.  The teeth are 20o full depth involute with 18 teeth on pinion.  Both the pinion and gear are made of steel with a maximum safe static stress of 180N/mm2.  Assume a safe stress of 40 N/mm2 for the material of the shaft and key.

8.a)	What conditions must be satisfied in order that a pair of spur gears may have a constant velocity ratio?

b)	A motor shaft rotating at 1500 rpm has to transmit 15 kW to a low speed shaft with a speed reduction of 3:1.  The teeth are 14½ o involute with 25 teeth as the pinion.  Both the pinion and gear are made of steel with a maximum safe stress of 200N/mm2.  A safe stress of 40N/mm2 may be taken for the shaft on which the gear is mounted and for the key.  Design a spur gear drive to suit the above conditions.  Assume starting torque to be 25% higher than the running torque.

@@@@@