Code No.311454

OR

- III-B.Tech. I-Semester Supplementary Examinations, June 2003

INTRODUCTION TO ELECTRONIC CIRCUITS

(Mechatronics)

Time: 3 Hours							   Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks.

---

1.a)	Explain self biasing of a transistor in CE configuration.  Derive expression for Ic in active region.

b)	In  a common Emitter amplifier with self bias, R1 = 90 k, R2 = 10 k, and          = 0.98.  Calculate operating point stability factor ‘S’ for (i) RE = 1K,              (ii) RE = 2000.

2.a)	Analyse a transistor amplifier by calculating AI, Av, Ri and Yo in terms of h-parameters.

b)	Classifying amplifiers based on coupling using CE amplifiers.

3.a)	For the CE transistor amplifier shown below.  Calculate A1, Av, AIs, Avs, Ri and Zo if hie = 1.1k, hre = 2.5 x 10-4 , hfe = 50,  hoe = 25µA/v.

b)	What is thermal runaway in transistors.

4.a)	A multistage amplifier has 3 stages.  The voltage gain of 3 stages are 40, 60 and 80 respectively.  Calculate overall gain.

b)	Explain JFET small signal model, using CS.

5.a)	What is the effect of feed back on gain and frequency response of an CE amplifier?

b)	How Ri and Ro of an amplifier changes when voltage shunt feed back is applied with CE?

6.a)	Explain the working Colpitt’s oscillator?

b)	Calculate the frequency of oscillation of Hartly oscillator, with C = 400PF and   L1 = L2 = 2mH.

7.a)	What are the merits of push pull amplifier?

b)	Derive an expression for efficiency of a class B power amplifier.

8.	Write short notes below: a) Self biasing of a JFET, b) Classification of power amplifiers, with CE amplifier, c) Barkhausen criteria.

@@@@@