Code No: 311351

OR

III-B.Tech. I-Semester Supplementary Examination, June 2003

COMMUNICATION ENGINEERING

(Electronics and Control Engineering)

Time: 3 hours.				                 		      Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1.	Explain the method of generation of FM by indirect method.

2.	Explain with the aid of waveforms, how a grid modulated class C amplifier generates AM.

3.a)	Why is frequency stability of a transmitter important?   Explain one method to            	achieve good stability?

b)	A typical transmitter is to operate on a frequency of 146.94MHz and is to have a deviation z.  The Bandwidth of the audio amplifier is 300 – 3000 Hz. The Modulating signal is 1 KHz Sine wave.   Determine the frequency of the 'Crystal Oscillator.

4.a)	Draw practical diode detector and explain its working.

b)	Why AGC is required in radio receivers? Distinguish between Simple AGC and       delayed AGC.

5.a)		Write short note on “Image frequency and its reduction”.

b)	Explain the role of mixer and IF stages in AM receiver.

6.	What is meant by continuous wave modulation? Discuss with example.

7.	Draw the block diagram of FSK transmitter and explain.

8.a)	Explain mechanical, electrical, functional specification of E1A – 232 interface        standard.

b)	 What is the purpose of a null modem?

&&&&&