Code No.320454

OR

III B.Tech II Semester Supplementary Examinations December 2002/January 2003

DIGITAL SIGNAL PROCESSING

(Electronics and Communications Engineering, Electronics and Instrumentation Engineering, Electronics and Control Engineering, Electronics & Telematics)

Time: 3 Hours							      Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1. a)	If a system is represented by the following difference equation

y(n) = 3y2(n-1)-n x(n) + 4x(n-1) – 2 x(n+1) for  n0

- Is the system linear

- Is the system shift invariant

- Is the system causal

b)	Given a linear shift invariant system specified by the following difference equation

y(n) + 1 y(n-1) = x(n) – 1 x(n-1) ; n  0

2		   2

- Find the frequency response H(ejw) for the system.

- Find the |H(ejw)|.

- Find arg H(ejw).

2. a)	Consider a system with unit sample response h(m) given by h(n)= 2-n u(n).  If the input to the system is x(n) = 2 (n)+4(n-1) + 4(n-2)

  - Calculate and plot y(n) for n=0, 1, 2, 3, 4 5 and 6.

  - Is this system BIBO stable?  Is it causal?

b)	Find the unit sample response of the system represented by the difference equation

y(n) – a y(n-1) = x(n)

Assume y(n) = 0 for n > 0.

3. a)	If {y(n)} = {x1 (n)}N1 * {x2(n)}N2. prove that  { y(n)} will have N1 + N2 –1 samples.

b)	How are the DFT coefficients of {x1(n)} and {x2(n)} related if {x1(n)} =x(0), x(1), x(2), x(3) while {x2(n)} = x(3), x(0), x(1), x(2).

4. a)	Explain the symmetry conditions in DFT, which permit the development of FFT algorithms.

b)	Draw a 8-point Radix-2, DIF FFT flow graph and obtain the DFT of the following sequence.  Show all intermediate results x(n) = {0, 1, 2, 0, -1, -2, 0, 0}.

Contd…2

Code No. 320454				.2.			OR

5. a)	Evaluate x(-2), x(-1), x(0), and x(1) using the complex inversion integral for

x(Z) =   1    	with ROC |Z| > 2

(Z-2)

- Given a causal system specified by its system function H(Z) as follows

H(Z)	=   	Z(Z-1)

(Z-½) (Z + ¼)

i)	Find a difference equation realization of the system.

- Give the analytical form of the impulse response h(n)

6.	Design an IIR digital band pass filter with – 3dB lower cut off of 0.4 rad and upper –3dB cutoff of 0.5  rad.  The transition band for both upper and lower frequencies is 0.1 rad with minimum stop band attenuation of at least 40 dB.

7.	Compute 20 log|w(ejw)| at w=0 for the following windows:

a) Bartlett window for add N

b) Rectangular window

c) Hamming window

d) Blackman window

8. a)	With neat sketches, show the location of the poles (by labeling their co-ordinates in the s-plane) for Butter worth filter of orders (i) 4 and (ii) 5.

b)	An FIR digital filter has the following impulse response

h(n)	=        1	0  n  4

0	otherwise

use Hamming window and compute the impulse response of the FIR digital filter with linear phase.  Sketch its magnitude and phase response as a function of frequency for W  .

###