Code No.:320854

OR

III-B.Tech.II Semester Supplementary Examinations, December 2002/January 2003

CHEMICAL ENGINEERING ANALYSIS

(Chemical Engineering)

Time: 3 hours  							        Max. Marks: 70

Answer any FIVE questions

All questions carry equal marks

1.	Discuss a flow process in which a precipitation is being carried out by mixing two streams A and B to form a third stream C in which the precipitate is carried away.

2.	Discuss about the Rate equations for Homogeneous Chemical Reactions.

3.	If  P is a function of both x and y as given by P=Ax+Bxy−ay+cxy where A, B, C and a are constants.  Obtain expressions for the values of x and y (in terms of the constants) corresponding to maximum or minimum values of P.

4.	Prove that  ∇A⋅B=A⋅∇B+B⋅∇A+AX∇XB+BX∇XA

5.	Suppose that a slab (extending indefinitely in the y and z directions) at an initial temperature T1 has its two faces suddenly cooled to T0.  What is the relation between temperature, time after quenching and position within the slab.

6.	A solid sphere of radius R is placed in an incompressible, inviseid fluid of infinite extent.  The flow was initially uniform and parallel to the Z-axis flowing with a speed Vo in the negative Z direction.  Determine the velocity of the fluid after at sphere is placed in the flow and steady state has been achieved.

7.	The diffusivity D for a binary perfect – gas mixture is to be obtained by measuring the rate of inter diffusion of two gases originally confined in the two ends of a hollow cylinder.  A thin diaphragm separating the gases divides the cylinder into two sections of equal volume.  The diaphragm is suddenly removed and the gases allowed to diffuse for a measured time.  The diaphragm is then replaced and the gas in one-half of the cylinder is well mixed and analysed.  It is assumed that convection effects are not important.  Pure gas A is originally contained in one-half the cylinder and pure gas B in the other half, and the pressure and temperature of the system are kept uniform and constant.  Derive an expression which permits the calculation of the diffusivity.

8.	Explain: (a) utility of the Laplace Transform method.  (b) Equation of motion of a perfect fluid.

- - -