Code No.321354

OR

# III-B.Tech. II-Semester Supplementary Examination December 2002/January 2003

# ADVANCED CONTROL SYSTEMS

(Electronics and Control Engineering)

Time:3 hours								Max.Marks:70

# Answer any Five questions

All questions carry equal Marks

---

1.a)	State and prove the theorem for testing a time invariant (continuous) system for observability.

- Given the system

̣

X(t) =    -2   1	x(t) + 	1		u(t)

1  -2                        0

y(t) = 	  1   -1     x(t)

is the system observable.

2.a)	State and prove Lyapunov’s instability theorem.

- Obtain the phase variable form of the state space equation for the system whose transfer function is given by

C(s)      =  3sr+5s+13

R(s)          (s+2)(sr+4s+8)

3.a)	Explain clearly reduced order observer.

- A system represented without feedback as

̣

x (t) =    0   1        x(t) +        0     u(t)

-1  -2                          1

y(t) = 1     2	x(t)

is it controllable and observable? Check for both with feedback.

u(t) = x +   2   -1	x(t).

(Contd…2)

Code No.321354			-2-				OR

4.a)	Explain the procedure to formulate the optimal control problem:

- For minimum time

- For output regulator.

b)	A first order sytem

̣

X = -x + u

Is to be controlled to minimize

1

J= 1   ò (x2 + u2)dt.

2   0

find the optimal control law.

5.	What is a multi variable system? Discuss the Dynamic feedback compensation in frequency derivative.

6.a)	Explain Model Matching compensation.

- Discuss the control system tool box and its functions.

7.	Distinguish between the following:

- Combined Dynamic feedback and feed forward compensation.

- Controllable and observable realization.

8.	Write short notes on:

- Modal control and optimal control.

- State regulator problem.

- PC MaT Lab software package.

***     ***     ***