Code No.320206

Set NO.

1

III-B.Tech. II-Semester Supplementary Examination December,2002/January,2003

UTILIZATION OF ELECTRICAL ENERGY

(Electrical and Electronics Engineering)

Time:3 hours								Max Marks:80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1.a)	Explain the construction and working of a sodium vapour lamp.

b)	A lamp giving out 1200 lumens in all directions is suspended 8m above the 	working plane.  Calculate the illumination at a point on the working plane 6m 	away from the foot of the lamp.

2.a)	Explain the principle of arc welding.  Compare and contrast carbon and metallic 	arc welding processes.

b)	A slab of insulating material 150 cm2 in area and 1 cm thick is to be heated by 	dielectric heating.  The power required is 400 walts at 30 MHZ.  Material has 	permittivity 5 and p.f 0.05.  Determine the necessary voltage necessary.  Absolute 	permittivity = 8.854x10-12 F/m.

3.a)	What are the advantages of group drives over individual drives.

b)	What are the factors to be considered while selecting an electric drive for a 	particular application.

4.a)	A during hall 30mx15m with a ceiling height of 5m is to be provided with 	illumination of 120 lux.  Taking a coeffcient of utilization of 0.5 and depreciation 	actor of 1.4, determine the number of fluorescent tubes required, their spacing, 	mounting height and total voltage.  Take luminous efficiency of fluorescent  tube 	as 40 lumens/watt for 80 Watt tube,

b)	What are the various factors to be considered in the design of factory lighing.  	Explain them.

5.a)	Define crest speed, average speed and schedule speed and discuss the factors 	which affect the schedule speed of a train.

b)	A train is required to run between the two stations 1.5 km apart at a schedule 	speed of 36 km ph, the duration of stop being 25 sec.  The braking retardation is 3 	kmphps.  Assuming a trapezoidal speed/time curve, calculate the acceleration if 	the ratio of maximum speed to average speed is to be 1.25.

6.a)	Explain the terms

(i)	Adhesive weight

(ii)	Train resistance

(iii)	Speed time curve

Contd…..2

Code No:320206			-2-				Set No:1

b)	A 200 tonne electric train with scheduled speed of 40 kmph runs between two 	stations 2 km apart with an acceleration of 2 kmphps and braking retardation of 3 	kmph 	ps.  The train resistance is 50 Nw-m / tonne, effect of rotational inertia 10%, 	over all efficiency 70% and station stop 10 sec. calculate.

(i)	The maximum power output from the wheels

(ii)	The specific energy consumption.

7.a)	Describe how plugging, rheostatic braking and regenerative braking are employed 	with d.c motors, in electric fraction.

b)	Review the existing electric traction systems in India.

8.         Write a short note on the following:

(i)	Resistance welding

(ii)	Temperature rise in electric drives

- Speed time characteristics for different services (traction)

*******

Code No.320206

Set NO.

2

III-B.Tech. II-Semester Supplementary Examination December,2002/January,2003

UTILIZATION OF ELECTRICAL ENERGY

(Electrical and Electronics Engineering)

Time:3 hours								Max Marks:80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1.a)	What is an electric drive?  Classify various types of electric drives and discuss 	their merits and demerits .

b)	Suggest, with reasons the electric drive used for the following applications.

(i) Rolling mills  (ii) Tex tile mills  (iii) Cement mills (iv) Paper mills  (v) Coal 	mining  (vi) Lift, Cranes, Lathes and pumps.

2.a)	Explain why very high frequencies should not be used for dielectric heating.

b)	A wooden board 30 cms x 15 cms x 2 cms is to be heated from 200C to 1800C in 	10 minutes by dielectric heating using 40 MHZ supply.  Specific heat of wood 	0.35 and density 0.55 gm/cc.єr = 5 and p.f 0.05.  Estimate the voltage across the 	specimen and current during heating.  Assume loss of energy by  conduction, 	convection and radiation as 10%.

3.a)	Describe various types of Electric Arc welding processes.

b)	Compare resistance welding and arc welding.

4.a)	Explain clearly the following

Illumination, Luminous efficiency, MSCP,MHCP and solid angle.

- A small light source with uniform intensity is mounted at a height of 10 metres above a horizontal surface.  Two points A and B both lie on the surface with point A directly beneath the source.  How far is B from A if the illumination at B is only 1/15th of that at A?

5.a)	Discuss the (i) Specular reflection principle

(ii)  Diffusion principle of street lighting

b)	Determine the height at which a light source having uniform spherical distribution 	should be placed over a floor in order that the intensity of horizontal illumination 	at a given distance from its vertical line may be greatest.

6.a)	Discuss various traction systems you know of ?

b)	Explain the requirements for ideal traction and show which drive satisfies almost 	all the requirements.

Contd…..2

Code No:320206			-2-			Set No:2

7.a)	Define the term ‘co-efficient of adhesion’ and explain the factors on which it 	depends.

b)	Calculate the adhesive weight of a locomotive which accelerates up a gradient of 	1 in 100 at 0.8 kmphps.  The self weight of locomotive is 350 Tonnes.  	Coefficient of adhesion is 0.25.  Assume a train resistance of 45 N-m/Tonne and 	allow 10% for the effect of rotational inertia.

8.	Write short notes on the following;

a) What is a Glare? (b) The Ajax-wyatt funace (c) Factors affecting specific 	energy consumption.

***********

Code No.320206

Set NO.

3

III-B.Tech. II-Semester Supplementary  Examination,December,2002/January,2003

UTILIZATION OF ELECTRICAL ENERGY

(Electrical and Electronics Engineering)

Time:3 hours								Max Marks:80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1.a)	Explain with the help of a diagram, the four quadrant speed-torque characteristic 	of an induction motor when running in  (i) forward direction   (ii) reverse 	direction.

b)	A 100 hp, 500 rpm d.c. shunt motor is driving a grinding mill through gears.  The 	moment of inertia of the mill is 1265 kg m2. If the current taken by the motor 	must not exceed twice full load current during starting, estimate the minimum 	time taken to run the mill upto full speed.

2.a)	Discuss the different methods of electric heating and their relative merits.

b)	Dielectric  heating is to be employed to heat a slab of insulating material of 20mm 	thick and 1500mm2 in area. The power required is 200 watts at a frequency of     	30MHz. The material has a permittivity of 5 and a power factor of 0.05. 	Determine the voltage necessary and the current which flows through the 	material.

3.a)	Briefly explain the different methods of electric welding and state their relative 	merits.

b)	Give the comparison between ac and dc welding.

4.a)	With the help of a neat diagram, explain the principle of operation of fluorescent 	lamp.

b)	A machine shop 30m long and 15m wide is to have a general illumination of 150 	lux  on the work plane provided by lamps mounted 5m above it. Assuming a 	coefficient of utilization of 0.55, determine suitable number and position of light. 	Assume any data if required.

5.a)	Explain the general features of traction motors.

b)	A 250 tonne electric train maintains a scheduled speed of 30 kmph between 	stations situated 5km apart, with station stops of 30 sec. The acceleration is 1.8 	kmph ps. and the braking retardation is 3 kmph ps.  Assuming a trapezoidal 	speed-	time curve, calculate   (i) maximum speed of the train  (ii)  energy output 	of the 	motors if the tractive resistance is 40 NW  per tonne.

6.a)	Discuss the relative merits of electric traction and the factors on which the choice 	of traction  system depends.

b)	Explain the different methods of electric braking of a 3 phase induction motor.

Contd….2.

Code No. 320206			-2-				Set No.3

7.a)	Explain the terms (i) tractive effort  (ii) coefficient of adhesion  (iii) specific 	energy consumption of train  (iv) tractive resistance.

b)	A 100 hp  motor has a temperature rise of 50oC when running continuously on 	full load. It has a time constant of 90 minutes. Determine ½ hr rating of the motor 	for same temperature rise. Assume that the losses are proportional to the square of 	the load and motor cools completely between each load period.

8.	Write short notes on two of the following:

a)   Laws of illumination    (b)  Working Principle of sodium vapour lamp

c)   Existing traction systems in India.

- x -

Code No.320206

Set NO.

4

III-B.Tech. II-Semester Supplementary  Examination December,2002/January,2003

UTILIZATION OF ELECTRICAL ENERGY

(Electrical and Electronics Engineering)

Time:3 hours								Max Marks:80

Answer any FIVE questions.

All questions carry equal marks.

- - -

1.a)	Explain ‘load equalisation’. How  this can be achieved in industrial drives.

b)	A 50 hp, 400v, 750rpm synchronous motor has a moment of inertia 20 kgm2 and 	employs rheostatic braking for obtaining rapid stopping in case of emergency 	when the motor is running at full load, star connected braking resistor of 2 ohm 	per phase is switched on. Determine the time taken and the number of revolutions 	made before the motor is stopped. Assume an efficiency of 90%  and a full load 	power factor of 0.95.

2.a)	Explain the terms tractive effort, coefficient of adhesion, train resistance and 	specific energy consumption of train.

b)	An electric train maintains a scheduled speed of 40 kmph between stations 	situated at 1.5 km apart. If is accelerated at 1.7 kmph.ps. and is braked at 3.2 	kmph.p.s. Draw the speed-time curve for the run. Estimate the enrgy consumption 	at the axle of the train. Assume tractive resistance constants at 50 NW per tonne 	and allow 10% for the effect of rotation inertia.

3.a)	Explain the principle of operation of sodium vapour lamp and its advantages.

b)	A corridor is lighted by lamps spaced 9.15m apart and suspended at a height of   	4.75m above the centre line of the floor. If each lamp gives 100 candle power in 	all directions, find the maximum and minimum illumination on the floor along the 	centre line. Assume any data if required.

4.a)	State the advantages of electric heating.

b)	Briefly explain the different methods of electric heating .

c)	Estimate the energy required to melt 500 kg of brass in a single phase Ajax-wyatt 	furnace. If the melting is to be carried out in 34 hour, what must be the average 	power input to the furnace.

5.a)	Explain the different methods of electric welding and their relative advantages.

b)	Discuss  the laws of illumination and its limitations in practice.

c)	State the functions of starter and choke in a fluorescent lamp.

Contd…..2.

Code No. 320206			-2-				Set No.4.

6.a)	Explain the advantages of series parallel control of starting as compared to the 	rheostatic starting for  a pair of dc traction motors.

b)	Discuss the main features of various train services. What type of services 	correspond to trapezoidal and quardrilateral speed-time curves.

7.a)	Obtain the expression for temperature rise of an electrical machine. State the 	assumptions made if any.

b)	A 75kw, 500 rpm dc shunt motor is used to drive machinery for which the stored 	energy per kw is 5400  Joules.  Estimate the time taken to start the motor, if the 	load torque is equal to full load torque during the starting period and the current is 	limited to    1 ½  times the full load current.

8.	Write short notes on

a)	Regenerative braking of induction motor

b)	Existing electric traction system in India.

c)	Mercury Vapour Lamp.

- x -