Code No.320254

### OR

# III- B.TECH. II-Semester Supplementary Examination December 2002/ January,2003

## POWER ELECTRONICS

## (Electrical & Electronics Engineering)

Time: 3 hours								Max. Marks:70

## Answer any FIVE questions

All questions carry equal marks

- - -

1   (a)	Explain the operation of SCR by means of two transistor analogy.

(b)	Describe briefly the triggering circuits for SCR.  Explain the circuits to protect SCR 	from false triggering.

2   (a)	Explain how two-quadrant operation is possible in phase controlled converters with

SCRS.

- Explain the influence of source inductance of the output voltage of a 3-phase 6 pulse

fully controlled converter.  Draw the necessary wave forms and derive an expression 	for the output DC voltage.

3   (a)	Distinguish between natural commutation and forced commutation.  With circuit 	diagram and necessary wave forms explain the class C Commutation.

- With a neat circuit diagram and necessary wave forms explain the working of Morgan’s    chopper.

4   (a)	Explain clearly what do you understand by voltage commutation and current

Commutation in inverter circuits.

- Briefly discuss the relative merits of current fource inverter over voltage source inverters.  Explain the commutation process in a single phase CSI feeding into an R-L Load.

5   (a)	Explain the principle of operation of 3- phase cyclo converter.  Distinguish between

circulating and non – circulating current modes.

(b)  For a class – A chopper, Vs=230V, L=1MH R=0.2 ohm, EL=20V, T=2500 micro seconds   and tON=1000 micro second.

Calculate

(i)  average current  (ii)minimum and maximum values of instantaneous currents.

6   (a)	Explain different modes of operation of modified Mc- Murray – Bed ford half – 	bridge inverter.

- Explain the operation of single – phase capacitor commutated current source inverter with resistive load. Draw neat waveforms.

7	Explain principal of operation of, Jones chopper and oscillation chopper with neat

diagrams.

Contd….2.

Code No.320254			-2-					OR

8	Write short notes on any THREE of the following.

(a) Dual converter  (b)Free wheeling diode operation (c)Parallel inverter

(d)   Pulse width Modulation Techniques.

= = =