Code No.421002

Set No.

1

IV-B.Tech. II-Semester Examination April, 2003

TELEMETRY AND TELECONTROL

(common to Electronics and Instrumentation Engineering, Electronics and Control Engineering.)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Give a brief account of various types of transducers.

b)	What is meant by telemetry? Mention some uses of it.

2.a)	Describe the working of a synchro-transmitter and receiver system.

b)	Explain the uses of R.F telemetry system.

3.a)	Describe the working of F.M Generator.

b)	Describe the working of any photo electric type transducer.

4.a)	Differentiate between analog and digital techniques in tele control.

b)	Explain the working of electromechanical chopper demodulator.

5.a)	Describe the working of any tele control system using optical methods.

b)	Discuss the advantages of telemetry in industry.

6.a)	Describe the expression for an AM wave and analyze it for frequency components.

b)	The frequency and amplitude of a carrier wave are 1000KHz at 80V supply. It is modulated by an AM signal of 4 KHz producing 50% modulation. Find the upper and lower side frequencies.

7.	Write short notes on

a)	Photo emissive type transducer

b)	Example of radio telemetry system.

8.	Write short notes on

a)	State of the art-telemetry

b)	Tele control using information theory

***     ***     ***

Code No.421002

Set No.

2

IV-B.Tech. II-Semester Examination April, 2003

TELEMETRY AND TELECONTROL

(common to Electronics and Instrumentation Engineering, Electronics and Control Engineering.)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Describe the working of a synchro transmitter and receiver telemetry system.

b)	Give an account of classification of telemetry.

2.a)	Differentiate between motion balanced and force balanced telemetry systems.

b)	Explain the use of RF telemetry system.

3.a)	Describe the working of a FM generator.

b)	Explain the telemetry system using time division multiplexing principle.

4.a)	Explain the working of a ring type demodulator in a telemetry system.

b)	Discuss the advantages of optical fibres for signal transmission.

5.a)	Describe the working of photo emissive type transducer.

b)	Differentiate between electronic and mechanical commutators.

6.a)	Explain the uses of optical telemetry.

b)	The frequency and amplitude of a carrier wave are 1000KHz at 80V. It is modulated by an AM signal of 4 KHz producing 50% modulation. Find the upper and lower side frequencies.

7.	Write short notes on

a)	Example of land line telemetry system.

b)	State of the art-telemetry.

8.	Write short notes on

a)	Semi conductor demodulator.

b)	Example of tele control system.

***     ***     ***

Code No.421002

Set No.

3

IV-B.Tech. II-Semester Examination April, 2003

TELEMETRY AND TELECONTROL

(common to Electronics and Instrumentation Engineering, Electronics and Control Engineering.)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	With a neat sketch describe the working of a force-balance telemetry system.

b)	Give an example of optical telemetry system.

2.a)	Give an account of classification of telemetry?

b)	A transmitter supplies along 10kw to an aerial when unmodulated. Determine the total power when modulated to 40%.

3.a)	Derive the expression for AM wave and analyze it for frequency components.

b)	Give an example of photo electric type transducers. Describe its use in telemetry.

4.a)	What is meant by signal conditioning? Mention a few advantages of it.

b)	Differentiate between electronic and mechanical commutators.

5.a)	Explain the working of a semiconductor demodulator.

b)	Give a brief account of tele control using information theory.

6.a)	Illustrate a voltage telemetry system.

b)	Differentiate between hydraulic and pneumatic telemetry.

7.	Write short notes on

a)	Advantages of pulse width modulation.

b)	Optical fibres for signal transmission.

8.	Write short notes on

a)	Example of land line telemetry system.

b)	Multiplexing and demultiplexing techniques in telemetry.

***     ***     ***

Code No.421002

Set No.

4

IV-B.Tech. II-Semester Examination April, 2003

TELEMETRY AND TELECONTROL

(common to Electronics and Instrumentation Engineering, Electronics and Control Engineering.)

Time: 3 hours							Max.Marks:80

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Describe how current is used as a measure of pressure in a current telemetry system.

b)	What is meant by telemetry? Mention some uses of it?

2.a)	Describe how telemetry is used for satellite control system.

b)	What are the advantages of electrical transducers in telemetry?

3.a)	What is meant by signal conditioning? Mention a few examples.

b)	Differentiate between electronic and mechanical commutators.

4.a)	Describe the working of a semiconductor demodulator.

b)	A transmitter supplies along 8 kw to an aerial when unmodulated. Determine the total power when modulated to 30%.

5.a)	What are the advantages of photoelectric type transducers?

b)	Describe the working of a.c. signal conditioning system.

6.a)	What are the uses of RF telemetry?

b)	The frequency and amplitude of a carrier wave are 1000KHz at 80V. It is modulated by an AM signal of 4 KHz producing 50% modulation. Find the upper and lower side frequencies.

7.	Write short notes on

a)	Advantages of optical fibres for signal transmission.

b)	Tele control using information theory.

8.	Write short notes on

a)	Example of a tele control system.

b)	Trends in fibre-optic device development.

***     ***     ***