Code No.: 420203

Set No.

1

# IV-B.Tech. II-Semester Examinations April 2003

# Energy management

(Electrical and Electronics Engineering)

## Time: 3 Hours  							Max. Marks: 80

# Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain in detail the steps involved in Energy Management program.

b)	Explain the objectives of Energy Management.

2.a)	What is meant by cost index.

b)	An Industrial concern uses 1.5x103 tonnes of industrial coke, 1.8x103 therms of Gas and 1x109 Wh of electricity in one year.  The energy costs were as follows: coke Rs:72.00/- per tonne; Gas Rs:0.20/- therm; Electricity Rs:2.25/- per 103 Wh.  This industry produces 15x103 tonnes per year find out cost indices.  Assume

1 Joule = 947x10-6btu = 238.8x10-6calories = 9.478x10-9therm = 2778x10-6wh

c)	Discuss about use and cost of industrial energy

3.a)	Explain the various steps to be undertaken in order to achieve significant energy conservation in the industrial sector

b) 	List out the various data to be collected during an energy audit of a small factory.

4.a)	Explain the disadvantages of low pf in an electrical system from the consumer’s point of view

b) 	A 500 Kw load has pf of 0.8 lagging. It is desired to raise the pf to 0.95 lagging using a synchronous motor. The motor has also to drive a mechanical load of 250 HP at an efficiency of 84 %. Estimate the KVA input to the synchronous motor and its pf.

5.a)	Explain the qualities of energy manager

b)	Discuss about the duties check list for top management.

6.a)	What is meant by time value of money?

b)	Calculate the depreciation rate using the Straight-line method, sum of years digit method for the asset with salvage value zero.   Life of asset 5 years.   Cost Rs 1,80,000.  For decline balance method use 40% rate.

7.a)	What are the situations, which make the replacement of items necessary?

b)	The cost of a machine is Rs 6100 and its scrap value is only Rs.100.

The maintenance cost are found from experience to be:

Year		:	1	  2	  3	  4	  5	  6	  7	  8

Maintenance

Cost in Rs.	:	100	250	400	60	900	1250	1600	2000

When should be the machine be replaced?

Contd…2

Code No.: 420203				-2-				       Set No. 1

8.a)	Discuss the importance of planning and leading energy management program in an organization.

b)	Discuss the function of an energy manager in initiating and managing an energy management program with particular reference to planning, leading and controlling.

^^^

Code No.: 420203

Set No.

2

# IV-B.Tech. II-Semester Examinations April 2003

# energy management

(Electrical and Electronics Engineering)

## Time: 3 Hours  							Max. Marks: 80

# Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Discuss about essential imperatives and steps in supply side Energy Planning

b)	Explain about Energy planning flow for supply side.

2.a)	Discuss about general energy auditing.

- Draw the Pie diagram representing energy consumption of a company which uses on an hourly basis 11.72x103 therms gas 500x103 w electricity and 4.32x109 wh.

- Draw the Pie diagram showing consumption of electricity of a company.

Consider electricity usage by a company as office heating 150x103 W, lighting 120x103 w, Boiler house 90x103w and process 18x103W.

3.a) 	Explain the short term energy conservation scheme

b) 	List out the steps required for the implementation of an energy conservation programme.

4.a)	Explain how pf improvement plant can increase the Kw capacity of a generating station of given KVA capacity

b) 	A factory has a load of 325 Kw at a pf of 60 % lagging. A new load of 150 Kw is to be added, which can be carried by a synchronous motor. What must be the rating of the synchronous motor to make the overall pf to 95 % lagging. Also calculate the pf of the motor.

5.a)	Explain the functions of Energy manager.

b)	A process industry located in I.D.A. sanathnagar uses oil and electricity. It has an oil fired boiler and electric fired Furnaces and ovens it also uses pneumatic equipment during the process. As an energy manager suggest suitable energy management methods for the industry assume any other data required suitably.

6.a)	Distinguish between physical depreciation and functional depreciation with examples.

b)	An asset has a first cost of Rs 1,54,000, and has a salvage value of 32,750 at the end of 7 years of life.  Calculate the depreciation and book value by (i) straight line method (ii) decline balance method of depreciation.

Contd…2

Code No.: 420203				-2-				        Set No. 2

7.a)	Explain why replacement of machinery is necessary in any industry.

b)	A truck is priced at Rs. 60,000 and running costs are estimated at Rs. 6000 for 	each of the first four years, increasing by Rs. 2000 per year in the fifth and 	subsequent years.  If the money is worth 10 percent per year, when should the 	truck be replaced? Assume that the truck will eventually be sold for scrap at 	negligible price.

8.a)	Discuss the importance of promoting and monitoring an energy management program in an organization.

b)	Discuss the functions of an energy manager in initiating and managing an energy management program with particular reference to promoting, monitoring and Reporting.

^^^

Code No.: 420203

Set No.

3

# IV-B.Tech. II-Semester Examinations April 2003

# Energy management

(Electrical and Electronics Engineering)

## Time: 3 Hours  							Max. Marks: 80

# Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Explain how Energy management can be defined.

b)	Explain the importance of energy conservation and management.

c)	What are the important management objectives in promoting Energy conservation schemes in various industries?

2.a)	A company monitors its steam consumption and production (in tonnes) for a number of weeks and the results are shown in table.  Weekly steam consumption (106 Kg) against production (tonnes).

| Steam | 0.40 | 0.35 | 0.45 | 0.31 | 0.51 | 0.55 | 0.45 | 0.50 | 0.40 | 0.51 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Production | 50 | 55 | 65 | 50 | 95 | 90 | 85 | 80 | 60 | 90 |
| Steam | 0.40 | 0.60 | 0.45 | 0.55 |  |  |  |  |  |  |
| Production | 70 | 110 | 60 | 105 |  |  |  |  |  |  |

Discuss about trend relating production output and steam consumption by plotting weekly steam consumption against production.

- Discuss about load profile of a company with examples.

3.a)	Explain the three categories of Energy conservation schemes.

b)	Explain the different procedures and techniques used in energy auditing, with particular reference to data collection, and special tests.

4.a)	Under what conditions would you recommend installation of pf improvement plant for augmenting the Kw loading capacity of generating station in preference to installing additional generating equipment

b) 	Explain the pf improvement by the use of static capacitors

5.a)	What is the role of a Energy Manager in a functional organization of process industry.

b)	“The main role of Energy manager in any organization is energy conservation”. Comment on this statement.

6.a)	Explain with suitable examples, the different methods of depreciation.

b)	Cost of an air conditioning plant is Rs 1.0 lakh. During an estimated life of 10 years, two major overhauls were carried out, each involving Rs 20,000. If the scrap value of the plant is Rs 15,000, calculate depreciation rate on the basis of repairs provision method.

Contd…2

Code No.: 420203				-2-				       Set No. 3

7.a)	Discuss briefly the various types of the replacement problems

b)	The following table gives the running costs per year and resale price of a certain equipment whose purchase price is Rs. 5000.

Year		:	   1	    2	   3	    4	   5	   6	    7	   8

Running Cost

(Rs.)		:	1500	1600	1800	 2100	2500	2900	3400	4000

Resale value

Rs.)		:	3500	2500	1700	 1200	  800	  500	  500	  500

When should be the machine be replaced?

8.a)	Discuss the importance of planning and leading energy management program in an organization.

b)	Discuss the function of an energy manager in initiating and managing an energy management program with particular reference to planning, Leading and controlling.

^^^

Code No.: 420203

Set No.

4

# IV-B.Tech. II-Semester Examinations April 2003

# Energy management

(Electrical and Electronics Engineering)

## Time: 3 Hours  							Max. Marks: 80

# Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Describe the general principle of Energy management with suitable examples.

b)	A forging industry located at patancheru needs an energy management programme. Suggest a suitable programme step by step.

2.a)	An industry uses four forms of energy as shown below. Their monthly energy consumption and the costs are also shown.

Type			Quantity/month		cost in Rs./unit

Coal (MT)		60				410.00/MT

Oil (liters)		15,000				5.50/Liter

Gas (Therms)		8,000				6.50/therm

Electricity (Kwh)	2,43,000			1.10/Kwh.

Production rate = 35000MT/year

Calorific value of coal = 27.9  106 J/Kg

Calorific value of oil	 = 3.89  106 J/Liters

One Giga Joule 1GJ = 278 Kwh = 9.5 therms.

Out of the total energy purchased from coal and oil, 60% is used for process, 20% for boiler house and 10% for environment. From the total energy purchased from gas, 52% is used for process and 37% for environment and out of the total energy purchased from electricity 64% is used for process and 26% for air conditioning. Asssume any other data. Calculate the cost index, energy index. Draw a pie chart and sankey Diagram.

3.a)	Explain the difference between “Energy Efficiency” and “Energy Conservation”.

b)	Discuss the different approaches to Energy Conservation indicating clearly the benefits to be desired and the relative commercial viability.

4.a)	What do you understand by the term power factor, explain. Also explain the necessity of improving the power factor

b) 	A 3-phase induction motor takes 300 Kva at 400 V, 0.8 pf lag. If 100 Kva capacitor bank is connected in parallel with the motor with a view to give same heating in the supply cycle, with the increased load, calculate the Kva and Kw increase in the motor load. If however, the motor load remains unchanged at 300 Kva, 0.8 pf lag, calculate the reduction in the main cable current.

Contd…2

Code No.: 420203				-2-				        Set No. 4

5.a)	“Energy Manager is first and foremost a manager” in the light of this statement, discuss the functions of Management.

b)	Explain clearly the steps to be taken by Energy Managers to improve the efficiency of an organization.

6.a)	What are the different methods of depreciation.   With a suitable example show that the depreciation will be almost same even though different methods are adopted.

b)	What is the present worth of a replacement cost of Rs. 5,000 that occurs at the end of 5 years when the discount rate is 5 percent, compounded yearly?

7.a)	Explain the concept of group replacement.

b)	Explain how the theory of replacement is used in the following problems:

(i)	Replacement of items whose maintenance cost varies with time.

(ii)	Replacement of items that fail completely.

8.a)	Discuss the importance of promoting and monitoring an energy management program in an organization.

b)	Discuss the functions of an energy manager in initiating and managing an energy management program with particular reference to promoting, monitoring and Reporting.

^^^