Code No.422051

OR

IV-B.Tech. II-Semester Examination April, 2003

AUTOMATION IN MANUFACTURE

(common to Production Engineering, Mechatronics.)

Time: 3 hours							Max.Marks:70

Answer any FIVE questions

All questions carry equal marks

---

1.a)	Define Automation. Explain the different types of automation.

b)	Write a note on pneumatic component circuits.

2.a)	What is Buffer storage? Explain the reasons for the use of Buffer storage zones.

b)	Explain the following rotary transfer mechanisms:

i)	Ratchet and pawl

ii)	Geneva mechanism

3.a)	The following data apply to a 12-station in-line transfer machine:

P =  0.01 (all station have an equal probability of failure)

Tc = 0.3 min

Td = 3.0 min

Using the lower-bound approach, compute the following for the transfer machine:

- F, the frequency of line stops.

- Rp, the average production rate.

- E, the line efficiency.

What proportion of work parts are removed from the transfer line.

- Write a note on Flexible assembly lines.

(Contd…2)

Code No.422051					-2-		OR

4.	The following list defines the precedence relationships and element times for a new model toy:

| Element | Te(min) | Immediate predecessors |
| --- | --- | --- |
| 1 | 0.5 | - |
| 2 | 0.3 | 1 |
| 3 | 0.8 | 1 |
| 4 | 0.2 | 2 |
| 5 | 0.1 | 2 |
| 6 | 0.6 | 3 |
| 7 | 0.4 | 4,5 |
| 8 | 0.5 | 3,5 |
| 9 | 0.3 | 7,8 |
| 10 | 0.6 | 6,9 |

a)	Construct the precedence diagram for this job.

b)	If the ideal cycle time is to be 1.0 min, what is the theoretical minimum number of stations required to minimize the balance delay?

c)	Determine the assignment of work elements to stations using the ranked positional weights (RPW) method. How many stations are required? Compute the balance delay.

5.a)	What is an automated guided vehicle system? Explain different types of automated guided vehicle systems (AGVS).

b)	Explain vehicle guidance and routing in an automated guided vehicle system.

6.a)	Explain the interfacing of handling and storage systems with manufacturing.

b)	List the areas of application of automated storage/retrieval systems. Explain in brief.

7.a)	Explain adaptive control with constraints.

b)	Discuss the application of adaptive control in machining operations.

8.a)	What is ERP? Explain. Explain important modules in a typical ERP software.

b)	 Explain Rapid prototyping.

***     ***     ***