Code No.: 421801

Set No.

1

# IV-B.Tech. II-Semester Examinations April 2003

X-Ray metallography

(Metallurgy and Material Technology)

## Time: 3 Hours  							Max. Marks: 80

# Answer any five questions

All questions carry equal marks

---

1.a)	Explain the industrial applications of X-rays.

- Explain the differences between continuous spectrum and Characteristic spectrum.

- Explain the factors that influence the wavelength of lines emitted from an X-ray tube.

2.a)	Explain in detail Bragg’s law of X-ray diffraction.

b)	Explain how does X-ray diffraction by crystals differ from reflection of visible    light by mirrors.

3.	Explain the following

(i)	Measurement of X-ray intensity.

(ii)	Stereographic projections.

(iii)	Resolving power of a camera.

4.a)	Explain in detail the scattering of X-rays by an atom.

b)	What is structure factor? Derive an expression for the structure factor calculation for Nacl.

5.a)	Explain different types of cameras that are used in the Debye-Scherrer method.

b)	A monochromatic radiation of cobalt (1.79o A) is used in Debye-Scherrer camera in determining crystal structure of copper by powder technique. The lattice constant of copper is 3.610A. Obtain the first three observed values of ‘l’ when the diameter of camera is 114.6mm.

6.a)	What is a filter? Explain how a suitable filter is selected.

- How are indexing patterns of non-cubic crystal done by analytical methods

- A transmission pin hole photograph is made of copper with Cu K radiation. The film measures 4 by 5 inches. What is the maximum specimen the film distance which can be used and still have the first two Debye rings completely recorded on the film.

Contd…2

Code No.:421801			          -2-				       Set No.: 1

7.a)	Explain how the order-disorder transformation is find out for the structure of     Au Cu3.

b)	The powder pattern of aluminium made with Cu K	 radiation contains ten lines. Whose sin2 values are 0.1118, 0.1487, 0.294, 0.403, 0.439, 0.583, 0.691; 0.727; 0.872 & 0.981. Index these lines and calculate the lattice parameters.

8.	Write short notes on the following.

- Focusing cameras

- Techniques used in stress measurement.

- Phase diagram construction by X-ray diffraction method.

^^^

Code No.: 421801

Set No.

2

# IV-B.Tech. II-Semester Examinations April 2003

# X-Ray metallography

(Metallurgy and Material Technology)

## Time: 3 Hours  							Max. Marks: 80

# Answer any five questions

# All questions carry equal marks

---

1.a)	What is Electromagnetic radiation? Explain the properties of Electro magnetic radiation.

- Explain in detail how the X-rays are produced.

- Write a short notes on stereographic projection.

2.a)	Explain how with the help of X-ray diffraction techniques, the lattice dimensions are determined.

- The metal Iridium has an FCC Crystal structure. If the angle of diffraction for the (220) set of planes occurs at 69.220 (first order reflection). When Monochromatic radiation having a wavelength of 0.1542 nm is used. Compute (i) the interplanar spacing for this set of planes, and (ii) the atomic radius for an Iridium atom.

3.a)	Explain the procedure for determining the crystal structure and lattice parameter of a substance using the powder method.

b)	The powder pattern of a cubic material was made using Cu K radiation. Sin values corresponding to the first seven lines are 0.3451, 0.5007, 0.5997, 0.7084, 0.7921, 0.8676 and 0.8997. Calculate the lattice parameter and determine Bravais lattice.

4.a)	Explain the scattering of X-rays by a unit cell.

- Explain the structure factor calculations for a body centered cubic cell.

- Explain how are cameras classified in a Lane photograph.

5.a)	Draw a neat and labeled sketch of X-rays spectrometer. Explain its operation fully.

b)      The first three lines from the powder pattern of a cubic crystal have the following s values: 24.95, 40.9 and 48.05 mm. The camera radius is 57.3 mm. Molybdenum K radiation of wavelength 0.710 A are used. Determine the structure and the lattice parameter of the material.

6.a)	What are the errors that can be occurred and how they are classified in the precise parameter measurements.

- Compare and contrast Long-range order & short-range order.

- Explain in detail order-disorder transformations.

Contd…2

Code No.: 421801			            -2-				     Set No.: 2

7.	Explain the following

- Chemical analysis by parameter measurement.

- Techniques used in stress measurement.

- Phase –diagram determination/construction by X-ray diffraction method.

8.	Write short notes on the following

- Focusing cameras

- Filters

- Lane method of X-ray diffraction

- Continuous radiation.

^^^

Code No.: 421801

Set No.

3

# IV-B.Tech. II-Semester Examinations April 2003

# X-Ray metallography

(Metallurgy and Material Technology)

## Time: 3 Hours  							Max. Marks: 80

# Answer any five questions

# All questions carry equal marks

---

1.a)	What are X-rays? Explain in detail the production of X-rays.

- List the properties of X-rays.

- What is stereographic projection and explain.

2.a)	Derive Bragg’s equation. Explain the meaning of first order, second order and third order reflections.

- What are several techniques employed to determine the crystal structure of solids? Compare their advantages and disadvantages.

3.a)	Distinguish between Coherent and Incoherent radiations.

- Explain in detail the scattering of X-rays by an electron in an atom.

4.a)	What is absorption factor. Derive an expression for the absorption factor of a diffractometer specimen in the form of a flat plate of finite thickness.

b)	Explain the factors to be considered in calculating the intensities of X-ray diffraction.

5.a)	State and explain the usual steps in determining the structure of a crystal.

b)	Name the various methods of determining the crystal structure and explain any one method indetail.

6.a)	What is camera? Explain how cameras are classified in a Lane photograph.

b)	Define a powder photograph. How are powder photographs classified? Explain in detail.

7.a)	Explain in detail order-disorder transformations.

b)	A close packed Hexagonal cell has two atoms of the same kind located at 000,1/3, 2/3 ,1/2. Derive simplified Expression for F2.

8.	Write short notes on the following

- Focusing cameras

- Chemical analysis by diffraction.

- Errors in the measurement of lattice parameter.

^^^

Code No.: 421801

Set No.

4

# IV-B.Tech. II-Semester Examinations April 2003

# X-Ray metallography

(Metallurgy and Material Technology)

## Time: 3 Hours  							Max. Marks: 80

# Answer any five questions

# All questions carry equal marks

---

1.a)	State and derive Bragg’s law of X-ray diffraction.

b)	The BCC crystal is used to measure the wavelength of some X-rays. The Bragg angle for reflection from (110) planes is 20.20. What is the wavelength? The lattice parameter of the crystal is 3.150A.

2.a)	What is diffraction? Write in detail about diffraction directions and different diffraction methods.

b)	Distinguish between Coherent radiation and Incoherent radiation.

3.a)	What is structure factor. Derive structure factor calculations for Nacl.

b)	A certain tetragonal crystal has four atoms of the same kind per unit cell located at 0 ½ ¼ , ½ 0 ¼ , ½ 0 ¾ & 0, ½ , ¾ (do not change axes).

- Derive simplified expressions for F2.

- What is the Bravais lattice of the crystal?

- What are the values of F2 for the 100,111,011 reflections?

4.a)	Explain how do you determine the crystal structure of cubic, tetragonal crystals by analytical & graphical methods.

b)	What is a camera? What are the different types of cameras? Explain the different types of cameras that are used in Debye-Scherrer Method.

5.a)     A Debye-Scherrer pattern tungsten (BCC) is made with Cu K radiations. The first 4 lines on this pattern were observed to have the following q values

Line			  q

- 20.30

- 29.20

- 36.70

- 43.60

Index these lines and calculate their relative integrated intensities.

b)     The transmission Lane pattern is made up of aluminium crystal with a specimen to film distance of 5 cm. The (iii) planes of the crystal make an angle of 30 with the incident beam. What Minimum tube voltage is required to produce a (iii) reflection?

Contd…2

Code No.: 421801			          -2-				     Set No.: 4

6.a)	Why is a diffractometer used for the measurement of the intensity of X-rays? Explain the general features of this diffractometer.

- Explain how the indexing pattern of non-cubic crystals done by analytical and    graphical methods.

7.a)	Explain in detail order-disorder transformations in Au Cu3.

- Explain the following.

  - Techniques used in stress measurement.

  - Errors in the measurement of lattice parameter.

8.	Write short notes on the following

- Characteristic spectrum.

- Properties and application of X-rays

- Pinhole photographs

- Phase-diagram determination.

^^^