Code No:420256

OR

IV-B.Tech. II Semester Examinations April 2003

ELECTRICAL DISTRIBUTION SYSTEMS

(Electrical and Electronics Engineering)

Time: 3 Hours							        Max. Marks: 70

Answer any Five questions

All questions carry equal marks

---

1.a)	With a block diagram explain an energy monitoring and controlling station.

b)	Obtain a relation between load factor and loss factor.

2.a)	Sketch a typical primary loop feeder system and explain the same.

b)	Discuss the various methods used for correction of system voltage problems.

3.a)	What are the factors governing the selection of voltage levels in distribution systems? Briefly explain along with application.

b)	The annual input to a sub-transmission system is 1,75,200 MW hrs.  On the peak-load day of the year, the maximum demand is 50 MW and the energy input that day is 600 MW hrs.  Find the load factor for the year and for the peak load day.

4.a)	Explain a simple radial secondary system of distribution with a single line diagram.

b)	Discuss the basic design practice of secondary distribution systems.

5.	A three phase three wire 240 V secondary system with balanced load is shown in fig.5.  Determine the following:

a)	Calculate the total voltage drop in one phase of the lateral using the     approximate method.

b)	Calculate the real power per phase for each load.

c)	The reactive power per phase for each load.

50 Amps  0.9 p.f. lag

20 Amps  0.5 lag  p.f.

30 Amps  anti p.f.

(0.05 + j0.01) A  (0.1 + j0.02) B  (0.05 + j0.05) C

d) The kilovolt ampere output and load power factor of the distribution transformer.

Fig. 5.

Contd…..2.

Code No.420256				:: 2 ::					OR

6.	Discuss the various faults that occur and their probability.  Also give the procedure for fault current calculation in 3 phase faults.

7.a)	Explain in detail the shunt compensation and the procedure to determine the best capacitor location.

b)	What are the benefits from optimal location of substations?  Briefly explain.

8.	Write short notes on the following:

a) Series compensation.

b) Coordination of protective devices.

c) Switched shunt capacitors.

@@@@@