Code No.420454 OR

IV B.Tech. II Semester Examinations April 2003

SATELLITE COMMUNICATIONS

( Electronics and Communication Engineering )

Time: 3hours		                                                                          Max.Marks: 70

Answer any Five questions

All questions carry equal marks

---

1.a)	Write the electromagnetic spectrum and explain different bands.

b)	Draw the block diagram of a satellite communication and explain each block.

2.a) 	Compute the

i)   Carrier frequency corresponding to 1mm wave length.

ii)  The wavelength corresponding to 1GHz frequency.

iii) The frequency separation corresponding to a given wave length separation 8λ= λ1-λ2.

- What are the different types of multiple access and explain.

3.a)	What are the basic antenna types and write the relationships.

b)	The earth subtends an angle of 170 when viewed from a Geostationary orbit.

What are the dimensions and gain of a horn antenna that will provide global coverage at 4GHz ?

4.a)	Draw and explain the uplink experiment for the transmitting position of a digital earth station.

b)	Explain about the design of low-capacity satellite links with diagrams.

5.a)	Explain about the voice signal multiplexing with block diagrams.

b)	Explain different tracking techniques used in tracking satellites with large antenna.

6.	Explain TDMA frame structure for a satellite system in detail.  What is unique word and explain unique word correlator.

Cont…2

Code No..420454			-2-					OR

7.a)	Explain about transmission and reception of  Baseband zero-ISI pulses with waveforms.

b)	If we have a 4GHz receiver with following gains and noise temperative

T1n=500K			GRF=23dB

TRF=500k			GM=0 dB

Tm=5000K			GIF=30dB

TIF=10000K

Calculate system noise temperature.

- Write short notes on:

  - Power systems used in satellite.

  - CDMA Techniques.

  - GIT ration.

&&&&&