Code No.: 420304

Set No.

1

IV-B.Tech. II-Semester Examination April, 2003

INTERNAL COMBUSTION ENGINES

(Mechanical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1.a)	What are the methods of introduction of fuel and why are they required in 		internal combustion engines?

b)	How is it achieved in the principal engine types of the day? Explain any method 	with a neat schematic layout.

2a)	Explain what you understand by flame speed and what is it made up of?

b)	Why is flame speed important in combustion of S.I.engines?

3.a)	Explain the principle of carburetion.

b)	Briefly discuss the air-fuel ratio requirements of a petrol engine from no load to 	full   load.

4.a)	 What is meant by delay period ? It is usually divided in to two parts Name and   describe them.

b)  	Explain unit injector system.

5.a)  	What do you mean by supercharging?

b) 	A four stroke C. I. Engines, ( 125mm X 150 mm) runs at 200 rpm. The fuel 	consumption, when a supercharging with   compression ratio of 2 and isentropic 	efficiency of 0.70 is 0.2 kg/kWh. An  after  air cooler  cools the air with an 	effectiveness of 0.75. The engine   has a brake output of 360 kW and mechanical 	efficiency is 85%. Calculate the Output   of the engine   when the after cooler is

not used. Inlet conditions are 1 bar and 298 K.

6.a) 	List the  parameters  by which  performance  of an engine is evaluated ?

b)	A single cylinder oil engine has a compression ratio of 8 to 1. The specific fuel 	consumption  is 0.5 kg/ kWh. The  Calorific value of fuel is  4400kJ/kg. 	Calculate.(i) The thermal efficiency   and   (ii) the relative efficiency

Assume the engine is working on constant  volume cycle and  ratio of specific 	heats as 1.4.

Contd…2

Code No.: 420304			: 2 :			Set No.1

7.	What are the effects of the following factors on the formation of NOx?

  - Air fuel ratio

  - Spark ignition timing

  - Recycling

  - Speed.

8.a) What is boiling range of fuel?

b) How is it important in diesel engine operation?

c) How is the term ‘Volatility’ important for fuels?

-----

Code No.: 420304

Set No.

2

IV B.Tech II-Semester Examination April, 2003

INTERNAL COMBUSTION ENGINES

(Mechanical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1.a)	Explain the four stroke and two stroke cycle engine concepts.

b)	Discuss the relative merits and demerits of these engines.

2.a) 	 What do you understand by ignition delay in S.I.engine combustion.

b)	 Explain flame velocity and the effect of the speed of engine operation on it.

3a)	 Explain how the temperature of incoming  air and the design of carburetor effect     	 the process of carburetion.

b) 	 Briefly explain compensating jet device.

4.a)	 What is the importance of delay period ? Should the delay period be zero?

- Briefly explain solid injection system.

5.a) 	 What is a supercharger?

b) 	 How degree of supercharging is measured?

c )      How supercharging  improves the power  output?

6.a)   	What are the methods to find  the frictional power  of an engine?

b)  	An engine having 85% mechanical efficiency develops a brake power of 31 kW.

What will be the  mechanical efficiency at half load if frictional power  is     	assumed  constant? Find its indicated  power and frictional power.

7.	Write the short notes on the following : emission and pollution control measures 	for Internal combustion engines.

8.a) 	What is specific gravity of fuel and how it affects engine performance?

b)	What is ignition quality of fuel and explain different ways of indicating the 	ignition quality of fuel?

-----

Code No.: 420304

Set No.

3

IV B.Tech II-Semester Examination April, 2003

INTERNAL COMBUSTION ENGINES

(Mechanical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1a)	Give complete classification of i.c. engines.

b)       Explain methods of fuel introduction and their relative merits with two-stroke             	cycle engines.

2.       Explain the process of normal combustion in S.I.engines with the aid of  pressure-        	crank angle diagram.

3.a)	 Briefly explain a movable piston type accelerating pump.

b) 	Why rich mixture is required for the Maximum power ?

4.a) 	Explain the phenomenon of diesel knock. Compare it with the phenomenon of 	detonation in SI engine.

b) 	Explain distributor injection system.

5. a)	 What are the methods of  supercharging?

b) 	Derive an expression for the power required for an IC engine supercharger.

6  a)	 What is meant by specific energy consumption?

b)	A single cylinder diesel engine working on  the 4- stroke  cycle and delivers 	40  W at 600 rpm. The compression ratio  of the engine  is 14:1, fuel cut- off  	takes place at 5%  of the stroke, ratio of the stroke  to  bore  of the engine  is 1.5 	o 1. Index  of compression and expansion   curves are  1.4  and 1.3 respectively.

Calculate the  bore of the engine. Assume  the pressure at the beginning  of

compression as 1 atm.

7.a) 	What are the pollutants emitted by SI engines? Indicate their sources.

b)	 Indicate the effort of mixture strength on the pollutants formed.

8.a)	What will happen due to water and sediment content in the fuel?

b)	Explain the relationship between ignition quality and antiknock characteristics       of a fuel.

-----

Code No.: 420304

Set No.

4

IV B.Tech II-Semester Examination April, 2003

INTERNAL COMBUSTION ENGINES

(Mechanical Engineering)

Time: 3 hours							Max. Marks: 80

Answer any five questions

All questions carry equal marks

---

1a)	Discuss the need and importance of scavenging.

b)       Explain the method of loop scavenging.

2.a)	Explain the term flame speed in engine combustion.

b)	Discuss as to how very low flame speed and very high flame speed influence 	engine combustion and performance.

3. a)	 Explain why a simple carburetor cannot meet the engine requirement.

b) 	Explain how the temperature of entering fuel and speed effect the

carburetion.

4.a)  	Discuss the variables effecting the delay period.

b) 	What are the main advantages and disadvantages of the CI engine compared to 	the SI engine?

5.a)  	What are the  effects of supercharging on the following  parameters?

    - Power output

    - Mechanical efficiency.

    - Fuel consumption.

b)   Compare the idle   working cycle  exhaust  turbocharging and Miller  	turbocharging.

6. a)   	Define compression  ratio, explosion  ratio and cut - off  ratio of  thermodynamic

cycle.

b) 	Compare the performance aspects for different operational aspects for engines 	working on various thermodynamic cycle.

7.a)	What are the causes of NOx emission?

b) 	What is smog?

c)	What is exhaust gas recirculation? What is the effect on emissions?

8.a) 	What is the Composition of chemical elements that form Alcohols?

b)	What are the desirable properties of alcohols used as  fuels?

-----