Code No:421203

Set No.

1

IV-B.Tech. II Semester Examinations April, 2003

Digital speech and image processing

(Computer Science and Information Technology)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	With a brief note on sampling and quantization Explain the effect of varying sampling intervals and quantization levels on the quality of the image.

2.a)	Write a short note on image model.

b)	Explain in detail about different types of CCD sensors used for image acquisition.

3.	Explain any two image smoothing techniques.

4.	The speed of a bullet in flight is to be estimated by using high-speed imaging techniques.  The method of choice involves the use of a TV camera and flash that exposes the scene for Ks.  The bullet is 2.5 cm long, 1 cm wide, and its range of speed is 750  250 m/s.  The camera optics produce an image in which the bullet occupies 10% of the horizontal resolution of a 256 x 256 digital image.

a) Determine the maximum value of K that will guarantee that the blur from

motion does not exceed 1 pixel.

b) Propose a segmentation procedure for automatically extracting the bullet from

a sequence of frames.

5.	Explain how the Gradient and Laplacian operators are used in image segmentation applications in detail.

6.	Explain in detail the morphological operations used in gray scale image representation.

7.a)	What do you mean by bandwidth Compression?

b)	Explain the two different types of image Compression Techniques and their 	applications.

c)	In digital Image Compression how are the different types of redundancies  	applicable? Explain.

8.a)	Explain about  (i) slope overload   (ii) granular noise.

b)	List out the advantages and drawbacks of different types of lossy compression techniques.

@@@@@

Code No:421203

Set No.

2

IV-B.Tech. II Semester Examinations April, 2003

Digital speech and image processing

(Computer Science and Information Technology)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	Write a short note on uniform sampling and quantization. Also, discuss the false contouring and check boarded effect in detail.

2.a)	What are the advantages of colour images over monochrome images?

b)	Which is the most relevant colour image model for image processing? Why? Explain the same in detail.

3.	Give a brief note on Histogram Equalization Technique?  Explain why the discrete histogram equalization technique does not, in general yield a flat histogram?

4.	The speed of a bullet in fight is to be estimated by using high-speed imaging techniques.  The method of choice involves the use of a TV camera and flash that exposes the scene for Ks.  The bullet is 2.5 cm long, 1 cm wide, and its range of speed is speed is 750  250 m/s.  The camera optics produce an image in which the bullet occupies 10% of the horizontal resolution of a 256 x 256 digital image.

a)  Determine the minimum number of frames per second that would have to be taken in order to guarantee that at least two complete images of the bullet are obtained during its path through the field of view of the camera.

b)   Propose a method for automatically determining the speed o the bullet.

5.	Write short notes on the following:

a) Roberts operator 	b) Prewitt operator 	c) Sobel operator    d)  Zero crossing.

6.	Explain how the connected components of an image can be extracted using morphological operations.

7.	Write short notes on dilation and erosion.

a)  Define (i)    Compression ratio  (ii)    Relative data redundancy

(iii)   Mappings	        (iv) Quantization in Psycho visual redundancy.

b)  How is the interpixel redundancy differentiated from the coding Redundancy

and Psycho visual Redundancy?

8.a)	What do you mean by relative  address coding?

b)	Differentiate one dimensional and two dimensional Run length coding.

c)	What are the advantages of white black skipping?

@@@@@

Code No:421203

Set No.

3

IV-B.Tech. II Semester Examinations April, 2003

Digital speech and image processing

(Computer Science and Information Technology)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	What are the disadvantages of uniform sampling and quantization? Explain how these can be eliminated in non-uniform sampling and quantization.

2.	What do you understand by range images? Explain how range images are acquired?

3.	What is a convolution function?  How is the same implemented using a scan line Algorithm?

4.	Find the edge corresponding to the minimum-cost path in the subimage shown.  The numbers in brackets are gray levels and the outer numbers are spatial coordinates.  Assume that the edge column  and ends in the last column.

1           2         3

1       [2]       [1]     [0]

2       [1]       [1]      [7]

3       [6]       [8]     [2]

5.	Explain various detection of discontinuity methods in detail with suitable examples.

6.a)	Define thinning.  Explain the thinning using morphological operations with an example.

b)	Define thickening operation.  Explain with an example.

7.	How statistical Coding is differentiated from spatial coding. Give  one example  	                                            for both the coding. Explain.

8.a)	Explain direct contour tracing. How is it differentiated from predictive differential quantizing coding?

b)	What is double delta coding?

c)	Draw and mention the parameters of the PDQ algorithm.

@@@@@

Code No:421203

Set No.

4

IV-B.Tech. II Semester Examinations April, 2003

Digital speech and image processing

(Computer Science and Information Technology)

Time: 3 Hours							  Max. Marks: 80

Answer any FIVE questions

All questions carry equal marks

---

1.	What are the elements required to acquire digital images? Explain in detail about various imaging sensors in detail.

2.	With a neat block diagram, explain an image capturing system. Also, explain various image capturing devices.

3.	Write short notes on:

i)	Log Transformations	      ii) 	Power-Law Transformation.

4.	Find the edge corresponding to the minimum-cost path in the subimage shown.  The numbers in brackets are gray levels and the outer numbers are spatial coordinates.  Assume that the edge column and ends in the last column.

1       2      3

1    [5]   [6]   [1]

2    [6]    [7]   [0]

3    [7]   [1]   [3]

5.	Explain the following edge linking methods and boundary detection methods in detail.

(a) Local Processing  (b) Global Processing via the Hough Transform.

6.a)	Explain with example the boundary extraction using morphology operations.

b)	Write short notes on opening and closing.

7.	What are the types of compression used in Image application? Mention the requirements of compression. Briefly explain.

8.a)	Explain distortionless predictive coding.

b)	Explain Huffman-coding algorithm.

c)	What are the limitations and advantages of bit plane coding?

@@@@@