Code No: 420301

Set No

1

IV-B.Tech. II Semester Examination April, 2003

JET PROPULSION AND ROCKET ENGINEERING

(Mechanical Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.	The gas turbine takes air at 15oC the pressure ratio is 4 and maximum 	temperature is limited to 560oC. Assuming isentropic efficiencies of 	compressor and turbine as 83% and 86% respectively. Determine the overall 	efficiency (a) with out heat exchanger and (b) with heat exchanger of an 	effectiveness of 0.75.

2.a)	What is meant by jet propulsion? What are the basic differences between jet 	propulsion cycle and shaft power cycle?

b)	The exit velocity from a jet unit is 650 m/s from an air flow of 40 kg/s through 	the unit. The air craft is flying at 250 km/hr. Calculate the thrust developed, the 	thrust power and the propulsion efficiency. Neglect the effect of fuel.

3.	A Turboprop engine operates an aircraft at an altitude of 3000 meters above mean 	sea level at a speed of 525 kmph. The data for the engine  is given below.

Inlet diffuser efficiency 			=	0.875

Compressor efficiency 			=	0.790

Velocity air at compressor entry 		=	90 m/s.

Temperature rise through the compressor 	=	2300C

Properties of air :  = 1.4, CP = 1.005 kJ/Kg K.

From the above data calculate.

(i)Pressure rise through the inlet diffuser  (ii)Pressure ratio developed by the 	compressor  (iii)Power required by the compressor per unit flow rate of air and

(iv)The air standard efficiency of the engine.

4.a)	Describe the working  of a ramjet engine.

b)	Depict the various thermodynamic processes occurring in it on 
	h-S diagram.

c)	What is the effect at flight mach number on its efficiency.

5.a)	What are the requirements of an ideal rocket propellant?

b) 	Derive the equation for calculation of effective jet velocity in rocket?

6.a)	What is meant by monopropellant and bipropellant fuels for rocket engine?

b) 	Give the examples for monopropellant and bipropellant fuels and their uses.

Contd…..2

Code No.:420301				-2-				Set No.1

7.a)	Describe briefly three important applications of rocket propulsion?

b)	What is the purpose of injectors in rocket engines? Describe an injector with the aid of sketch?

8.a)	With the help of a neat diagram, write about nuclear rocket engine.

b) 	What are the advantages it has over other rocket engines.

- - -

Code No: 420301

Set No

2

IV-B.Tech. II Semester Examination April, 2003

JET PROPULSION AND ROCKET ENGINEERING

(Mechanical Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Explain the terms air rate, specific power, work ratio in gas turbine plants.

b)	A simple ideal gas turbine operates with a pressure ratio 8. The compressor and 	turbine inlet temperatures are 300 K and 800 K respectively. If the volume flow 	rate is 	250 m3/sec. compute the net power output and cycle efficiency.

2.a)	Explain the principle of jet propulsion and how the thrust is determined.

Explain clearly the various efficiencies associated with  propulsion devices.

3.a)	A Turbojet aircraft flies at 875 kmph at an altitude of 10000 m above mean sea

level. Calculate.

i)air flow through the engine

ii)thrust,

iii)Specific thrust

iv)thrust power and TSFC from the following data.

Diameter of the air inlet section = 0.75 m.

Diameter of jet pipe at exit = 0.5 m

Velocity of the gases at the exit of the jet pipe = 500 m/s

Pressure at the exit of the jet pipe = 0.3 bar.

Air to fuel ratio  = 40.

4.a)	Why a ram jet engine does not require a compressor and a turbine ?

b)	How an air craft having a ram jet engine takes off ?

c)	Give two important difference between Ram jet and pulse jet engine?

5.a)	What are the important applications of rocket engines?

b) 	Can rocket work in vacuum? Explain.

6.a)	What are the important properties required for a good propellant used for a

rocket engine?

b) 	What are different advantages and disadvantages of solid propellant over a

liquid propellant?

Contd…..2

Code No: 420301				-2-				Set No.2

7.	Describe briefly the aid of graphs, the variation of the following quantities in rocket propulsion

a)	Rocket thrust with altitude.

b)	Thrust coefficient Vs Exhaust nozzle pressure ratio.

c)	Thrust coefficient Vs area ratio of exhaust nozzle.

d)	Acceleration due to gravity Vs altitude.

8.a)	With the help of neat diagram, explain the working of magneto-plasma rocket

engine.

b) 	Can rocket work in vacuum? Explain.

- - -

Code No: 420301

Set No

3

IV-B.Tech. II Semester Examination April, 2003

JET PROPULSION AND ROCKET ENGINEERING

(Mechanical Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Obtain an expression for the work output in terms of  Tmax,Tmin, rp, Cp and .

b)	An open cycle gas turbine plant receives air at 0.98 bar and 23oC. The air is 	compressed to 5.25 bar and reaches a maximum temperature of 650oC. The hot 	air expands back to 0.98 bar. Assuming the compression and expansion 	processes are isentropic. What is the ratio of the work required to drive the 	compressor to the work developed by the turbine.

2.a)	Explain Thrust Power, Propulsive Power and Propulsive efficiency with respect 	to atmospheric thermal jet engines.

b)	Obtain an expression for Propulsive efficiency of an atmospheric thermal jet 	engine and its thermal efficiency.

3.	A turbojet engine propels an aircraft at a mach number of 0.8 in level at an 	altitude of 10 km. The data for the engine is given below stagnation temperature 	at the turbine inlet = 1200 K stagnation temperature rise through the compressor = 	175 K, Calorific value of fuel = 43 MJ/kg.

Compressor efficiency 	=	 0.75

Combustion chamber efficiency 	= 	0.975

Turbine efficiency 	=	0.81

Mechanical efficiency of the power transmission between turbine and   	compressor 	=	 0.98

Exhaust nozzle efficiency 	=	0.97

Specific impulse 	=	25 seconds

Assuming the same properties for air and combustion gases calculate.

(i)fuel – air ratio  (ii)compressor pressure ratio  (iii)Turbine pressure ratio

(iv)Exhaust nozzle pressure ratio (v)Mach number of exhaust jet.

4.a)	Draw the sketch of a pulse jet engine. Write down its main advantages and 	disadvantages.

b)	Describe the working of a scramjet engine. What is its advantages over the ram jet 	engine.

5.a)	How the propulsive efficiency of rocket engine can be estimated?

b) 	What are the major differences between Air breathing engine and rocket

engine.

Contd…..2

Code No.:420301				-2-				Set No.3

6.		Sketch the layout of liquid propellant rocket engine and explain its working. Give 	its applications.

7.a)	Draw a neat sketch of turbo pump feed system employed in rockets.

b)	How does cavitation occur in this system and what measures are taken to prevent it?

c)	What is the effect of cavitation on the operation of the rocket.

8.	What are different types of electric rocket propulsion engines? Explain briefly

about all of them.

- - -

Code No: 420301

Set No

4

IV-B.Tech. II Semester Examination April, 2003

JET PROPULSION AND ROCKET ENGINEERING

(Mechanical Engineering)

Time: 3 Hours							Max. Marks: 80

Answer any Five questions

All questions carry equal marks

---

1.a)	Why maximum temperature in gas turbine cycle is limited to 850oC, why lean 	A:F 	ratio is used in gas turbines and what is the range of it.

b)	In  a gas turbine plant air enters the compressors at 1 bar and 27oC, the 	pressure ratio is 6. The temperature at turbine inlet is 1000 K. the mass 	flow 	rate of air is 10 kg/sec. Determine  (i) Power required to drive the 	compressor and the turbine power output 	(ii) thermal efficiency.

2.a)	Explain the energy flow in an atmospheric thermal jet engine and what are 	powers corresponding to brake and indicated power of I.C. engines.

b)	What is the need for thermal jet engines and discuss the applications of 	thermal jet engines.

- A turbojet air craft is flying at 800 km/hr at an altitude where the pressure is 0.25 bar and temperature is – 40 C. The pressure ratio in the compressor is 10:1 and the maximum cycle temperature is 8000C. Calculate the thrust developed and specific fuel consumption. Assume the following data : isentropic efficiency of compressor  = 90% Total head pressure loss in the combustion chamber = 0.15 bar, calorific value of fuel = 40200 kJ/hr combustion efficiency = 98%, isentropic efficiency of Turbine = 92% and Jet Nozzle = 94%, nozzle outlet area = 800 cm2. The nozzle in the turbojet is convergent.

4.a)	Distinguish between Ram jet and serqujet engine

b)	Explain the following terms as applied to Rockets and derive equations for them 	(i) Thrust (ii) Thrust power (iii) Propulsive efficiency.

5.a)	Discuss the possibility of rocket powdered vehicle having a flight velocity

greater than the exhaust velocity of the rocket motor?

- What are different types of rocket engines? Explain.

6.	What are different types of liquid propellants used in rocket engines? How these 	fuels can be combined as bipropellants?

Contd…..2

Code No.:420301				-2-				Set No.4

7.a) 	Draw a simple sketch of a multi stage rocket vehicle depicting clearly the hooster stage, propellant tanks and exhaust nozzle instruments and navigational equipment.

b)	Explain briefly the meaning of the following:

i)	Booster rocket stage.

ii)	Sustainer stage.

iii)	Retro Rockets.

8.a)	With the help of a neat diagram, explain the working of liquid bi-propellent

gas pressurization system.

b) 	Discuss the possibility of rocket-powered vehicle having a flight velocity 	greater than the exhaust velocity of the rocket motor.

- - -