Code No.420201

Set No.

1

IV-B.Tech.  II  Semester Examinations April, 2003

INSTRUMENTATION

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Briefly describe the different criteria for selection of transducer for a particular application

b)	Explain the differences between Primary and Secondary transducers.

2.a)	Derive the expression for time response of first order system subjected to step input.

b)	A RC circuit consists of a capacitor of 1F in series with a resistor of 1k. A dc voltage of 50V is suddenly applied across the circuit. Calculate the value of voltage after 10 mSecs.

3.	Draw and explain the circuit of a digital frequency meter. What are the different methods used for high frequency determination? Explain each of them briefly.

4	Describe the circuits and working of wave analysers used for audio frequency and megahertz ranges.

5.a)	What  are the primary detectors? Explain in detail.

- A torque bar of 30 mm diameter is used for measurement of a torque of 100    NM.Calculate the angle of twist if shear modulus of mild steel  is 80X109 N/M2

6.a)	Discuss in detail the operation of LVDT?

b)      What are the advantages and disadvantages of LVDT?

7.a) 	Explain gauge sensitivity in case of a strain gauge. Derive an expression for the output voltage of a wheat stone bridge when one of the arms is a strain gauge.

b) 	A strain gauge bridge has one arm as strain gauge with a gauge factor of 2.2. the resistance value of strain gauge and other arms is 120. Find the bridge output voltage    ( with output open circuited) for a supply voltage of 3V when the strain gauge is subjected to 600 micro strain.

8.a)	Describe the measurement of liquid level using capacitive transducer in case of

insulating and conducting liquids.

- Explain how the capacitance transducers are used in measurement of non-   electrical quantities? Give neat sketch.

  - x -

Code No.420201

Set No.

2

IV-B.Tech.  II  Semester Examinations April, 2003

INSTRUMENTATION

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	i)	Describe the international standard of mass and length

ii)	What are atomic standards for frequency and time?

- What are different standard test signals for studying the dynamic response of a     system? Explain them with suitable sketches.

2.	Briefly explain the following systems with examples

(i) Underdamped system (ii)  Overdamped system (iii)Critically damped system

3.a)	 Describe the working of an integrating type voltmeter.

b)	 A 412 digit voltmeter is used for voltage measurements. (i) Find its resolution. (ii) How would 12.98 V be displayed on 10V range? (iii) How would 0.6973 be displayed on 1 V range? (iv) How would 0.6973 be displayed on 10 V range?

4.a)	 Describe the engineering applications of wave analysers

- Explain the different types distortions caused by amplifiers.

5.a)	Explain the method of measurement of linear displacement using potentiometer      b)	 A helipot is provided with 40 turns/mm. The gearing arrangement is such that the 	motion of the main shaft by one revolution causes 5 revolutions of the 	potentiometer shaft. Calculate the resolution of the  Potentiometer?

6.a)	Explain in detail the application of  RVDT?

b)	Discuss in detail the operation of a synchros?

7.a) 	Explain the method of adjacent arm compensating gauge for temperature compensation in strain gauge with a neat circuit.

b) 	A single electrical resistance strain gauge  of resistance 120 and having a gauge factor of  2 is bonded to steel having an elastic limit stress of 400MN/m2 and modulus of elasticity 200 GN/m2 . calculate the change in resistance due to a change of temperature of 200C. co-efficient of linear expansion of  steel is 12x10-6/0C.

8.a)	Explain the principle of operation of electromagnetic flow meter. Describe how flow is measured with a neat sketch.

b)	Give the block diagram and explain the operation of a servo-operated electromagnetic flow meter to measure flow rate.						                                        -x-

Code No.420201

Set No.

3

IV-B.Tech.  II  Semester Examinations April, 2003

INSTRUMENTATION

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1a)	Describe the following signals with  suitable plots:

(i)Continuous time periodic signals	(ii) Aperiodic signals

b) 	Explain how sampling can be done using an impulse function?

2.	A 50F capacitor is charged to a voltage of 100Volts at time t=0Secs. It is discharged through a 1M resistor. Determine the time for the voltage across the capacitor to reach 10 Volts? Derive the expressions for the time response of second order system subjected to impulse input

3 (a)	A 312 digit voltmeter is used for measuring voltage. (i) Find its resolution.              (ii) How would a voltage of 14.53 V be displayed on 10V range? (iii) How would a reading of 14.53 V be displaced on 100V range?

(b)	Draw and explain the block diagram of storage oscilloscope

4. Describe the basic circuit of a spectrum analyzer. Explain how the spectra of the following are displayed

- continuous wave signals. (ii) amplitude modulated signals.

- Frequency modulated signal.

5. a)	What are the advantages and disadvantages of a capacitive transducer?

b)	A barium titanate pick up has the dimensions of 5mmx5mmx1.25mm.The force acting on it is 5N.The charge sensitivity of  barium titanate is 150 PC/N and its    permittivity is 12.5x10-9 F/M. The force acting on it is 5N. If the modulus of elasticity of barium titanate is 12x106 N/M2. Calculate the strain, the charge and the capacitance

6.a)	Explain the operation of a thermocouple for the measurement of temperature?

b)      Explain in detail about photo voltaic  and photo conductive cells?

Contd…2.

Code No.420201				-2-				Set No.3.

7. a) 	What is a load cell? Explain the working of a load cell strain gauge bridge.

b) 	A load cell consists of a solid cylinder of steel 40mm in diameter with four strain gauges bonded to it and connected in to the four arms of a voltage sensitive bridge. The gauges are mounted to have Poisson’s arrangement. The gauge factor is 2.1 and each gauge has 1000 ohms resistance. The bridge excitation voltage is 6v. Determine the sensitivity of the cell in V/KN modulus of elasticity for steel is 200GN/m2 and Poisson ratio is 0.29.

8.a)	Discuss the merits demerits of constant temperature method over constant current method of measurement at flow using hot wire anemometer .

- With a neat sketch explain the measurement by constant – temperature     anemometer.

-x-

Code No.420201

Set No.

4

IV-B.Tech.  II  Semester Examinations April, 2003

INSTRUMENTATION

(Electrical and Electronics Engineering)

Time: 3 hours							        Max. Marks:80

Answer any FIVE questions

All questions carry equal marks

- - -

1.a)	Describe the process of obtaining Discrete time signal from continuous time signal. Draw the necessary plots.

- What are the major classification of signals? Explain them in detail with suitable        sketches.

2.a)	A second order instrument is required to measure the input signals upto 100Hz with an amplitude inaccuracy of less than 3%. Calculate the natural frequency requirement if the damping ratio is 0.5.

b)	 What is meant by static and dynamic response of systems?

3.a)	Explain the functioning of digital phase angle meter.

b)	Describe the working of the successive approximation type DVM.

4.a) 	Draw and explain the circuit of a Vector impedance meter.

b) 	Explain the principles of Peak reading and RMS voltmeters.

5.a)   	A resistive position transducer with a resistance of 5 k and a shaft stroke          	of 8 cm is applied with a voltage of 5V.When the wiper is 3cm from the          	reference, what is the value of the output voltage?

b)	A resistance strain gauge with a gauge factor 2.04 is fastened to a beam           	which is subjected to a strain of 1X 10-6.If the original resistance of the          	gauge is 120 , calculate the change in resistance.

6.a)	Explain in detail about photo diode  and photo transistors?

- Explain in detail about thermocouples?

7.a) 	What is the gauge sensitivity? Explain with a neat sketch to find the sensitivity of a half  bridge.

b) 	Two electrical strain gauges are bonded to a duralumin cantilever and connected a bridge as adjacent arms. Each gauge has a resistance of 100 and a gauge factor of 2.1. the input voltage is 4V. The stress is 200MN/m2. Find the current through the detector  if its resistance is 400. Modulus of elasticity of duralumin is 70 GN/ m2.

Contd….2.

Code No.420201				-2-				Set No.4

8.a)	What is dynamic compensation ? Explain the compensation adjustment scheme in a hot wire anemometer using square wave  current source.

- Explain how torque can be measured using an inductive transducer. Give the sketch for arrangement of inductive transducers with respect to shaft.

-x-